BlueSim Training Guide

Alarm Monitoring System (AMS)

Monitoring & Control

Integrated Alarm Monitoring System for engine room automation with 40+ monitored points, trend viewer, alarm ACK workflow, and watchkeeping timer.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Integrated Alarm Monitoring System for engine room automation with 40+ monitored points, trend viewer, alarm ACK workflow, and watchkeeping timer.

## Key Features

Engine room mimic panel with 40+ monitored points across all systems.

Priority 1-4 alarm list with individual and group acknowledgement.

Trend chart viewer for any monitored point (5-minute rolling data).

Watchkeeping timer and maintenance inhibit mode with logging.

Multi-scenario training: flooding, ME LO drop, watchkeeping alarms.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| ALARM SUMMARY | Displays real-time system data, controls, and status indicators. |
| WATCHKEEPING | Displays real-time system data, controls, and status indicators. |
| SYSTEM STATUS | Displays real-time system data, controls, and status indicators. |
| MAIN ENGINE | Displays real-time system data, controls, and status indicators. |
| AUXILIARY ENGINES | Displays real-time system data, controls, and status indicators. |
| TANKS &amp; BILGE | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## AMS — Alarm Monitoring System Technical Reference

### Overview

The Integrated Alarm Monitoring System (AMS) provides centralized monitoring and alarm management for all engine room and machinery systems. It replaces traditional hard-wired alarm panels with a distributed digital system, providing trend data, alarm history, and watchkeeping support per SOLAS and class rules.

### Regulatory Framework

**SOLAS II-1/Regulation 51** — Machinery space automation requirements

**IMO MSC/Circ.1280** — Guidelines for vessels with periodically unattended machinery spaces

**IACS UR M26** — Periodically unattended machinery spaces

**IACS UR E10** — Electrical installations

**MCA MGN 349** — Guidance on UMS operation

**DNV/LR Class Notation** — AUT, AUT-UMS for unmanned machinery spaces

### Alarm Priority Classification

| Priority | Color | Description | Response Time |

|----------|-------|-------------|---------------|

| 1 — Emergency | Red/Flash | Immediate danger to ship, personnel, or cargo | Immediate |

| 2 — Alarm | Red | Abnormal condition requiring prompt action | 1-2 minutes |

| 3 — Warning | Amber | Condition approaching alarm threshold | 5-10 minutes |

| 4 — Advisory | Blue/Cyan | Information requiring monitoring | 15-30 minutes |

### Monitored Systems and Points

#### Main Engine

| Tag | Description | Normal Range | Alarm |

|-----|-------------|--------------|-------|

| ME-LOP | LO (Lube Oil) Pressure | 3.5-5.0 bar | <3.0 bar |

| ME-JCT | JCW (Jacket Cooling Water) Temp | 75-85°C | >90°C |

| ME-TCR | Turbocharger Speed | 15,000-22,000 RPM | <12,000 RPM |

| ME-RPM | Main Engine RPM | 80-100 RPM | Per load |

| ME-EXT | Exhaust Gas Temp avg | 300-420°C | >450°C |

| ME-FOT | Fuel Oil Temp | 130-145°C | <120 / >150°C |

#### Auxiliary Engines (×3)

| Tag | Description | Normal Range |

|-----|-------------|--------------|

| AE1/2/3-LOP | LO Pressure | >3.0 bar |

| AE1/2/3-JCT | JCW Temperature | <88°C |

| AE1/2/3-EXH | Exhaust Temperature | <450°C |

#### Boiler

| Tag | Description | Normal Range | Alarm |

|-----|-------------|--------------|-------|

| BO-STP | Steam Pressure | 6.0-7.5 bar | <5.5 / >8.0 bar |

| BO-WTL | Water Level | 60-75% | <40% / >90% |

| BO-FOT | FO Temperature | 110-130°C | >140°C |

#### Generators

| Tag | Description | Normal |

|-----|-------------|--------|

| G1/2/3-VLT | Voltage | 440V ±5% |

| G1/2/3-FRQ | Frequency | 60 Hz ±0.5 |

| G1/2/3-KW | Load (kW) | Per capacity |

#### Pumps

| Tag | Description |

|-----|-------------|

| BP-01-03 | Bilge pumps ×3 |

| BP-BAL | Ballast pump |

| FO-SVC | FO Service pump |

| LO-CME | LO circulating pump (ME) |

#### Tank Levels

| Tag | Description | Alarm Low | Alarm High |

|-----|-------------|-----------|------------|

| TK-FO-SB | FO service tank SB | <20% | >95% |

| TK-DO | DO tank | <15% | >95% |

| TK-FW-DK | FW domestic | <20% | >95% |

| TK-BLG | Bilge tank | — | >70% |

| TK-SLG | Sludge tank | — | >85% |

| TK-BAL-FP | Ballast FP tank | — | >95% |

### Watchkeeping (UMS)

Chief Engineer responsible for UMS operation

AMS watchcall alarm to accommodation: 2 minutes response required

Engineer call: Activates audio alarm in cabins

Inhibit mode: Maximum 30 minutes (with override log)

Bridge notification: Priority 1 alarms auto-forwarded to bridge panel

### Trend Analysis

5-minute trend charts for all monitored points

Settable trend period: 5 min, 30 min, 4 hr, 24 hr

Historical data: 30 days

Export: CSV via USB for class/owner reports

### Alarm Inhibit (Maintenance Mode)

Engineer enters tag number and reason

Maximum inhibit duration: 30 minutes (configurable per class)

Inhibit logged in alarm history

Inhibited points shown with distinct indicator on overview

### Scenarios in Simulator

| Scenario | Key Learning Points |

|----------|---------------------|

| Multiple Simultaneous Alarms | Alarm prioritization, group ACK, flooding response |

| Watchkeeping Alarm Response | Timer-based alarm, engineer call function |

| ME LO Pressure Trend Analysis | Trend viewer, identifying gradual degradation |

| Inhibit/Maintenance Mode | Proper inhibit procedure, logging, safety |

### References

SOLAS 2024 Consolidated Edition, Chapter II-1 Regulation 51

IMO MSC/Circ.1280: Guidelines for Periodically Unattended Machinery Spaces

IACS Unified Requirement M26 (Rev.7, 2020)

DNV Rules for Ships Pt.4 Ch.9: Instrumentation and Automation

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System