BlueSim Training Guide

Battery Hybrid Propulsion Control

Propulsion & Emerging Tech

Battery Energy Storage System (BESS) integrated with diesel-electric propulsion. Hybrid, zero-emission, and peak shaving modes with live energy flow diagram.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Battery Energy Storage System (BESS) integrated with diesel-electric propulsion. Hybrid, zero-emission, and peak shaving modes with live energy flow diagram.

## Key Features

2,000 kWh BESS with SoC, cell voltage, temperature, and health monitoring.

Four operating modes: Diesel Only, Hybrid, Battery Only, Charging.

Peak shaving display comparing DG load with and without BESS.

Carbon emission meter in gCO2/kWh with real-time calculation.

Three scenarios: ECA Zero-Emission, DG Failure Takeover, Battery Low SoC Management.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| BATTERY ENERGY STORAGE (BESS) | Displays real-time system data, controls, and status indicators. |
| ENERGY FLOW DIAGRAM — HYBRID SYSTEM | Displays real-time system data, controls, and status indicators. |
| MODE CONTROL & STATUS | Displays real-time system data, controls, and status indicators. |
| EMISSIONS | Displays real-time system data, controls, and status indicators. |
| PEAK SHAVING COMPARISON | Displays real-time system data, controls, and status indicators. |
| SCENARIO INFO | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## Battery Hybrid Propulsion Control — Technical Reference

### Overview

Battery Energy Storage Systems (BESS) integrated with diesel-electric propulsion allow ships to operate with reduced emissions, improved fuel efficiency, and enhanced operational flexibility. Hybrid systems combine diesel generators with lithium-ion battery banks to optimise power delivery across all operating profiles.

### Regulatory Basis

**DNV Rules Pt.6 Ch.2** — Additional class notation for battery systems (Battery(Power), Battery(Safety))

**IEC 62619:2022** — Secondary cells and batteries — Safety requirements for stationary applications

**IEC 62620** — Secondary cells and batteries for use in industrial applications

**IGF Code (MSC.391(95))** — International Code of Safety for Ships Using Gases or Other Low-Flashpoint Fuels (future amendments to include battery guidance)

**SOLAS II-1** — Fire safety provisions for battery spaces

**IACS UR E27** — Use of lithium batteries on board ships

**EU ETS** — Emissions monitoring applicable to battery-assisted vessels

### System Architecture

#### Battery Pack Specifications

| Parameter | Value |

|-----------|-------|

| Chemistry | Lithium Iron Phosphate (LiFePO4) or NMC |

| Total Capacity | 2,000 kWh (typical medium vessel) |

| Nominal Voltage | 690 V DC |

| Max Charge Rate | 1,000 kW |

| Max Discharge Rate | 1,500 kW |

| Operating SoC Range | 10% – 90% (operating), 5%–95% absolute limits |

| Cell Temperature Range | 0°C – 45°C operating, < 35°C preferred |

| Battery Management System | BMS with cell-level monitoring |

#### Diesel Generators

DG1: 1,500 kW

DG2: 1,500 kW

Both feed to 440V / 60Hz AC bus

BESS connected via bidirectional inverter to same bus

### Operating Modes

#### 1. Diesel Only Mode

All load supplied by DG1 + DG2

Battery in standby (no charge/discharge)

Traditional diesel-electric operation

Used when: Battery SoC low or battery system offline

#### 2. Hybrid Mode (Peak Shaving)

DGs supply base load at optimum efficiency point

Battery covers load peaks (e.g., bow thruster start, manoeuvring)

Battery charges during low-demand periods

Benefit: DGs run at constant optimal load → lower fuel consumption, lower NOx

#### 3. Battery Only (Zero Emission Mode)

DGs stopped

All load supplied from battery bank

Used in: ECA zones, harbour approach, marina areas

Duration: limited by SoC and load — typically 1–4 hours depending on capacity

#### 4. Charging Mode

DGs run at increased load to charge battery

Excess DG output flows to battery via inverter

Also: shore power charging when available

### BESS Protection System

| Protection | Setpoint | Action |

|-----------|----------|--------|

| Over-temperature | > 45°C cell temp | Disconnect, activate cooling |

| Over-voltage | > 100% nominal cell voltage | Disconnect battery |

| Under-voltage | < 10% SoC | Disconnect, start DG |

| Over-current charge | > Max charge rate | Limit via BMS |

| Over-current discharge | > Max discharge rate | Limit via BMS |

| Insulation failure | < 50 kΩ | Alarm, inhibit charge |

| Thermal runaway detection | Gas/smoke sensors | Emergency ventilation, fire suppression |

### Peak Shaving Explained

Without BESS: DGs must be sized for maximum instantaneous demand (e.g., 3,000 kW for bow thruster operation). DGs run inefficiently at partial load most of the time.

With BESS: DGs sized for average load (e.g., 1,500 kW). Battery provides the transient peak. DGs run continuously at optimal load point (typically 70–85% rated power), improving:

Fuel consumption by 10–25%

NOx emissions by 15–30%

Maintenance intervals (less transient load cycling)

### Carbon Emission Calculation

Diesel generators: approximately 200–220 g/kWh fuel consumption (g diesel/kWh electrical output)

CO2 factor for marine diesel: approximately 3.17 g CO2/g fuel

Therefore: 630–700 g CO2/kWh from diesel generators

Battery discharge from renewable-charged grid: 0–50 g CO2/kWh (depending on source)

Battery discharge from DG-charged: same as DG but with round-trip efficiency loss (~15%)

### Vessel Suitability

Best suited to:

Ferries and RoPax (regular routes, predictable ports for charging)

OSVs/Platform supply vessels (transit + DP operations)

Harbour tugs (high transient demand + idle periods)

Research vessels (quiet/zero emission requirements)

Small cargo vessels (short-sea shipping)

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System