BlueSim Training Guide

Ammonia Cargo Tanker Simulator

Cargo Simulators

42,000 m³ fully refrigerated ammonia tanker — IGC Code Ch.19 and IMO MSC.1/Circ.1613. Full voyage from dry dock to discharge with PPE and IDLH monitoring.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

42,000 m³ fully refrigerated ammonia tanker — IGC Code Ch.19 and IMO MSC.1/Circ.1613. Full voyage from dry dock to discharge with PPE and IDLH monitoring.

## Key Features

4 prismatic tanks, reliquefaction, water curtain, and gas detection.

Emergency: deck leak, pump room flooding, manifold rupture, ESD.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| CARGO SYSTEM | Displays real-time system data, controls, and status indicators. |
| P&ID — NH₃ CARGO TANKER (IGC CODE CH.19) | Displays real-time system data, controls, and status indicators. |
| ALARM MANAGEMENT SYSTEM | Displays real-time system data, controls, and status indicators. |
| GAS DETECTION — NH₃ ppm (IDLH: 300 ppm) | Displays real-time system data, controls, and status indicators. |
| SCENARIO / TREND MONITOR | Displays real-time system data, controls, and status indicators. |
| PPE STATUS / SAFETY SYSTEMS | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.1 Gauges & Indicators

| Parameter | Unit |
| --- | --- |
| Tank 1 Pressure | bar |
| Tank 1 Temp | °C |
| Tank 1 Level | % |
| Tank 2 Pressure | bar |
| Tank 2 Level | % |
| Tank 3 Pressure | bar |
| Tank 3 Level | % |
| Tank 4 Level | % |
| Reliq Duty | % |

Colour coding: Green = normal, Amber = warning, Red (blinking) = alarm.

## 4.2 Valve Controls

Click a valve button to toggle its state. Valve states are colour-coded:

Green: OPEN

Red: CLOSED

Amber: TRANSITIONING

Valve Name

WATER CURTAIN

DECK DELUGE

PUMP-1

PUMP-2

STRIP

COMP

`

${v.label}

DOME-T${i+1}

## 4.3 Alarm Definitions

The alarm management system monitors the following conditions:

| Alarm ID | Severity | Description |
| --- | --- | --- |
| TANK1_HH | ALARM | Tank 1 — Pressure HIGH-HIGH (>0.5 bar) |
| TANK2_HH | ALARM | Tank 2 — Pressure HIGH-HIGH (>0.5 bar) |
| TANK3_HH | ALARM | Tank 3 — Pressure HIGH-HIGH (>0.5 bar) |
| TANK4_HH | ALARM | Tank 4 — Pressure HIGH-HIGH (>0.5 bar) |
| TANK_TEMP_HI | WARNING | Tank temp deviation — reliq check required |
| TANK_LO_LO | ALARM | Tank level LO-LO (<5%) — stripping required |
| RELIQ_FAIL | ALARM | Reliquefaction plant failure — pressure rising |
| PPM_25_DECK | WARNING | NH₃ 25 ppm — Manifold / Deck area |
| PPM_100_DECK | ALARM | NH₃ 100 ppm — Deck area EVACUATE |
| PPM_25_DOME | WARNING | NH₃ 25 ppm — Tank dome area |
| PPM_25_PUMP | WARNING | NH₃ 25 ppm — Pump room |
| PPM_100_PUMP | ALARM | NH₃ 100 ppm — Pump room EVACUATE |
| PPM_IDLH | ALARM | IDLH 300 ppm EXCEEDED — SCBA MANDATORY |
| ESD_FIRED | ALARM | EMERGENCY SHUTDOWN ACTIVATED — All valves closed |
| PUMP_FAIL | ALARM | Cargo pump failure — check motor overload |
| IG_FAIL | WARNING | Inert gas O₂ high — check IG generator |
| TANK_OVERFLOW | ALARM | Tank level overflow risk (>98%) |

## 4.4 Pipeline Line-Up

The pipeline panel allows valve-by-valve line-up verification for each operational mode.

| Valve ID | Description |
| --- | --- |
| T1_IV | No.1 Tank Dome Isolation Valve |
| T2_IV | No.2 Tank Dome Isolation Valve |
| T3_IV | No.3 Tank Dome Isolation Valve |
| T4_IV | No.4 Tank Dome Isolation Valve |
| CP1_DV | Cargo Pump 1 Discharge Valve |
| CP2_DV | Cargo Pump 2 Discharge Valve |
| LH_XV | Liquid Header Crossover Valve |
| MAN_P | Port Manifold Isolation |
| MAN_S | Stbd Manifold Isolation |
| VRL_MAN | Vapour Return Line Manifold |
| RELIQ_IN | Reliq Unit Suction Isolation |
| RELIQ_OUT | Reliq Unit Discharge Isolation |
| N2_IV | N\u2082 Hold Space Purge Valve |
| SCBA_HK | SCBA Connection Valve (verify) |
| WATER_CUR | Water Curtain Isolation Valve |
| STRIP_IV | Stripping Line Isolation |

# 5. Training Scenarios

Select a scenario from the modal dialog when the simulator loads. Each scenario has specific learning objectives and a checklist of actions to complete.

## 5.1 NORMAL LOADING OPERATIONS

Loading anhydrous NH₃ at −33°C. Monitor tank levels, pressures, reliq plant. Checklist per IGC Ch.19.

### Actions Required

Step 1: Confirm all tanks inerted (O₂ <2%)

Step 2: Cool down tanks to −33°C

Step 3: Open manifold valve

Step 4: Start cargo pump or gravity load

Step 5: Monitor tank levels every 15 min

Step 6: Confirm reliq plant running

## 5.2 DECK LEAK — NH₃ RELEASE

Manifold flange failure. Gas rising on deck. Evacuate area, activate ESD, water curtain.

### Actions Required

Step 1: Acknowledge deck area gas alarm

Step 2: Activate water curtain

Step 3: Close manifold valve

Step 4: Activate Emergency Shutdown (ESD)

Step 5: Don SCBA before deck entry

Step 6: Muster all crew at safe haven

## 5.3 PUMP ROOM FLOODING

Sea water ingress. NH₃ liquid contact generating vapour. Emergency bilge, muster crew.

### Actions Required

Step 1: Sound general alarm

Step 2: Stop cargo pumps

Step 3: Close pump room flooding valve

Step 4: Start emergency bilge pump

Step 5: Don SCBA — pump room entry

## 5.4 MANIFOLD RUPTURE

Major cargo pipe failure. Rapid NH₃ release. Full emergency — ESD, deluge, abandon area.

### Actions Required

Step 1: Sound ABANDON AREA alarm

Step 2: Activate Emergency Shutdown

Step 3: Activate deck deluge system

Step 4: Muster all crew — bridge muster

Step 5: Notify coastguard / terminal

Step 6: Prepare emergency tow-off

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System