BlueSim Training Guide

Cargo Compressor (LPG/LNG)

Cargo & Tanks

LPG/LNG gas carrier cargo compressor trainer covering HD and LD reciprocating compressors, vapour return, BOG compression, and safety trip responses.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

LPG/LNG gas carrier cargo compressor trainer covering HD and LD reciprocating compressors, vapour return, BOG compression, and safety trip responses.

## Key Features

HD and LD compressor simulation with live gauges.

Vapour return, BOG compression, and tank pressure control modes.

Safety trip logic: high discharge temp, LO low pressure, vibration, overcurrent.

Valve sequencing with 1.5s transition delay.

Inlet separator liquid level monitoring.

Audible alarms with ACK ALL function.

Coolant system and capacity control (bypass unloading).

Four training scenarios with guided procedures.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| HD COMPRESSOR | Displays real-time system data, controls, and status indicators. |
| SYSTEM SCHEMATIC — CARGO COMPRESSOR UNIT | Displays real-time system data, controls, and status indicators. |
| LD COMPRESSOR | Displays real-time system data, controls, and status indicators. |
| ALARMS &amp; TRIPS | Displays real-time system data, controls, and status indicators. |
| OPERATIONS LOG | Displays real-time system data, controls, and status indicators. |
| COOLANT &amp; SCENARIO | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.2 Valve Controls

Click a valve button to toggle its state. Valve states are colour-coded:

Green: OPEN

Red: CLOSED

Amber: TRANSITIONING

Valve Name

SUCT [O]

DISCH [O]

BYPASS [C]

LIQ RET [O]

SEAL GAS [O]

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## Cargo Compressor Simulator — Technical Reference

### Overview

Gas carrier cargo compressors are used on LPG and LNG carriers to handle vapour during cargo operations. This simulator covers both High Duty (HD) and Low Duty (LD) reciprocating-type compressors as fitted on modern LPG carriers.

### Compressor Types

#### High Duty (HD) Compressor

**Application**: Vapour return to shore during loading, BOG (Boil-Off Gas) compression

**Type**: Reciprocating, double-acting

**Capacity**: Up to 4,500 m³/h vapour at inlet conditions

**Motor**: 185–250 kW, 440V AC

**Speed**: 400–600 RPM

**Max Discharge Pressure**: 8 bar g (LPG propane/butane service)

#### Low Duty (LD) Compressor

**Application**: Tank pressure control, minor BOG management

**Type**: Reciprocating, single-acting

**Capacity**: Up to 1,500 m³/h

**Motor**: 55–75 kW, 440V AC

**Speed**: 300–500 RPM

**Max Discharge Pressure**: 6 bar g

### System Components

#### Inlet Separator

Liquid knock-out drum before compressor suction

Liquid level monitored — high level triggers compressor stop

Fitted with safety relief valve

#### Inter/After Coolers

Shell-and-tube heat exchangers (seawater-cooled)

Inter-cooler: fitted between stages on two-stage compressors

After-cooler: cools discharge gas before delivery

Fouling of seawater side raises discharge temperature

#### Lube Oil System

Splash/forced lubrication for bearings and cylinder walls

LO pre-lube pump runs before start (3–5 min)

LO pressure trip: < 1.5 bar stops compressor

LO temperature monitored (max 65°C)

#### Seal Gas System

Prevents cargo gas escaping along shaft

Inert gas (N₂) or condensate vapour seal

Seal gas pressure maintained > crankcase pressure

### Valves

| Valve | Function | Normal State |

|-------|----------|-------------|

| Suction Stop | Isolates compressor from suction line | Open |

| Discharge Stop | Isolates compressor from discharge line | Open |

| Bypass (Unloading) | Recirculates gas for capacity control | Closed (running) |

| Liquid Return | Returns separated liquid to cargo tank | Open |

| Seal Gas Supply | Feeds seal gas to shaft seals | Open |

### Operating Parameters

#### Normal Operating Ranges

| Parameter | Low Duty | High Duty |

|-----------|----------|-----------|

| Suction Pressure | 0.2–0.8 bar g | 0.2–0.8 bar g |

| Discharge Pressure | 4–6 bar g | 6–8 bar g |

| Motor Current | 30–65 A | 80–180 A |

| LO Pressure | 2.5–4.5 bar | 3.0–5.0 bar |

| Vibration | < 4.5 mm/s | < 6.0 mm/s |

| Gas Temp In | 0–30°C | 0–30°C |

| Gas Temp Out | 40–90°C | 50–100°C |

### Safety Trips

| Trip | Set Point | Action |

|------|-----------|--------|

| High Suction Pressure | > 1.5 bar g | Stop compressor |

| High Discharge Temperature | > 130°C | Stop compressor |

| LO Low Pressure | < 1.5 bar | Stop compressor |

| Vibration High | > 7.0 mm/s | Stop compressor |

| Inlet Separator High Liquid Level | > 80% | Stop compressor |

| Motor Overcurrent | > 110% FLC | Stop motor |

### Operational Modes

#### Vapour Return During Loading

Shore pumps cargo liquid into tanks → vapour is displaced upward → compressor draws vapour from cargo tanks → returns vapour to shore via vapour return line. Maintains cargo tank pressure within limits.

#### BOG Compression

Boil-off gas generated during sea passage or discharge is compressed by HD compressor → delivered at elevated pressure to condensing system or reliquefaction plant → condensate returned to cargo tanks.

#### Tank Pressure Control (LD Mode)

Low Duty compressor maintains cargo tank pressure within operating range (typically 0.05–0.2 bar g above atmospheric) during sea passage.

#### Capacity Control

Bypass valve opens to recirculate portion of discharge gas back to suction

Step-unloaders can cut individual cylinder heads (reciprocating type)

Variable speed: not commonly fitted on reciprocating cargo compressors

Typical capacity steps: 100%, 75%, 50%, 25%

### Coolant System

Seawater-cooled inter/after coolers

SW inlet temperature: ≤ 32°C design

SW flow rate typically 20–40 m³/h per cooler

High SW temp → higher discharge temp → potential trip

### Start Sequence (Normal)

1. Confirm suction/discharge stop valves open

2. Start LO pre-lube pump; confirm LO pressure ≥ 1.5 bar

3. Confirm inlet separator liquid level LOW

4. Open seal gas supply

5. Crack open bypass valve

6. Start compressor motor — confirm ampere draw normal

7. Gradually close bypass valve — load up

8. Monitor suction/discharge pressure, temperature, vibration

### Stop Sequence (Normal)

1. Open bypass valve fully (unload)

2. Stop compressor motor

3. Close suction stop valve

4. Close discharge stop valve

5. Run LO post-lube for 5 minutes

6. Isolate seal gas

### Regulatory References

**IGC Code Chapter 5**: Cargo containment and process systems

**SOLAS II-2 Regulation 4**: Fire protection (gas carrier)

**IACS UR M28**: Cargo compressors — survey requirements

**ICS Tanker Safety Guide (Gas)**: Chapter 7

### Alarms Summary

| Alarm Tag | Description | Level |

|-----------|-------------|-------|

| CC-001-H | HD Compressor — High Discharge Temp | A |

| CC-002-L | HD Compressor — LO Low Pressure | A |

| CC-003-H | HD Compressor — High Vibration | A |

| CC-004-H | HD Compressor — High Suction Pressure | A |

| CC-005-H | Inlet Separator — High Liquid Level | W |

| CC-006-H | HD Compressor — Overcurrent | A |

| CC-011-H | LD Compressor — High Discharge Temp | W |

| CC-012-L | LD Compressor — LO Low Pressure | A |

| CC-013-H | LD Compressor — High Suction Pressure | W |

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System