BlueSim Training Guide

Crude Oil Tanker Simulator

Cargo Simulators

VLCC 300,000 DWT — MARPOL Annex I, ISGOTT 6th Ed, SOLAS II-2. Full COW, IG system, 14 cargo tanks, and ballast management across 10 voyage phases.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

VLCC 300,000 DWT — MARPOL Annex I, ISGOTT 6th Ed, SOLAS II-2. Full COW, IG system, 14 cargo tanks, and ballast management across 10 voyage phases.

## Key Features

IG deck pressure, O₂%, COW washing progress, top-deck canvas view.

Emergency: IG failure, cargo overflow, pump room fire, manifold leak.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| CARGO / IG SYSTEM | Displays real-time system data, controls, and status indicators. |
| P&ID — VLCC CRUDE OIL TANKER (TOP-DECK VIEW) | Displays real-time system data, controls, and status indicators. |
| ALARM MANAGEMENT | Displays real-time system data, controls, and status indicators. |
| TANK LEVELS — 14 CARGO TANKS | Displays real-time system data, controls, and status indicators. |
| SCENARIO / TREND | Displays real-time system data, controls, and status indicators. |
| BALLAST / FIRE / IG | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.1 Gauges & Indicators

| Parameter | Unit |
| --- | --- |
| IG Deck Press | mmWG |
| IG O₂ Content | % |
| Tank 1C Level | % |
| Tank 2C Level | % |
| Tank 1P Level | % |
| Tank 1S Level | % |
| Slop P Level | % |

Colour coding: Green = normal, Amber = warning, Red (blinking) = alarm.

## 4.2 Valve Controls

Click a valve button to toggle its state. Valve states are colour-coded:

Green: OPEN

Red: CLOSED

Amber: TRANSITIONING

Valve Name

BALLAST-P1

BALLAST-P2

IG BLOWER

IG SCRUBBER

DECK SEAL

CP-1

CP-2

CP-3

STRIP

`

${v.label}

IG-FWD

IG-MID

IG-AFT

COW SUP

P/V BRAKER

## 4.3 Alarm Definitions

The alarm management system monitors the following conditions:

| Alarm ID | Severity | Description |
| --- | --- | --- |
| IG_PRESS_LOW | ALARM | IG deck pressure LOW (<20 mmWG) — tank atmosphere risk |
| IG_O2_HIGH | ALARM | IG O₂ content HIGH (>8%) — unsafe atmosphere |
| IG_O2_WARN | WARNING | IG O₂ content elevated (>5%) — check IG generator |
| IG_FAIL | ALARM | Inert gas system FAILURE — blower tripped |
| TANK_OVERFLOW | ALARM | Cargo overflow risk — tank level >98% |
| TANK_OVERFLOW_WARN | WARNING | High tank level warning (>95%) |
| PUMP_CAVIT | WARNING | Cargo pump cavitation — check suction pressure |
| TRIM_ALARM | WARNING | Trim exceeds 3° — redistribute ballast/cargo |
| LIST_ALARM | ALARM | List >2° — immediate ballast correction required |
| FIRE_PUMP | ALARM | FIRE DETECTED — Pump room — STOP ALL PUMPS |
| FIRE_ENGINE | ALARM | FIRE ALARM — Engine room |
| MANIFOLD_LEAK | ALARM | Vapour detected at manifold — possible crude leak |
| PV_OPEN | WARNING | P/V valve lifting — tank pressure high |
| ESD_FIRED | ALARM | EMERGENCY SHUTDOWN ACTIVATED |
| HIGH_VAPOUR_PRESS | WARNING | High vapour pressure in void space — check ventilation |
| LINEUP_FAULT | H | PUMP LINEUP FAULT — Pipeline incorrectly lined up |

## 4.4 Pipeline Line-Up

The pipeline panel allows valve-by-valve line-up verification for each operational mode.

| Valve ID | Description |
| --- | --- |
| P1_3_HDR | Port No.1–3 Tank Header Valve |
| P4_6_HDR | Port No.4–6 Tank Header Valve |
| S1_3_HDR | Stbd No.1–3 Tank Header Valve |
| S4_6_HDR | Stbd No.4–6 Tank Header Valve |
| CX_PS | Port-Stbd Crossover Valve |
| CP1_SV | Cargo Pump 1 Suction Valve |
| CP1_DV | Cargo Pump 1 Discharge Valve |
| CP2_SV | Cargo Pump 2 Suction Valve |
| CP2_DV | Cargo Pump 2 Discharge Valve |
| CP3_SV | Cargo Pump 3 Suction Valve |
| CP3_DV | Cargo Pump 3 Discharge Valve |
| MAN_P_FWD | Port Manifold Fwd Isolation |
| MAN_P_AFT | Port Manifold Aft Isolation |
| MAN_S_FWD | Stbd Manifold Fwd Isolation |
| COW_IV | COW Machine Isolation Valve |
| COW_P | COW Port Wing Wash Valve |
| COW_S | COW Stbd Wing Wash Valve |
| STRIP_IV | Stripping Pump Isolation Valve |
| STRIP_DV | Stripping Pump Discharge Valve |
| IG_DV | Inert Gas Deck Main Valve |
| IG_XV | IG Crossover (port-stbd tanks) |
| BALLAST_SV | Ballast Pump Suction Sea Valve |
| BALLAST_DV | Ballast Pump Discharge Valve |

# 5. Training Scenarios

Select a scenario from the modal dialog when the simulator loads. Each scenario has specific learning objectives and a checklist of actions to complete.

## 5.1 NORMAL DISCHARGE OPERATIONS

Discharging crude oil at terminal. IG maintained positive. 3 cargo pumps at 4000 m³/h each.

### Actions Required

Step 1: Confirm IG system running — O₂ <5%

Step 2: Verify tank pressures positive

Step 3: Open manifold valve

Step 4: Start cargo pumps sequentially

Step 5: Monitor trim and list

Step 6: Check P/V valves every 30 min

## 5.2 IG FAILURE AT SEA

IG blower tripped. O₂ rising in tank atmospheres. Emergency IG procedure per ISGOTT §9.

### Actions Required

Step 1: Acknowledge IG failure alarm

Step 2: Start standby IG blower

Step 3: Check scrubber water flow

Step 4: Monitor tank O₂ — do NOT discharge if >8%

Step 5: Notify master — ISGOTT emergency

## 5.3 CARGO OVERFLOW RISK

Loading rate excessive. Tanks 1C and 2C approaching capacity. Immediate action.

### Actions Required

Step 1: Acknowledge high level alarm

Step 2: Reduce loading rate immediately

Step 3: Redistribute to wing tanks

Step 4: Notify terminal of rate reduction

Step 5: Confirm level <95% in all tanks

## 5.4 COW OPERATION

Crude oil washing per ISGOTT §11 during discharge. COW machines running, IG confirmed positive.

### Actions Required

Step 1: Confirm IG positive (+50 mmWG min)

Step 2: Activate COW operation button

Step 3: Set washing pressure 8-10 bar

Step 4: Monitor COW coverage to 100%

Step 5: Confirm stripping pump on completion

## 5.5 FIRE IN PUMP ROOM

Fire alarm actuated in pump room. Stop all pumps, activate CO₂ fixed system, muster.

### Actions Required

Step 1: Acknowledge pump room fire alarm

Step 2: Stop ALL cargo pumps immediately

Step 3: Close pump room ventilation fans

Step 4: Activate CO₂ fixed system

Step 5: Sound general alarm — muster all crew

Step 6: Notify coastguard

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System