BlueSim Training Guide

LPG Tanker Cargo Simulator

Cargo Simulators

VLGC 84,000 m³ fully refrigerated — IGC Code 2016, STCW V/1-2. Carries propane, butane, ammonia, and VCM with Type A prismatic tanks.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

VLGC 84,000 m³ fully refrigerated — IGC Code 2016, STCW V/1-2. Carries propane, butane, ammonia, and VCM with Type A prismatic tanks.

## Key Features

11 phases: purging, gassing-up, cooling, loading, discharge, cargo change.

Emergency: reliq failure, liquid deck release, ammonia exposure, ESD.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| PROPANE | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.4 Pipeline Line-Up

The pipeline panel allows valve-by-valve line-up verification for each operational mode.

| Valve ID | Description |
| --- | --- |
| T1_IV | No.1 Tank Dome Isolation Valve |
| T2_IV | No.2 Tank Dome Isolation Valve |
| T3_IV | No.3 Tank Dome Isolation Valve |
| T4_IV | No.4 Tank Dome Isolation Valve |
| CP1_DV | Deepwell Pump 1 Discharge Valve |
| CP2_DV | Deepwell Pump 2 Discharge Valve |
| LH_XV | Liquid Header Crossover Valve |
| HEATER_IV | Cargo Heater Inlet Isolation |
| MAN_P | Port Manifold Isolation Valve |
| MAN_S | Stbd Manifold Isolation Valve |
| VRL_IV | Vapour Return Line Isolation |
| VRL_MAN | Vapour Return Line Manifold |
| RELIQ_IN | Reliq Unit Suction Isolation |
| RELIQ_OUT | Reliq Unit Discharge Isolation |
| COMP1_SV | Compressor 1 Suction Valve |
| COMP2_SV | Compressor 2 Suction Valve |
| N2_IV | N₂ Hold Space Inlet Valve |

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## 01 LPG Tanker Reference

## LPG Tanker (VLGC) Cargo Simulator — Regulatory Reference

### IGC Code 2016 · STCW V/1-2 · SIGTTO · −55°C Fully Refrigerated · Type A Tanks

---

### 1. VLGC vs LNG Carrier Key Differences

| Feature | LNG Carrier | VLGC (LPG) |

|---------|------------|-------------|

| Temperature | −162°C | −55°C (propane −42°C) |

| Pressure | Atmospheric | Atmospheric (fully refrigerated) |

| Tank type | Membrane or Moss Type B | Type A prismatic |

| BOG | 0.07–0.15%/day | Lower — better insulation ratio |

| Secondary barrier | Required (independent) | Required — hull acts as secondary |

| Hold space | Inerted with N₂/IG | Inerted when flammable cargo |

| Discharge | Deepwell to re-gas terminal | To pressurised OR refrigerated storage |

| Cargoes | Methane only | Propane, Butane, Ammonia, VCM, Propylene |

| Size | 125,000–266,000 m³ | 50,000–84,000 m³ (VLGC) |

---

### 2. Cargo Types on VLGC

#### Propane (C₃H₈)

Boiling point: −42°C at atmospheric pressure

Vapour density: 1.5× air — **SINKS**

LEL: 2.1%, UEL: 9.5%

Detectors at FLOOR LEVEL

Fully refrigerated at −42°C / near atmospheric

#### Butane (C₄H₁₀)

Boiling point: −1°C at atmospheric pressure

Even denser vapour than propane

Sometimes carried at slightly elevated pressure when ambient temperature > boiling point

LEL: 1.8%, UEL: 8.4%

#### Ammonia (NH₃)

Boiling point: −33°C

**TOXIC** — IDLH 300 ppm

Vapour density: 0.6× air — rises (unlike propane)

Completely incompatible with copper, brass, zinc — all fittings must be SS or carbon steel

Requires SCBA for all cargo connection work

Maximum allowable concentration (MAC): 25 ppm (8-hour)

Water-soluble — dilution with water effective for spills

#### VCM (Vinyl Chloride Monomer) / Propylene

Additional refrigeration requirements

Strict compatibility checks with tank materials and seals

---

### 3. IGC Code — Type A Tank Requirements

Type A tanks — prismatic, flat-surfaced, atmospheric boiling point cargo:

Maximum design pressure: 0.7 barg

**Secondary barrier: MANDATORY — must contain complete tank volume at defined heel**

Secondary barrier must contain leakage for ≥ 15 days

Hold space between primary and secondary barrier: inerted with N₂/IG when carrying flammable cargoes

Hold space O₂ < 2% at all times (flammable cargoes)

Material: Carbon-manganese steel or special alloy (for −55°C, fine-grain C-Mn steel; below −55°C requires nickel alloy)

PRV (Pressure Relief Valve) settings for Type A:

Typical set pressure: 0.25 barg

If tank pressure rises above PRV setting → vents to vent mast

This indicates: reliquefaction failed or BOG exceeds reliq capacity

---

### 4. Reliquefaction Plant

Key difference from LNG carriers: LPG reliquefaction is simpler (higher boiling points):

Refrigerant: usually the cargo itself (propane as refrigerant for propane service)

Two-stage compression typical

First stage: suction from tank → intermediate pressure

Second stage: compress to condensing pressure

Condenser: seawater or air-cooled

Condensate returns to tank

If reliquefaction fails:

1. Tank pressure rises

2. PRV may lift → controlled vent to mast

3. Reduce cargo temperature by connecting to emergency cooling

4. Contact reliq maker and class

---

### 5. Hold Space Inerting

IGC Code requirement — when carrying flammable cargo:

Hold spaces continuously inerted

O₂ < 2% vol

N₂ generator or IG generator supplies inert gas

HC detectors monitor hold spaces — any HC rise indicates secondary barrier leak

If HC detected: ESD standby, investigate, reduce cargo tank pressure

---

### 6. Discharge — Pressurised vs Refrigerated Storage

VLGC can discharge to both types (more flexible than LNG):

Refrigerated shore storage:

Discharge at low temperature — liquid pumped directly

Simple — minimal cargo heating required

Most large import terminals

Pressurised shore storage:

Cargo must be warmed to above boiling point before discharge

Cargo heater warms LPG

Booster pumps increase discharge pressure

Typical at smaller regional terminals

---

### 7. Cargo Change Procedure

When changing from one cargo grade to another (e.g., propane to ammonia):

1. Pump previous cargo to lowest possible residue

2. Purge with inert gas (N₂) — purge previous cargo vapour from tank

3. Confirm O₂ and HC levels after purging

4. Check compatibility matrix — new vs previous cargo

5. Review MSDS for new cargo

6. Update IGC COF if new cargo type not previously approved

7. Check all fittings, seals, gaskets for compatibility with new cargo

Critical incompatibilities:

Ammonia AFTER any hydrocarbon: complete purge required — avoid formation of explosive mixtures

Any cargo AFTER ammonia: purge required — residual ammonia is toxic

---

### 8. LPG as Ship Fuel (IGC Code §16.9)

Modern VLGCs increasingly use cargo LPG as ship fuel:

Approved per IGC Code §16.9 — use of cargo as fuel

MAN ES ME-LGI engine — dual fuel, LPG injection

ABS, LR, DNV all have class notations

Fuel supply system (FGSS) separate from cargo system

Significant commercial advantage: eliminates or reduces bunker fuel cost on large voyages (Middle East → Japan)

---

### 9. Emergency Procedures

#### Reliquefaction Failure

1. Monitor tank pressure — if rising

2. Reduce speed if possible — less heat input

3. Prepare vent mast for controlled release

4. Contact technical superintendent

#### LPG Liquid Release on Deck

LPG vapour SINKS — accumulates in bilge, low spaces

No ignition sources — remove all hot work permits

Activate water curtain

Evacuate downwind

Allow controlled vaporisation — do not apply water to liquid

#### Ammonia Exposure

SCBA mandatory at IDLH 300 ppm

Move victim to fresh air upwind

Water wash if skin contact

Activate water spray to dilute vapour

Medical attention mandatory for any exposure >25 ppm

---

### 10. Sources

1. IGC Code (IMO MSC.391(95), 2016) — Chapter 4 (tank types), Chapter 8 (hold spaces), Chapter 16 (cargo as fuel)

2. STCW V/1-2 — Advanced Training for Liquefied Gas Tanker Cargo Operations

3. SIGTTO — Ship/Shore Safety Checklist for LPG Tankers

4. cultofsea.com — Gas Tanker Types, Tanks, Reliquefaction (July 2019)

5. Wikipedia — Gas carrier (October 2025) — VLGC specifications and tank types

6. sea-man.org — LNG/LPG Reliquefaction plant (June 2024)

7. owaysonline.com — Liquefied Gas Cargoes — Type A tanks (February 2025)

8. sea-man.org — Tank types for liquefied gas (November 2022)

9. Wärtsilä Gas Solutions — Cargo handling systems for VLEC/VLGC

10. Riviera MMR — "Five lessons about VLEC design" (ABS experience)

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System