BlueSim Training Guide

Cargo Pump Operation & Control

Cargo & Tanks

Pump room operations simulator for crude/product tankers with 3 cargo pumps, stripping, ballast and full discharge sequence training.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Pump room operations simulator for crude/product tankers with 3 cargo pumps, stripping, ballast and full discharge sequence training.

## Key Features

Three centrifugal cargo pumps with variable speed control, flow rate and motor current monitoring.

Stripping pump operation with automatic cut-in at 2% tank level.

Ballast pump with segregated system and ballast level tracking.

Four training scenarios: full discharge, stripping, emergency stop and ballasting.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| PUMP CONTROLS | Displays real-time system data, controls, and status indicators. |
| PUMP ROOM & CARGO PIPELINE SCHEMATIC | Displays real-time system data, controls, and status indicators. |
| MANIFOLD & ALARMS | Displays real-time system data, controls, and status indicators. |
| PIPELINE LINEUP STATUS | Displays real-time system data, controls, and status indicators. |
| DISCHARGE RATE TREND | Displays real-time system data, controls, and status indicators. |
| OPERATIONS LOG | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.2 Valve Controls

Click a valve button to toggle its state. Valve states are colour-coded:

Green: OPEN

Red: CLOSED

Amber: TRANSITIONING

Valve Name

PORT MFD [C]

STBD MFD [C]

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## Cargo Pump Operation & Control — Technical Reference

### Regulatory Framework

**MARPOL Annex I**: Regulations for the prevention of pollution by oil; controls cargo pump operations, slop tank management and residue handling.

**ICS Tanker Safety Guide (Petroleum) 6th Ed.**: Industry standard for safe tanker pump room operations.

**STCW II/1**: Officer in Charge of a Navigational Watch — cargo operations competency including pump monitoring.

**OCIMF MEG4**: Mooring Equipment Guidelines; manifold and hose connection standards.

**Classification Society Rules (DNV/LR/ABS)**: Pump room ventilation, bilge alarm, gas detection requirements.

### Pump Types on Oil Tankers

#### Centrifugal Cargo Pumps (main)

Type: Vertical or horizontal centrifugal, submerged or deck-mounted turbine driven

Typical capacity: 1,000–3,000 m³/h per pump

Discharge pressure: 8–12 bar (typically)

Drive: Electric motor (high voltage, 6.6 kV or 11 kV) or steam turbine

Speed control: Variable frequency drive (VFD) or turbine governor

#### Stripping Pump

Type: Reciprocating or centrifugal (smaller)

Purpose: Remove final cargo residues when centrifugal pump loses suction (< 1–2% tank level)

Capacity: 50–200 m³/h

Operates when cargo level drops below stripping cut-in level

#### Ballast Pump

Type: Centrifugal

Purpose: Loading/discharging ballast water

Segregated system: No connection to cargo piping (MARPOL requirement)

Capacity: 1,000–2,500 m³/h

### Key Performance Parameters

| Parameter | Normal | Warning | Alarm |

|-----------|--------|---------|-------|

| Pump speed | 50–100% | < 30% | Overload |

| Discharge pressure | 8–11 bar | > 12 bar | > 13 bar |

| Flow rate | 800–2500 m³/h | < 200 m³/h | Zero flow |

| Motor current | 70–95% FLA | > 95% FLA | > 105% FLA |

| Pump room temp | < 45°C | > 50°C | > 55°C |

| Bilge level | Dry | 50 mm | > 100 mm |

| Shaft seal leakage | Negligible | Visible | Major |

### Tank Configuration

Typical crude/product tanker:

**5-cargo tank arrangement**: No. 1–5 port and starboard + slop tanks

Centre tanks (C): used on VLCC for structural reasons

Each tank has: suction valve, crossover valve, ullage plug/gauging port

### Cargo Discharge Sequence (typical)

1. Check tank gauging — confirm level, trim, stability

2. Line up pump suction from source tanks

3. Line up discharge via manifold to shore terminal

4. Start pump on minimum speed

5. Gradually increase speed, confirm flow to terminal

6. Monitor: pressure, flow, current, tank levels

7. When level < 2%, switch to stripping operations

8. Stripping complete when suction lost and tank is dry

### Stripping Operations

Cut-in: when cargo level ≈ 1–2% (approximately 300–600 mm)

Use stripping pump or eductor

Final stripping important for MARPOL compliance (minimise residues)

Document in Oil Record Book (ORB) Part I

### Ballasting During Discharge

Segregated Ballast Tanks (SBT): MARPOL Annex I requirement for tankers > 20,000 DWT

Ballasting sequence: avoid excessive trim changes during cargo discharge

Ballast pump runs simultaneously with cargo discharge

Monitor: draft forward/aft, list, GM during ballasting

### Alarm Conditions

| Alarm | Cause | Action |

|-------|-------|--------|

| High pump temperature | Bearing failure, low cooling flow | STOP pump, investigate |

| High motor current | Overload, mechanical fault | Reduce speed or STOP |

| Shaft seal leakage | Seal wear | Monitor; STOP if major |

| Pump room bilge high | Cargo spill, seal failure | Investigate immediately |

| Tank overflow | Level sensor failure, over-filling | STOP loading pump |

| No-flow alarm | Cavitation, valve closed | Check lineup |

### Emergency Procedures

**Emergency pump stop**: Each pump has local and remote emergency stop

**Pump room fire**: Activate fixed CO₂ system, close ventilation dampers

**SOPEP**: Shipboard Oil Pollution Emergency Plan procedures for spills

**Pump room entry**: Only when ventilated, gas-free, permit-to-work issued

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System