BlueSim Training Guide

Crane & Derrick Operation

Deck Equipment

Ship's deck crane and derrick operations trainer with animated SVG crane schematic, SWL chart, limit switches, ATB protection, and wind speed trip.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Ship's deck crane and derrick operations trainer with animated SVG crane schematic, SWL chart, limit switches, ATB protection, and wind speed trip.

## Key Features

Animated SVG crane with jib angle, hook position, and load indicator.

SWL vs radius chart — SWL decreases as radius increases.

Hoist, slew, and luff controls with variable speed.

Limit switches: hoist upper limit, slew limits, luff limits.

Anti-Two-Block (ATB) protection simulation.

Wind speed indicator with automatic trip at 25 knots.

Emergency stop and emergency lower controls.

Lift log records each operation with load and parameters.

Four scenarios: Provision Stores, Overload Test, Wind Trip, Limit Switch Response.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| CRANE CONTROLS | Displays real-time system data, controls, and status indicators. |
| CRANE SCHEMATIC — DECK VIEW | Displays real-time system data, controls, and status indicators. |
| LOAD DATA &amp; SWL CHART | Displays real-time system data, controls, and status indicators. |
| ALARMS | Displays real-time system data, controls, and status indicators. |
| OPERATIONS LOG | Displays real-time system data, controls, and status indicators. |
| SCENARIO | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## Crane / Derrick Operation Simulator — Technical Reference

### Overview

Ship's deck cranes and derricks are used for cargo handling, provision lifting, and stores operations. This simulator covers a provision crane (5T SWL) and cargo derrick configuration as fitted on LPG carriers and product tankers.

### Provision Crane (5T SWL)

#### Specifications

**Safe Working Load (SWL)**: 5,000 kg at minimum radius

**Maximum radius**: 12 m

**Minimum radius**: 4 m

**Hoist speed**: 0–25 m/min (variable)

**Slew speed**: 0–1.5 rpm

**Luff speed**: 0–3°/sec

**Maximum jib angle**: 75°

**Minimum jib angle**: 25° (at maximum radius)

**Hoist height**: 25 m above deck

#### SWL vs Radius Chart (Provision Crane)

| Radius (m) | Max SWL (kg) |

|------------|-------------|

| 4 | 5,000 |

| 5 | 4,200 |

| 6 | 3,500 |

| 7 | 3,000 |

| 8 | 2,500 |

| 9 | 2,100 |

| 10 | 1,800 |

| 11 | 1,500 |

| 12 | 1,300 |

#### Drive System

Hydraulic power pack (typically 37–75 kW)

Hoist, slew, and luff each driven by separate hydraulic motor

Emergency power: manual hand pump for emergency lower only

### Derrick System

#### Specifications

**SWL**: 5,000–25,000 kg (derrick dependent)

**Topping lift**: Wire rope from masthead, controls boom angle

**Guy lines**: Lateral stability wires

**Cargo runner**: Hoisting line through head block and heel block

#### Derrick Operations

**Union purchase rig**: Two derricks used together for swinging cargo

**Single swinging derrick**: One derrick swings load from ship to shore

Boom angle controlled by topping lift winch

### Limit Switches

#### Hoist Upper Limit

Activates when hook reaches maximum height (approx. 0.5 m before head block)

Prevents two-blocking (ATB — Anti Two-Block protection)

IMMEDIATE hoist stop; pays out permitted

Reset: lower hook manually below limit switch level

#### Slew Limits

Port limit: usually -180° from centreline

Stbd limit: +180° from centreline (some cranes: ±270°)

Prevents hydraulic hose damage by overrotation

#### Luff Limits

Maximum up (over-topping): usually 75° from horizontal

Minimum (maximum radius): 25° — prevents mechanical jam

Load moment limit: jib prevented from luffing out when load is too heavy for that radius

### Anti-Two-Block (ATB) Protection

Sensor on top block; trigger weight on hook block

When trigger weight contacts top block, circuit breaks

Hoist-up motion stops immediately

Audible alarm and indicator light

### Wind Speed Limit

**Provision crane**: operations cease at sustained wind > 25 knots

**Cargo crane**: operations cease at sustained wind > 30 knots

**Heavy lift crane**: may operate to higher wind speeds per manufacturer

Automatic trip at set wind speed (anemometer-linked interlock on some cranes)

### Load Swing Damping

Experienced crane operator uses slow slew/luff to minimize pendulum effect

Some modern cranes: electronic anti-sway (predicts/counteracts swing)

Manual damping: slow movements, allow load to settle before next motion

### Emergency Controls

**Emergency Stop**: Immediately stops all motions; hydraulic brakes apply

**Emergency Lower**: Manual lowering of load at slow speed even on E-stop

Dead-man switch: must be held during operation (some cranes)

### Safe Lifting Procedures

1. Pre-lift inspection: hook condition, wire condition, shackles, lifting gear

2. Confirm SWL vs load weight (weighing ticket or estimate)

3. Check overhead obstructions, personnel exclusion zone

4. Attach load properly — correct sling angle, balanced load

5. Test lift: raise a few cm, check stability before full hoist

6. Lift slowly, communicate with banksman/rigger

7. Set load down slowly, release tension before unhooking

### Lift Log

Each lift recorded:

Date and time

Load weight

Crane used

Operator name

Condition notes

### Regulatory References

**SOLAS II-1**: Cargo handling equipment requirements

**ILO MLC 2006 (Maritime Labour Convention)**: Safe use of lifting equipment

**LOLER (Lifting Operations and Lifting Equipment Regulations)**: Thorough examination requirements every 12 months

**DNV Crane Rules**: Design and inspection standards for offshore/maritime cranes

**IACS UR M56**: Crane and lifting appliance surveys

**ICS Safety in Docks Code**: Provisions for shore-based operations

### Alarm Summary

| Alarm Tag | Description | Level |

|-----------|-------------|-------|

| CR-001 | Hoist Upper Limit — ATB Warning | A |

| CR-002 | Overload — Load > SWL at Radius | A |

| CR-003-H | Wind Speed Limit Exceeded | A |

| CR-004 | Slew Limit Reached | W |

| CR-005 | Luff Upper Limit | W |

| CR-006 | Luff Lower Limit (Min Angle) | W |

| CR-007 | Emergency Stop Active | A |

| CR-008 | Hydraulic Pressure Low | A |

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System