BlueSim Training Guide

Cyber Security Awareness Trainer

Bridge & Navigation

Maritime cyber security awareness training covering GPS spoofing, phishing, USB malware, ransomware, and AIS spoofing with response procedures.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Maritime cyber security awareness training covering GPS spoofing, phishing, USB malware, ransomware, and AIS spoofing with response procedures.

## Key Features

Interactive ship system map with IT/OT architecture visualization.

Six threat scenarios with animated attack vectors and response checklists.

Cyber risk matrix and vulnerability assessment dashboard.

Incident reporting procedure guidance per IMO MSC-FAL.1/Circ.3.

System status panel showing real-time vulnerability indicators.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| THREAT SCENARIOS | Displays real-time system data, controls, and status indicators. |
| SHIP IT/OT SYSTEM MAP | Displays real-time system data, controls, and status indicators. |
| THREAT DETAIL & RESPONSE | Displays real-time system data, controls, and status indicators. |
| INCIDENT LOG | Displays real-time system data, controls, and status indicators. |
| VULNERABILITY CHECKLIST'; rightPanel.insertBefore(hdr,right | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## Maritime Cyber Security Awareness Trainer — Technical Reference

### Overview

Maritime cyber security encompasses the protection of shipboard IT (Information Technology) and OT (Operational Technology) systems from unauthorized access, data corruption, and service disruption. This trainer covers threat recognition, incident response, and regulatory compliance.

### Regulatory Framework

**IMO MSC-FAL.1/Circ.3** — Guidelines on maritime cyber risk management

**IMO MSC-FAL.1/Circ.3 Rev.2 (2022)** — Revised guidelines

**BIMCO Cyber Security Guidelines** — Industry best practice (v4, 2021)

**IEC 62443** — Industrial automation and control systems security

**STCW Cyber Awareness** — As incorporated per Manila Amendments review

**NIST Cybersecurity Framework** — Identify, Protect, Detect, Respond, Recover

**IMO Resolution MSC.428(98)** — Maritime cyber risk management in SMS

### Ship IT/OT System Architecture

#### Bridge Systems (OT)

ECDIS (Electronic Chart Display and Information System)

AIS (Automatic Identification System)

GNSS (Global Navigation Satellite System)

ARPA/Radar

VDR (Voyage Data Recorder)

Autopilot / Track Control System

#### Engine Room (OT/ICS)

SCADA (Supervisory Control and Data Acquisition)

PMS (Power Management System)

IAS (Integrated Automation System)

Remote monitoring and control systems

#### Cargo/Deck (OT)

Loading computer

Ballast water management system

Cargo loading/discharging controls

#### Communications (IT)

Inmarsat Fleet Xpress / VSAT

Email / Internet

Shore-Ship data transfer

Remote access portals

### Threat Taxonomy

| Threat | Vector | Impact Level |

|--------|--------|-------------|

| USB/Removable Media Malware | Physical | High |

| Phishing Email | Network/Human | Medium-High |

| GPS Spoofing | RF Signal | Critical |

| AIS Spoofing | RF Signal | High |

| Ransomware | Network | Critical |

| Rogue Software Update | Trusted supply chain | Critical |

| Remote Access Exploitation | Network | High |

| Social Engineering | Human | Medium |

### Incident Response Procedure

1. Identify — Detect the anomaly; determine affected systems

2. Contain — Isolate affected systems from network; preserve evidence

3. Eradicate — Remove malware; restore clean backups

4. Recover — Restore normal operations; validate system integrity

5. Report — Notify company DPA, flag state, port state if required

6. Lessons Learned — Update risk assessment and procedures

### GPS Spoofing Indicators

Sudden unexplained position jump

Heading/COG discrepancy with gyrocompass

Multiple GPS receivers showing different positions

GNSS signal quality degradation (SNR drop)

Celestial fixes disagree with GPS

### Verification Methods

Cross-check GNSS with radar/ECDIS coastal fix

Compare with second GNSS receiver

Celestial navigation (sun/star sight)

Dead reckoning from last known position

Compare AIS own-ship position with shore-based AIS

### USB/Removable Media Policy

No personal USB drives on bridge or engine room computers

All removable media scanned on designated air-gapped scan station

Authorised media only (white-listed)

Software updates via company-approved process only

### Cyber Risk Matrix

| | Negligible | Minor | Moderate | Major | Critical |

|--|--|--|--|--|--|

| Almost Certain | Medium | High | High | Extreme | Extreme |

| Likely | Low | Medium | High | High | Extreme |

| Possible | Low | Low | Medium | High | High |

| Unlikely | Negligible | Low | Medium | Medium | High |

| Rare | Negligible | Negligible | Low | Low | Medium |

### Reporting Requirements

Company SMS DPA (Designated Person Ashore): Immediate notification

Flag State Administration: If safety-critical systems affected

Port State Control: Port entry report if incident during voyage

Classification Society: If ICS/automation systems compromised

BIMCO MARITIME CYBER INCIDENT REPORT form

### References

IMO MSC-FAL.1/Circ.3: Guidelines on Maritime Cyber Risk Management

BIMCO Guidelines on Cyber Security Onboard Ships v4.0

IEC 62443-2-1: Security Management System for IACS

ENISA: Guidelines for Cyber Security on Ships (2019)

NIST SP 800-82: Guide to Industrial Control System Security

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System