BlueSim Training Guide

EGR (Exhaust Gas Recirculation) Simulator

Engine Room Equipment

MAN B&W EGR system for Tier III NOx compliance per MARPOL Annex VI Reg.13 and NOx Technical Code 2008.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

MAN B&W EGR system for Tier III NOx compliance per MARPOL Annex VI Reg.13 and NOx Technical Code 2008.

## Key Features

MAN B&W EGR system with variable EGR ratio (0–30%) and blower speed control.

Live NOx output monitoring with Tier II (14.4 g/kWh) and Tier III (3.4 g/kWh) compliance thresholds.

NOx vs engine load trend graph showing emission profile across load range.

ECA entry Tier III activation sequence with blower warm-up and valve sequencing.

Scenarios: ECA Entry Tier III Activation, EGR Blower Fault, NOx Limit Exceedance.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| EGR CONTROLS | Displays real-time system data, controls, and status indicators. |
| P&amp;ID &amp; NOx TREND — MAN B&amp;W EGR | Displays real-time system data, controls, and status indicators. |
| GAUGES &amp; ALARMS | Displays real-time system data, controls, and status indicators. |
| ACTIVE ALARMS | Displays real-time system data, controls, and status indicators. |
| PIPELINE LINEUP | Displays real-time system data, controls, and status indicators. |
| OPERATION LOG | Displays real-time system data, controls, and status indicators. |
| NOx COMPLIANCE | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.2 Valve Controls

Click a valve button to toggle its state. Valve states are colour-coded:

Green: OPEN

Red: CLOSED

Amber: TRANSITIONING

Valve Name

EGR GAS [C]

BYPASS [C]

SCRUB W IN [C]

EGR COOL W [C]

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## Exhaust Gas Recirculation (EGR) — Technical Reference

### Regulatory Framework

#### MARPOL Annex VI — Prevention of Air Pollution from Ships

#### Regulation 13 — Nitrogen Oxides (NOx)

| Tier | Engine Speed (n) | NOx Limit |

|------|-----------------|-----------|

| Tier I | n < 130 rpm | 17.0 g/kWh |

| Tier I | 130 ≤ n < 2000 rpm | 45·n⁻⁰·²  g/kWh |

| Tier I | n ≥ 2000 rpm | 9.8 g/kWh |

| Tier II | n < 130 rpm | 14.4 g/kWh |

| Tier II | 130 ≤ n < 2000 rpm | 44·n⁻⁰·²³ g/kWh |

| Tier II | n ≥ 2000 rpm | 7.7 g/kWh |

| Tier III | n < 130 rpm | **3.4 g/kWh** |

| Tier III | 130 ≤ n < 2000 rpm | 9·n⁻⁰·² g/kWh |

| Tier III | n ≥ 2000 rpm | 2.0 g/kWh |

**Tier III**: Applies in NOx Tier III ECAs (North American ECA, US Caribbean ECA from 2016; Baltic Sea and North Sea from 2021)

Tier III limit for n < 130 rpm (slow-speed engines): **3.4 g/kWh**

Applies to engines with output ≥130 kW installed on ships constructed on or after 1 Jan 2016

#### NOx Technical Code 2008 (NTC 2008)

Defines test cycles (E2/E3 for propulsion engines), survey and certification requirements

Engine NOx Curve File (EIAPP Certificate) required for all Tier II/III engines

EGR system must be certified as part of the approved engine design

---

### MAN B&W EGR System

#### Operating Principle

Exhaust Gas Recirculation (EGR) reduces NOx formation by:

1. Recirculating a portion of exhaust gas back to the scavenge air receiver

2. Diluting the charge air with inert gases (CO₂, H₂O)

3. Reducing peak combustion temperature (NOx formation requires high temperature: >1600°C)

4. Typical NOx reduction: 60–80% (achieving Tier III compliance)

#### EGR Ratio

Definition: EGR% = (mass of recirculated gas) / (total charge air mass) × 100

Typical Tier III EGR ratio: 15–30%

Higher EGR ratio → lower NOx, but higher soot, lower efficiency

MAN B&W G90ME-C10.5 typical: 22% EGR ratio for Tier III

#### System Components

#### EGR Blower

Centrifugal blower

Pressurises recirculated exhaust to scavenge air receiver pressure

Speed: 2000–6000 RPM

Motor: Variable frequency drive (VFD)

#### EGR Scrubber / Cooler

Cools recirculated gas from ~300°C to ~50°C

Removes particulates (soot, sulphur compounds) from EGR stream

Scrubber water: freshwater, pH controlled (≥5.5)

Scrubber water drainage to EGR sludge tank

#### EGR Cooler

Reduces EGR gas temperature before blower

Cooling medium: HT fresh water

Inlet temp: 250–350°C → Outlet: 60–100°C

#### EGR Control Valve

Butterfly valve controlling recirculation ratio

Actuated by main engine control system (MPC)

Position: 0% (fully closed, Tier II) to ~40% (max EGR, Tier III)

#### Charge Air System Interaction

EGR gas mixed with scavenge air in mixing box before cylinders

Turbocharger efficiency must account for EGR

Charge air pressure: 3.5–4.5 bar (engine at full load)

---

### Performance Parameters

| Parameter | Tier II Mode | Tier III Mode |

|-----------|-------------|--------------|

| NOx output | ~14 g/kWh | <3.4 g/kWh |

| EGR ratio | 0% | 15–30% |

| EGR blower speed | Off | 2000–5500 RPM |

| Scrubber water pH | N/A | >5.5 |

| Charge air press | 4.2 bar | 3.8–4.2 bar |

| Engine efficiency | 100% | ~98.5% (slight penalty) |

---

### Alarm Setpoints

| ID | Description | Setpoint | Type |

|----|-------------|----------|------|

| EGR-001 | NOx exceeds Tier III limit | >3.4 g/kWh | Alarm |

| EGR-002 | EGR blower fault / trip | Stopped unexpectedly | Alarm |

| EGR-003 | Scrubber water low pH | <5.5 | Alarm |

| EGR-004 | EGR high temperature | >350°C (inlet) | Alarm |

| EGR-005 | EGR blower high vibration | — | Warning |

| EGR-006 | EGR cooling water low flow | — | Warning |

| EGR-007 | ECA entry — Tier III required | ECA position | Warning |

| EGR-008 | NOx exceeds Tier II limit | >14.4 g/kWh | Alarm |

---

### Tier II vs Tier III Operation

#### Tier II (Open Ocean / Non-ECA)

EGR system OFF or in standby

EGR control valve: fully closed

Engine burns HFO/MGO without NOx reduction

NOx ~14 g/kWh (compliant with Tier II limit of 14.4 g/kWh)

#### Tier III (NOx ECA)

EGR system activated 30–60 minutes before ECA entry

EGR control valve: opens to 15–25% position

EGR blower starts (Tier III pre-activation)

NOx reduced to <3.4 g/kWh

Slight fuel consumption increase (~1–2%)

---

### ECA Entry Procedure (Tier III Activation)

1. Receive ECA approach notification.

2. Start EGR scrubber water pump.

3. Start EGR blower (warm up period: ~5 minutes).

4. Gradually open EGR gas inlet valve.

5. Set EGR ratio setpoint (typically 20%).

6. Monitor NOx output until below 3.4 g/kWh.

7. Confirm system stable before ECA boundary crossing.

8. Log Tier III activation in engine log.

---

### References

MARPOL 73/78 Annex VI, Regulation 13 — NOx

NOx Technical Code 2008 (NTC 2008)

MAN B&W EGR System — Technology Review

IACS UR M74 — NOx Monitoring

IMO MEPC.176(58) — Amendments to MARPOL Annex VI

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System