BlueSim Training Guide

Ship Electrical Power Simulator

Engine & Electrical

Main and emergency switchboard — SOLAS Ch.II-1 Reg.41-45 and IEC 60092. Blackout recovery, generator changeover, earth fault finding, and preferential trip response.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Main and emergency switchboard — SOLAS Ch.II-1 Reg.41-45 and IEC 60092. Blackout recovery, generator changeover, earth fault finding, and preferential trip response.

## Key Features

Real-time SLD with live bus voltage and load display.

6 scenarios: blackout, DG trip, earth fault, overload, shore power.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.3 Alarm Definitions

The alarm management system monitors the following conditions:

| Alarm ID | Severity | Description |
| --- | --- | --- |
| BLACKOUT | F | BLACKOUT — Main power lost — start E/G immediately |
| DG1_TRIP | F | DG-1 tripped — overload / fault — start DG-2 |
| DG2_TRIP | F | DG-2 tripped on fault |
| OVERLOAD_A | WARNING | Bus A overloaded — shed non-essential loads |
| OVERLOAD_B | WARNING | Bus B overloaded — shed non-essential loads |
| EARTH_FAULT | E | Earth fault detected — insulation < 1 MΩ — locate immediately |
| LOW_INS | E | Low insulation resistance — check affected feeder |
| PREF_TRIP | WARNING | Preferential trip operated — non-essential loads shed |
| FREQ_LOW | WARNING | Bus frequency low — check generator governor |
| VOLT_LOW | WARNING | Bus voltage low — check AVR / excitation |
| SHORE_LIVE | I | Shore power available — ready for changeover |
| EG_RUNNING | I | Emergency generator running — restore main power |
| DG2_ONLINE | I | DG-2 online and on load |
| SYNC_OK | I | Synchronisation confirmed — safe to parallel |

# 5. Training Scenarios

Select a scenario from the modal dialog when the simulator loads. Each scenario has specific learning objectives and a checklist of actions to complete.

## 5.1 NORMAL OPERATIONS

DG-1 online supplying Bus A. DG-2 on standby. All essential loads energised. Monitor load balance and insulation resistance.

### Actions Required

Step 1: Verify DG-1 voltage 440 V ± 2%

Step 2: Confirm Bus A load < 80% rated

Step 3: Check insulation resistance > 1 MΩ

Step 4: Log generator running hours

Step 5: Test emergency generator start (weekly)

## 5.2 BLACKOUT RECOVERY

Total power failure. Emergency generator must start automatically within 30 s (SOLAS Ch.II-1 Reg.42). Restore MSB once E/G is stable.

### Actions Required

Step 1: Confirm blackout — all buses dead

Step 2: Emergency generator auto-started

Step 3: Emerg Bus energised — nav lights on

Step 4: Start DG-1 or DG-2 manually

Step 5: Synchronise DG to Bus A

Step 6: Close ACB — Bus A live

Step 7: Restore essential loads in priority order

Step 8: Shed emergency generator — transfer to main

## 5.3 DG-1 TRIP — GENERATOR FAULT

DG-1 has tripped on overcurrent. Bus A is dead. Start DG-2, close Bus Tie Breaker or close ACB DG-2 to Bus A, restore loads.

### Actions Required

Step 1: Acknowledge DG-1 trip alarm

Step 2: Identify trip cause — check fault annunciator

Step 3: Start DG-2

Step 4: Wait for DG-2 to reach rated voltage / freq

Step 5: Close ACB DG-2 to Bus A (via BTB or direct)

Step 6: Verify Bus A restored — check voltage

Step 7: Restore loads in priority order

Step 8: Do NOT restart DG-1 without investigation

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System