BlueSim Training Guide

Emergency Shutdown (ESD) System

Safety & Emergency

Ship-wide ESD panel for tankers and gas carriers. Three-level shutdown logic covering cargo manifold, pump ESD valves, engine room isolation, and HVAC shutdown per SOLAS II-2, IGC Code Ch.18, and IEC 61508 SIL 2.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Ship-wide ESD panel for tankers and gas carriers. Three-level shutdown logic covering cargo manifold, pump ESD valves, engine room isolation, and HVAC shutdown per SOLAS II-2, IGC Code Ch.18, and IEC 61508 SIL 2.

## Key Features

Three ESD levels: cargo ops stop, emergency stop with full valve closure, full ship emergency.

8 cargo manifold valves (4 port, 4 starboard) with real-time position feedback and closure animation.

Automatic ESD triggers: gas detection above 40% LEL and tank overpressure simulation.

Zone status LEDs (green normal / red tripped) for Cargo Manifold, Pumps, Engine Room, and HVAC.

ESD trip log with timestamps, cause identification, and reset sequence requiring key confirmation.

Shore ESD link indicator with hardwired and fibre optic backup status.

Scenarios: Cargo Manifold ESD Trip, Automatic Gas Detection Trip, ESD Reset Procedure.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| ESD CONTROLS | Displays real-time system data, controls, and status indicators. |
| ESD SYSTEM SCHEMATIC — CARGO AREA | Displays real-time system data, controls, and status indicators. |
| ZONE STATUS | Displays real-time system data, controls, and status indicators. |
| ESD TRIP LOG | Displays real-time system data, controls, and status indicators. |
| MANIFOLD VALVE MATRIX | Displays real-time system data, controls, and status indicators. |
| SYSTEM PARAMETERS | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## Emergency Shutdown (ESD) System — Technical Reference

### Overview

The Emergency Shutdown (ESD) system is a safety-critical system installed on tankers and gas carriers to rapidly stop cargo operations and isolate hazardous energy sources in the event of an emergency. It operates at multiple protection levels (SIL 2 rated per IEC 61508) and includes both manual and automatic trip functions.

### Regulatory Basis

**SOLAS II-2** — Fire protection, detection and extinction

**IGC Code Chapter 18** — Emergency shutdown systems for gas carriers

**IEC 61508** — Functional safety of electrical/electronic/programmable safety systems (SIL 2)

**OCIMF MEG4** — Mooring equipment guidelines (shore link requirements)

**ISO 16852** — Flame arrester standards referenced for valve closure times

### ESD Levels

| Level | Name | Actions |

|-------|------|---------|

| Level 1 | Cargo Ops Stop | Stop cargo pumps, close cargo manifold valves |

| Level 2 | Emergency Stop | All Level 1 + close all cargo valves, stop compressors, isolate IG |

| Level 3 | Full Ship Emergency | All Level 2 + ER ESD, HVAC shutdown, shore link open |

### Zone Layout

#### Cargo Manifold (Port & Starboard)

4 valves per side (CMOV-P01 to P04, CMOV-S01 to S04)

Pneumatic actuators, fail-closed

Closure time: ≤30 seconds

Position feedback: open/closed limit switches

#### Cargo Pumps

3 cargo pumps (CP-01, CP-02, CP-03)

1 stripping pump (SP-01)

ESD valve on discharge of each pump

Motor trip via ESD relay

#### Engine Room ESD

Main engine fuel oil shut-off

Generator fuel oil shut-off

Emergency stop button (independent circuit)

#### HVAC Shutdown

Accommodation HVAC supply fans

Cargo control room HVAC

Engine room ventilation fans

All recirculation systems

### Automatic ESD Triggers

| Trigger | Level | Setpoint |

|---------|-------|---------|

| Gas detection — cargo area | Level 2 | >40% LEL |

| Gas detection — accommodation HVAC | Level 1 | >20% LEL |

| Cargo tank overpressure | Level 2 | >0.25 bar above MARVS |

| Cargo loading arm failure | Level 1 | ERS activation |

| Shore ESD signal | Level 2 | Discrete input from terminal |

| High-high liquid level cargo tank | Level 1 | 98% tank volume |

| Fire detection — cargo area | Level 2 | Confirmed detector |

### ESD Trip Log

All ESD activations are logged with:

Date/time of trip

Zone/cause

ESD level activated

Reset time

Operator ID (if manual)

### Reset Procedure

ESD reset requires:

1. Key switch to RESET position (key #ESD-RESET-01)

2. Identify and clear cause of trip

3. Confirm safe conditions

4. Press RESET button (hold 3 seconds)

5. Check all valves have returned to required position

6. Obtain authorisation from Master/OOW before resuming cargo ops

### Communication Links

Shore ESD link: hardwired (primary) + fibre optic (backup)

Internal ESD stations: Bridge, CCR, Cargo manifold (P&S)

Battery backup: 24 VDC UPS, 30-minute capacity

### SIL 2 Requirements

Probability of Failure on Demand (PFD): 10⁻² to 10⁻³

Proof test interval: 12 months

Diagnostic coverage: >90%

Hardware fault tolerance: 1 (redundant channels)

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System