BlueSim Training Guide

Fire Detection & Alarm Panel

Safety & Emergency

Addressable fire detection and alarm system with 12 zones covering all ship spaces. Includes optical smoke, heat (ROR/fixed), and flame detectors with automatic fire-fighting actions per SOLAS II-2/Reg.7, FSS Code Ch.9, and IMO MSC.1/Circ.1477.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Addressable fire detection and alarm system with 12 zones covering all ship spaces. Includes optical smoke, heat (ROR/fixed), and flame detectors with automatic fire-fighting actions per SOLAS II-2/Reg.7, FSS Code Ch.9, and IMO MSC.1/Circ.1477.

## Key Features

12-zone addressable detection system: Bridge, Accommodation (A-D), Engine Room, CCR, Pump Room, Galley, Store.

Four detector types: optical smoke, heat (rate-of-rise + fixed 70°C), flame (UV/IR), and manual call points.

SVG ship cross-section zone map with colour-coded status (normal/alarm/fault/disabled).

Automatic actions on alarm: fire damper closure, emergency pump start, ventilation stop, general alarm.

Zone disable/enable, ACK, RESET, and EVACUATE signal controls.

Dual power supply monitoring: mains 230 VAC and 24 VDC battery backup with state of charge.

Scenarios: Accommodation Fire (Deck B), Engine Room Heat Alarm, False Alarm, Detector Fault.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| PANEL CONTROLS | Displays real-time system data, controls, and status indicators. |
| SHIP ZONE MAP — FIRE DETECTION | Displays real-time system data, controls, and status indicators. |
| FIRE ALARMS | Displays real-time system data, controls, and status indicators. |
| DETECTOR DETAIL | Displays real-time system data, controls, and status indicators. |
| EVENT LOG | Displays real-time system data, controls, and status indicators. |
| SYSTEM PARAMETERS | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## Fire Detection & Alarm System — Technical Reference

### Overview

An addressable fire detection and alarm system provides continuous monitoring of all spaces aboard a vessel. Each detector has a unique address, allowing precise identification of the alarm location. The system automatically activates fire-fighting actions and general alarm upon confirmed detection.

### Regulatory Basis

**SOLAS II-2/Regulation 7** — Detection and alarm

**FSS Code Chapter 9** — Automatic sprinkler, fire detection, and fire alarm systems

**IMO MSC.1/Circ.1477** — Guidelines for approval of fixed fire detection systems

**ISO 7240** — Fire detection and alarm systems

**IEC 60092-504** — Electrical installations in ships — special features

### System Zones (12 Total)

| Zone | ID | Space | Detector Types |

|------|----|-------|----------------|

| 01 | Bridge | Navigation Bridge | Optical smoke, heat (ROR) |

| 02 | Acc-A | Accommodation Deck A | Optical smoke, MCP |

| 03 | Acc-B | Accommodation Deck B | Optical smoke, MCP |

| 04 | Acc-C | Accommodation Deck C | Optical smoke, heat |

| 05 | Acc-D | Accommodation Deck D | Optical smoke, MCP |

| 06 | Eng.Room | Engine Room | Heat (fixed 70°C + ROR), flame (UV/IR) |

| 07 | CCR | Cargo Control Room | Optical smoke, heat |

| 08 | Pump Rm | Pump Room | Heat (fixed 70°C), flame detector |

| 09 | Galley | Galley & Pantry | Heat (ROR), thermal imaging |

| 10 | Store | Paint/Store Rooms | Optical smoke |

| 11 | CO2 Rm | CO2 Room | Optical smoke |

| 12 | Void | Void Spaces/Cofferdams | Optical smoke |

### Detector Types

#### Optical Smoke Detector

Operating principle: Light scattering (forward scatter)

Sensitivity: 0.1 to 2.0 dB/m

Alarm threshold: Configurable (typical 0.2 dB/m)

Response time: <30 seconds (slow smouldering fires)

#### Heat Detector — Rate of Rise (ROR)

Triggers when temperature rise exceeds 8–10°C/min

Fixed alarm threshold: 70°C

Suitable for engine room environments

#### Flame Detector (UV/IR)

Ultraviolet and infrared combined

Response time: <30ms to open flame

Detection range: Up to 20m

Immune to solar radiation (IR channel)

#### Manual Call Point (MCP)

Glass-break or push-button activation

Requires intentional physical action

Located at exit routes, max 30m travel distance

### Automatic Actions on Alarm

| Action | Trigger |

|--------|---------|

| Sound general alarm (GPA) | Any confirmed fire alarm |

| Close fire dampers (zone) | Alarm in ventilated zone |

| Start emergency fire pump | Confirmed alarm in ER or Pump Room |

| Stop supply air fans | Alarm in accommodation |

| Notify bridge | All alarms |

| Start sprinkler (if fitted) | Accommodation alarms |

### Panel Controls

**ACKNOWLEDGE** — Silence audible alarm, maintain visual

**RESET** — Clear alarm state (after fire extinguished)

**EVACUATE** — Sound continuous evacuation signal

**DISABLE zone** — Isolate zone during maintenance (max 24h)

**ENABLE zone** — Restore disabled zone

**ALARM TEST** — Test alarm outputs only (not detectors)

**LAMP TEST** — Test all panel LEDs

### Power Supply

| Supply | Specification |

|--------|--------------|

| Mains | 230 VAC, 50 Hz |

| Battery backup | 24 VDC, sealed lead-acid |

| Backup duration | 72 hours standby + 30 min alarm |

| Charger | Trickle/boost automatic |

### Zone Status Definitions

**NORMAL** — No alarm, no fault, zone enabled

**ALARM** — Fire detected — red indicator

**FAULT** — Open circuit / short circuit / detector removed

**DISABLED** — Zone manually isolated by operator

**TEST** — Zone in test mode

### Fault Indicators

Detector removed from base

Open wiring loop

Short circuit on detection loop

Low battery voltage (<23.5 VDC)

Mains power failure

Printer fault

Panel processor fault (watchdog)

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System