BlueSim Training Guide

Fixed Gas Detection System

Safety & Emergency

Fixed multi-gas detection system for gas carriers and tankers. Monitors HC (%LEL), H2S, O2, CO, and NH3 across 6 zones with alarm setpoints, automatic ESD/ventilation actions, and sensor fault diagnostics per IGC Code Ch.13, SOLAS II-2, and IEC 60092-504.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Fixed multi-gas detection system for gas carriers and tankers. Monitors HC (%LEL), H2S, O2, CO, and NH3 across 6 zones with alarm setpoints, automatic ESD/ventilation actions, and sensor fault diagnostics per IGC Code Ch.13, SOLAS II-2, and IEC 60092-504.

## Key Features

Six monitoring zones: Cargo Hold, Pump Room, Compressor Room, Cofferdam, Void Space, HVAC Intake.

Five gas types: HC (%LEL), H2S (ppm), O2 (%v/v), CO (ppm), NH3 (ppm) with warning and alarm setpoints.

Automatic actions on alarm: ventilation maximum, HVAC shutdown, ESD signal.

Sensor status matrix: normal, warning, alarm, fault, and inhibit states per zone.

Calibration due date tracking and bump test simulation for each sensor.

Historical reading log strip with trend indication per zone.

Scenarios: Cargo Hold Gas Alarm, Pump Room H2S Detection, Sensor Calibration Check, O2 Deficiency Alert.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| ZONE STATUS MATRIX | Displays real-time system data, controls, and status indicators. |
| GAS DETECTION PANEL — SHIP LAYOUT | Displays real-time system data, controls, and status indicators. |
| GAS ALARMS | Displays real-time system data, controls, and status indicators. |
| CALIBRATION STATUS | Displays real-time system data, controls, and status indicators. |
| GAS EVENT LOG | Displays real-time system data, controls, and status indicators. |
| SYSTEM SUMMARY | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## Fixed Gas Detection System — Technical Reference

### Overview

Fixed multi-gas detection systems are mandatory on gas carriers and tankers to continuously monitor for hazardous gas concentrations in enclosed and semi-enclosed spaces. The system provides warning and alarm outputs that can trigger automatic ventilation, ESD, and HVAC shutdown.

### Regulatory Basis

**IGC Code Chapter 13** — Instrumentation (gas detection)

**SOLAS II-2** — Harmful vapours and fire hazards

**IMO MSC.1/Circ.1370** — Recommendations for entering enclosed spaces

**IEC 60092-504** — Electrical installations in ships — process measurement and control

**IEC 61779** — Electrical apparatus for detection of flammable gases

### Gas Types Monitored

| Gas | Symbol | Unit | Alarm Setpoints | Notes |

|-----|--------|------|-----------------|-------|

| Hydrocarbons (LEL) | HC | % LEL | W: 20% / A: 40% | LPG, LNG, cargo vapours |

| Hydrogen Sulphide | H2S | ppm | W: 5 ppm / A: 10 ppm | Crude oil carriers, H2S crude |

| Oxygen | O2 | % v/v | Low W: 19.5% / A: 18% | High W: 23.5% / A: 25% |

| Carbon Monoxide | CO | ppm | W: 25 ppm / A: 50 ppm | Engine exhausts, fire indicator |

| Ammonia (if fitted) | NH3 | ppm | W: 25 ppm / A: 50 ppm | Refrigeration systems |

### Monitored Zones (6)

| Zone | ID | Location | Gases Monitored |

|------|----|----------|-----------------|

| Cargo Hold | CH | Between tanks, void spaces | HC, O2 |

| Pump Room | PR | Cargo pump room | HC, H2S, O2 |

| Compressor Room | CR | Cargo compressor space | HC, NH3, O2 |

| Cofferdam | CF | Fore/aft cofferdams | HC, H2S, O2 |

| Void Space | VS | Double bottom/wing voids | HC, O2 |

| HVAC Intake | HV | Accommodation HVAC inlet | HC, H2S, CO |

### Alarm Levels

#### HC (Hydrocarbons)

**Warning (20% LEL):** Increase ventilation, monitor continuously

**Alarm (40% LEL):** Stop all hot work, evacuate space, trigger ESD

**Note:** 100% LEL = Lower Explosive Limit (ignition possible above this)

#### H2S (Hydrogen Sulphide)

**Warning (5 ppm):** Don SCBA, increase ventilation

**Alarm (10 ppm):** Evacuate immediately (IDLH = 10 ppm immediate danger)

**Note:** H2S causes rapid olfactory fatigue — do not rely on smell

#### O2 (Oxygen)

**Oxygen Deficiency Warning:** < 19.5% v/v

**Oxygen Deficiency Alarm:** < 18.0% v/v (cannot enter without SCBA)

**Oxygen Enrichment Warning:** > 23.5% v/v (enhanced fire risk)

**Oxygen Enrichment Alarm:** > 25.0% v/v

#### CO (Carbon Monoxide)

**Warning:** 25 ppm (8-hour TWA = 25 ppm)

**Alarm:** 50 ppm (short-term exposure limit 100 ppm)

### Automatic Actions

| Condition | Action |

|-----------|--------|

| HC Alarm (40% LEL) | ESD signal, ventilation max, HVAC shutdown |

| H2S Alarm (10 ppm) | Sound alarm, HVAC shutdown, evacuate |

| O2 < 18% | Entry prohibition, SCBA requirement |

| HC at HVAC intake >20% LEL | HVAC shutdown, ESD Level 1 |

### Sensor Technology

#### Catalytic Bead (Pellistor) — HC Detection

Principle: Catalytic combustion heats bead, measures resistance change

Range: 0–100% LEL

Requires O2 to function correctly

Poison risk: silicone, lead, halides

#### Electrochemical Cell — H2S, CO, O2

Principle: Electrochemical oxidation/reduction

Cross-sensitivity: O2 cell affected by high CO2

Service life: 1–3 years typical

#### Infrared (NDIR) — HC, CO2

Principle: Infrared absorption at characteristic wavelengths

Advantage: No O2 dependency, longer life

Calibration gas: Methane for HC

### Calibration Procedure

1. Expose sensor to zero gas (clean dry air) — record zero reading

2. Apply certified calibration gas at known concentration

3. Adjust span if reading differs by >±5%

4. Record in calibration log

5. Bump test before each enclosed space entry

### Sensor Status Definitions

**NORMAL** — Within range, no alarm

**ALARM** — Reading above alarm setpoint

**WARNING** — Reading above warning setpoint

**FAULT** — Sensor failure, open circuit, or zero failure

**INHIBIT** — Manually isolated (maintenance)

**CALIBRATION DUE** — Service interval exceeded

### Inerting Requirements (Gas Carriers)

Cargo tanks must be inerted before gas-freeing or when HC >2% v/v in void spaces adjacent to tanks. Inert gas O2 content must be <5% v/v.

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System