BlueSim Training Guide

Fresh Water Generator

Engine Room Equipment

Alfa Laval JWP-26-C100 plate-type evaporator simulator using engine JCW waste heat with vacuum evaporation, salinometer, and 12nm coastal restriction per SOLAS II-1.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Alfa Laval JWP-26-C100 plate-type evaporator simulator using engine JCW waste heat with vacuum evaporation, salinometer, and 12nm coastal restriction per SOLAS II-1.

## Key Features

Vacuum evaporation simulation with ejector pump, JCW heat source, and condenser.

Live gauges: JCW inlet/outlet temperature, vacuum (mbar), evaporation temperature, salinity, production rate.

Salinometer with automatic 3-way divert valve when salinity exceeds 2 ppm.

12nm coastal restriction interlock — FWG auto-stops when within 12nm of land.

Alarms: high salinity divert, low vacuum, JCW temp low trip (<60°C).

Three scenarios: Normal Production, High Salinity Response, Low Vacuum Fault.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| CONTROLS | Displays real-time system data, controls, and status indicators. |
| FWG FLOW DIAGRAM | Displays real-time system data, controls, and status indicators. |
| GAUGES & ALARMS | Displays real-time system data, controls, and status indicators. |
| VALVE LINEUP | Displays real-time system data, controls, and status indicators. |
| VACUUM & SALINITY TREND | Displays real-time system data, controls, and status indicators. |
| PRODUCTION LOG | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## Fresh Water Generator (FWG) — Technical Reference

### Regulatory Framework

#### SOLAS II-1 — Fresh Water for Boilers and Systems

SOLAS requires ships to have adequate fresh water supply for safe operation.

The FWG produces distillate water from seawater using waste heat from the main engine jacket cooling water (JCW).

FWG production is a critical service for extended voyages.

#### WHO / IMO Health and Sanitation Standards

**Salinity limit for potable water**: <2 ppm (2 mg/L) TDS for distillate water.

**WHO Guidelines for Drinking Water Quality**: Conductivity <2500 μS/cm; pH 6.5–9.5.

IMO recommends treatment (mineralisation, UV sterilisation, chlorination) before potable water use.

Water produced within 12 nautical miles of shore must NOT be used as potable water or for distillation, due to potential biological and chemical contamination.

#### 12-Nautical Mile Rule

Many FWG manufacturers and ship operators implement a **12 nm interlock**:

- FWG must be stopped when within 12 nm of nearest land.

- OR production must be diverted to overboard/bilge if operating within 12 nm.

Based on MARPOL and WHO recommendations regarding coastal water quality.

Some class societies require FWG shutdown alarm at <12 nm.

#### Classification Requirements

FWG capacity must be sufficient for ship's needs (domestic, boiler feed, engine cooling makeup).

Typical requirement: 1–2 t/day per 100 kW main engine power (waste heat basis).

---

### Alfa Laval JWP-26-C100 — Technical Parameters

#### Operating Principle

The plate-type FWG uses vacuum evaporation (also called low-temperature evaporation):

1. Main engine JCW (~85–95°C) enters the evaporator (heat exchanger) and heats seawater.

2. Under vacuum (~90–95% vacuum, ~90–100 mbar absolute), seawater boils at only 40–45°C.

3. Evaporated water (steam) rises and is condensed by cool seawater in the condenser.

4. Distillate (fresh water) is pumped to the FW storage tank.

5. Salinometer continuously monitors distillate quality; if salinity >2 ppm, a three-way valve diverts to overboard/bilge.

6. Brine (concentrated seawater) and non-condensables are removed by the brine ejector pump.

#### FWG Specifications (JWP-26-C100)

| Parameter | Value |

|-----------|-------|

| Production Rate | 26 t/day (at design conditions) |

| JCW Inlet Temperature | 80–88°C (typical) |

| JCW Flow Rate | ~100 m³/h |

| Seawater Inlet Temp (cooling) | 32°C max (tropics) |

| Seawater Flow (cooling) | ~150 m³/h |

| Vacuum | 90–96 mbar absolute |

| Evaporation Temperature | 40–45°C |

| Distillate Salinity | <2 ppm (typical 0.5–1.5 ppm) |

| Distillate Production Temp | ~35–40°C |

| Brine Pump Type | Ejector |

| Overall Dimensions (approx.) | 1.8 × 0.9 × 1.6 m |

#### Performance Reduction Factors

| Condition | Effect on Output |

|-----------|-----------------|

| JCW temp reduced by 5°C | ~15% production loss |

| JCW temp <70°C | Production may be insufficient or stop |

| Sea temperature increase 5°C | ~8% production loss |

| Poor vacuum (>120 mbar) | Production falls significantly |

| High seawater salinity | Minor effect |

| High brine level | Flooding risk, stop unit |

#### Minimum JCW Temperature

**Minimum JCW temperature for operation: 60°C** (class requirement, various).

Below this, risk of insufficient evaporation and contamination of distillate.

DNV GL, LR: FWG should be stopped if JCW inlet temperature drops below 60°C (interlock).

Some operators use 65°C as their operating limit.

---

### Operating Procedure

#### Start-Up Sequence

1. Check position: Confirm vessel is >12 nm from nearest land.

2. Sea cocks: Verify seawater supply to ejector/condenser open.

3. Start ejector pump: Creates vacuum in evaporator unit.

4. Open brine ejector valve: Allows brine removal.

5. Check vacuum: Should reach 90+ mbar within 5–10 minutes.

6. Open JCW inlet: Allow JCW through evaporator.

7. Observe evaporation: Temperature and pressure gauges confirm boiling begins.

8. Start distillate pump: When distillate appears (check sight glass).

9. Salinometer ON: Monitor distillate quality.

10. Open FW tank valve: If salinity <2 ppm, direct to FW tank.

11. Monitor: Log production rate, vacuum, JCW temps, salinity every watch.

#### Shut-Down Sequence

1. Close JCW inlet valve.

2. Allow evaporation to cease (5–10 minutes).

3. Stop distillate pump.

4. Stop ejector pump.

5. Break vacuum slowly (open equalising valve if fitted).

6. Close seawater supply.

7. Log final production to ORB/deck log.

---

### Vacuum System

#### Ejector System (Liquid Ring / Jet Ejector)

Seawater-driven jet ejector maintains vacuum by:

1. High-pressure seawater flow through nozzle creates suction.

2. Draws non-condensable gases (air) from evaporator.

3. Maintains ~90–95 mbar absolute pressure (equivalent to ~600–700 mmHg vacuum gauge).

#### Vacuum Troubleshooting

| Symptom | Likely Cause | Action |

|---------|-------------|--------|

| Poor vacuum (<85 mbar) | Air leak in system | Check all flanges, sight glasses |

| Very poor vacuum | Ejector fault | Check SW pressure to ejector |

| Vacuum OK but no production | JCW too cold | Check JCW temperature |

| High distillate salinity | Vacuum too low, or too high production rate | Reduce throughput, check vacuum |

---

### Salinometer / Conductivity Monitor

#### Principle

Measures electrical conductivity of distillate water.

Conductivity is directly proportional to dissolved salt (NaCl) content.

Reference: **2 ppm NaCl ≈ 4 μS/cm conductivity**.

Actual reading in μS/cm; calibrated to ppm equivalent.

#### Three-Way Valve Action

If conductivity > 2 ppm equivalent:

- Three-way valve CLOSES to FW tank.

- OPENS to overboard/bilge.

- Alarm activated.

When conductivity returns to <2 ppm:

- After a hold-off timer (typically 30–60 seconds to confirm stable reading).

- Valve returns to FW tank position automatically or on manual confirmation.

---

### Salinometer Alarm Settings

| Parameter | Setting |

|-----------|---------|

| High Salinity Alarm | 2 ppm (automatic divert) |

| Salinometer High Warning | 1.5 ppm |

| JCW Temp Low Alarm | <65°C |

| JCW Temp Low Low Trip | <60°C (stop FWG) |

| Low Vacuum Warning | <85 mbar |

| Low Vacuum Alarm | <75 mbar |

| High Brine Level | 80% |

| Under-Production Warning | <50% rated output |

---

### Water Quality and Treatment

#### Before Using FWG Distillate as Potable Water

1. Mineralisation: Add calcium, magnesium (reverse osmosis/distillate lacks minerals).

2. pH adjustment: Target pH 7.0–7.5.

3. UV sterilisation: Kill any microorganisms.

4. Chlorination: 0.2–0.5 ppm residual chlorine maintained in the distribution system.

5. Regular testing: pH, chlorine residual, turbidity, microbiological (monthly).

#### FWG Distillate vs Potable Water Standards

| Parameter | FWG Output | WHO Drinking Standard |

|-----------|-----------|----------------------|

| TDS | <5 ppm | <600 ppm |

| Conductivity | <10 μS/cm | <2500 μS/cm |

| pH | — | 6.5–9.5 |

| Salinity (NaCl) | <2 ppm | <200 ppm |

---

### References

Alfa Laval JWP-26-C100 Technical Manual (TM-JWP26C100)

SOLAS Chapter II-1, Part E — Additional Requirements for Passenger Ships

WHO Guidelines for Drinking-Water Quality (4th ed., 2017)

IMO Circular MSC.1/Circ.1409 — Potable Water Recommendations for Ships

DNV GL Rules for Classification: Ship, Part 4 Chapter 6 — Piping Systems

ILO Maritime Labour Convention (MLC) 2006, Standard A3.2 — Food and Catering (water quality)

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System