BlueSim Training Guide

Fuel Cell Monitoring System

Fuel Simulators

Maritime PEM fuel cell simulator with 2-stack power monitoring, 5-stage cold start, H₂ supply management and ship power bus integration.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Maritime PEM fuel cell simulator with 2-stack power monitoring, 5-stage cold start, H₂ supply management and ship power bus integration.

## Key Features

Two 200 kW PEM stacks with voltage, current, temperature, efficiency and health degradation monitoring.

5-stage startup sequence with animated progress: pre-check, purge, conditioning, ramp-up, full power.

H₂ supply pressure monitoring with stack starvation prevention and gas detection alarms.

Educational panel explaining PEM electrochemistry plus real-time zero-CO₂ environmental metrics.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| FUEL CELL STACKS | Displays real-time system data, controls, and status indicators. |
| SYSTEM SCHEMATIC & POWER BUS | Displays real-time system data, controls, and status indicators. |
| H₂ SUPPLY & MONITORING | Displays real-time system data, controls, and status indicators. |
| SYSTEM SUMMARY | Displays real-time system data, controls, and status indicators. |
| POWER OUTPUT TREND | Displays real-time system data, controls, and status indicators. |
| OPERATIONS LOG | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## Maritime Fuel Cell Monitoring System — Technical Reference

### Regulatory Framework

**DNV-CG-0584**: Classification Guideline for hydrogen and fuel cell installations on ships (2023).

**ISO 23273**: Fuel cell road vehicles — Safety specifications (adapted for maritime).

**IMO MSC-MEPC.2/Circ.17**: Interim guidelines for vessels using fuel cells.

**IGF Code (future H₂ amendments)**: IMO working group developing H₂ fuel gas requirements under IGF Code scope.

**SOLAS II-1/Reg.55**: Systems using low-flashpoint fuels including hydrogen.

**IEC 62282**: Series — Fuel cell technologies, safety, performance, installation.

### Fuel Cell Types in Maritime Applications

#### PEM (Proton Exchange Membrane / Polymer Electrolyte Membrane)

Operating temperature: 60–80°C

Fuel: Pure hydrogen (H₂ purity > 99.97%)

Electrolyte: Nafion polymer membrane

Cold start capable: yes (minutes to full power)

Power density: high (~1 kW/kg)

Best for: low-to-medium power applications, APU, harbour/zero-emission operation

Manufacturers: Ballard, Hydrogenics, Toyota, Intelligent Energy

#### SOFC (Solid Oxide Fuel Cell)

Operating temperature: 700–1,000°C

Fuel: H₂, reformed LNG, methanol, ammonia

Electrolyte: Ceramic oxide (ZrO₂)

Start-up time: 30–120 minutes (thermal ramp required)

Efficiency: up to 60% electrical, 80–90% with heat recovery (CHP)

Best for: base load generation, continuous operation

Manufacturers: Wärtsilä, Bloom Energy, Convion

### Operating Principle — PEM Fuel Cell

#### Electrochemical Reaction

Anode (−): H₂ → 2H⁺ + 2e⁻

Cathode (+): ½O₂ + 2H⁺ + 2e⁻ → H₂O

Overall: H₂ + ½O₂ → H₂O + Electricity + Heat

Open Circuit Voltage (OCV): ~0.9–1.0 V per cell

Operating voltage: 0.5–0.8 V per cell (at rated load)

Stack voltage = N cells × cell voltage (e.g., 400 cells × 0.65V = 260V DC)

Efficiency: 45–65% (electrical, LHV basis)

### System Components

#### Stack Parameters (per stack)

| Parameter | Normal Range | Alarm |

|-----------|-------------|-------|

| Stack voltage | 200–320 V DC | < 180 V (low) |

| Stack current | 0–600 A | > 650 A (over-current) |

| Power output | 0–200 kW | — |

| Cell temperature | 60–80°C (PEM) | > 85°C |

| Coolant temp in | 55–65°C | > 70°C |

| Coolant temp out | 70–80°C | > 85°C |

| H₂ pressure | 1.5–3.0 bar | < 1.0 bar (low) |

| H₂ purity | > 99.97% | < 99.9% |

| Stack health | 100% (new) | < 70% (degraded) |

#### Power Conditioning

1. DC/DC Converter: Steps up/down stack DC voltage to DC bus voltage (typically 600–800 V DC)

2. Inverter (DC/AC): Converts DC bus to 440/690V AC 50/60 Hz

3. Power factor correction: Maintains PF > 0.95

4. Ship power bus integration: Parallel operation with diesel generators or other sources

### Start-Up Sequence (5 Stages)

1. Pre-check: Verify H₂ supply pressure, coolant level, no active faults

2. Purge: Purge anode side with H₂ to remove air/nitrogen contamination

3. Conditioning: Ramp up coolant flow, begin hydrogen supply

4. Ramp-up: Gradually increase load (dV/dt controlled to prevent flooding)

5. Full power: Stack at rated output, connected to power bus

### Degradation

PEM stack health degrades over time due to: membrane thinning, platinum catalyst loss, flooding, drying

Typical service life: 20,000–40,000 hours

Health monitoring: voltage per cell (mV), pressure drop across stack

Replacement interval: based on power curve or cell voltage standard deviation

### Safety — Hydrogen

| Parameter | Value |

|-----------|-------|

| LEL (H₂ in air) | 4% vol |

| UEL | 75% vol |

| Ignition energy | 0.017 mJ (very low!) |

| Auto-ignition temp | 500–571°C |

| Density | 0.09 kg/m³ (rises — ventilate top of space) |

| TLV | Simple asphyxiant; no toxicity |

| Flame | Invisible in daylight, UV detectable |

### Environmental Benefits

Zero direct CO₂ emissions at point of use

Water is the only exhaust product (H₂O vapour)

Zero SOx, zero PM (particulate matter)

NOx: < 0.01 g/kWh (orders of magnitude below IMO Tier III diesel)

Water production: ~0.9 litres per kWh produced (recoverable)

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System