BlueSim Training Guide

Battery / Hybrid Propulsion Simulator

Fuel Simulators

6 MWh BESS diesel-electric hybrid on a PSV — DNV-ST-E407 and IEC 62619 compliant, covering shore power, battery thermal management, and zero-emission berthing.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

6 MWh BESS diesel-electric hybrid on a PSV — DNV-ST-E407 and IEC 62619 compliant, covering shore power, battery thermal management, and zero-emission berthing.

## Key Features

Animated single-line electrical diagram with live power flow direction.

Thermal runaway precursor detection, cell voltage spread monitoring.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| BATTERY / GENERATOR CONTROLS | Displays real-time system data, controls, and status indicators. |
| SINGLE-LINE ELECTRICAL DIAGRAM — BESS / HYBRID BUS | Displays real-time system data, controls, and status indicators. |
| ALARM MANAGEMENT — DNV-ST-E407 | Displays real-time system data, controls, and status indicators. |
| POWER &amp; BATTERY GAUGES | Displays real-time system data, controls, and status indicators. |
| SCENARIO / POWER TREND | Displays real-time system data, controls, and status indicators. |
| EMISSION SAVINGS &amp; DP STATUS | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.1 Gauges & Indicators

| Parameter | Unit |
| --- | --- |
| SoC Bank A | % |
| SoC Bank B | % |
| Temp Bank A | °C |
| Temp Bank B | °C |
| Battery Current | A |
| Discharge Power | kW |
| DC Bus Voltage | V |
| Total Ship Load | kW |
| NOx Saving | kg/h |
| SOx Saving | kg/h |
| CO₂ Saving | kg/h |

Colour coding: Green = normal, Amber = warning, Red (blinking) = alarm.

## 4.2 Valve Controls

Click a valve button to toggle its state. Valve states are colour-coded:

Green: OPEN

Red: CLOSED

Amber: TRANSITIONING

Valve Name

0.5C

1C

2C

ECO

TRANSIT

DP

EMERG

SEA WATER COOL

CHILLER

## 4.4 Pipeline Line-Up

The pipeline panel allows valve-by-valve line-up verification for each operational mode.

| Valve ID | Description |
| --- | --- |
| BATT1_ISO | Battery Pack 1 Isolation Contactor |
| BATT2_ISO | Battery Pack 2 Isolation Contactor |
| BUS_TIE | DC Bus Tie Breaker |
| COOL1_IV | Battery Pack 1 Cooling Circuit IV |
| COOL2_IV | Battery Pack 2 Cooling Circuit IV |
| COOL_XV | Coolant Crossover Valve |
| COOL_PUMP1 | Cooling Pump 1 Discharge Valve |
| CHILLER_IV | Chiller Unit Isolation Valve |
| SHORE_IV | Shore Power Connection Isolation |
| CHARGER_IV | Battery Charger Circuit Isolation |
| GEN1_ACB | Generator 1 ACB (Air Circuit Breaker) |
| GEN2_ACB | Generator 2 ACB |
| CONV_IV | Converter/Inverter Isolation |

# 5. Training Scenarios

Select a scenario from the modal dialog when the simulator loads. Each scenario has specific learning objectives and a checklist of actions to complete.

## 5.1 NORMAL HYBRID TRANSIT

G1+G2 running, battery at 85% SoC. Monitor power split and thermal management.

### Actions Required

Step 1: Verify SoC > 30% on both banks

Step 2: Confirm battery temp 20–30°C

Step 3: Monitor cell voltage spread < 50mV

Step 4: Log fuel consumption vs diesel-only baseline

## 5.2 BATTERY-ONLY HARBOUR MANOEUVRING

Zero-emission operation. Generators offline. SoC will deplete — watch for low battery alarm.

### Actions Required

Step 1: Shutdown G1 and G2

Step 2: Confirm battery-only mode active

Step 3: Monitor SoC — plan return to pier when < 40%

Step 4: Record NOx/SOx savings for ECA log

## 5.3 SHORE POWER CONNECTION

Connect shore power at berth. Synchronise voltage, start charging at 1C rate.

### Actions Required

Step 1: Confirm shore cable connected

Step 2: Verify voltage match (6.6 kV)

Step 3: Verify frequency match (50 Hz ± 0.5)

Step 4: Close shore power contactor

Step 5: Set charge rate 1C, monitor temperature

## 5.4 DP OPERATIONS — BATTERY BOOST

DP2 mode engaged. Battery provides instant response for thruster demand spikes.

### Actions Required

Step 1: Enter DP mode on PMS

Step 2: Verify battery boost active

Step 3: Monitor thruster demand peaks

Step 4: Confirm response time < 100ms (battery vs 8s diesel start)

## 5.5 BANK A HIGH TEMPERATURE WARNING

Temperature rising in Bank A. Rate of rise alarm triggered. Activate emergency cooling.

### Actions Required

Step 1: Acknowledge temperature warning alarm

Step 2: Activate chiller cooling

Step 3: Reduce discharge power from Bank A

Step 4: Monitor rate of rise < 2°C/min

## 5.6 G1 GENERATOR TRIP — BATTERY BACKUP

G1 trips unexpectedly. Battery picks up load instantly. Restore generation.

### Actions Required

Step 1: Acknowledge generator trip alarm

Step 2: Confirm battery absorbing load

Step 3: Start G3 to restore capacity

Step 4: Reduce load shedding once G3 online

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System