BlueSim Training Guide

Biofuel Ship Simulator

Fuel Simulators

Biofuel blend operations for marine vessels — covering FAME/HVO blends, compatibility monitoring, filter management, and CII compliance.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Biofuel blend operations for marine vessels — covering FAME/HVO blends, compatibility monitoring, filter management, and CII compliance.

## Key Features

Blend ratio controls, cold filter plugging point monitoring.

Fuel quality alerts and engine compatibility checks.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.1 Gauges & Indicators

| Parameter | Unit |
| --- | --- |
| Fuel Consumption | t/h |
| Flashpoint | °C |
| Viscosity | cSt |
| Acid Number | mgKOH/g |

Colour coding: Green = normal, Amber = warning, Red (blinking) = alarm.

## 4.3 Alarm Definitions

The alarm management system monitors the following conditions:

| Alarm ID | Severity | Description |
| --- | --- | --- |
| SEP_HIGH | WARNING | Separator differential high — check for wax/deposits |
| FILTER_HI | WARNING | Filter differential pressure high — FAME solvency fouling |
| WATER_HI | WARNING | Water content > 0.2% — microbial growth risk |
| WATER_MAX | O | Water content > 0.5% — drain service tank |
| COMPAT_FP | O | Flashpoint near limit — check blend ratio |
| ACID_HI | WARNING | Acid number high — FAME oxidation / degradation |
| CII_LOW | B | CII rating improved — good CII compliance with current blend |
| FUEL_LOW | WARNING | Fuel level low — prepare bunkering |

## 4.4 Pipeline Line-Up

The pipeline panel allows valve-by-valve line-up verification for each operational mode.

| Valve ID | Description |
| --- | --- |
| STTANK_IV | Service Tank (Biofuel) Isolation |
| STTANK_OUT | Service Tank Outlet Valve |
| FTANK_IV | Fuel (Storage) Tank Isolation |
| TRANS_P1 | Transfer Pump 1 Suction Valve |
| TRANS_P1DV | Transfer Pump 1 Discharge Valve |
| BOOST_P1 | Booster Pump 1 Suction Valve |
| BOOST_P1DV | Booster Pump 1 Discharge Valve |
| HEATER_IV | Biofuel Heater Inlet Isolation |
| HEATER_OV | Biofuel Heater Outlet Isolation |
| FILTER_IV | Duplex Filter Inlet Valve |
| FILTER_OV | Duplex Filter Outlet Valve |
| ENG_SUP_V | Engine Supply Valve |
| RETURN_V | Fuel Return Line Valve |
| OVRFL_V | Overflow Return Valve |
| BUNK_IV | Bunker Hose Connection Isolation |
| BUNK_CHK | Bunker Check Valve (confirm flow) |

# 5. Training Scenarios

Select a scenario from the modal dialog when the simulator loads. Each scenario has specific learning objectives and a checklist of actions to complete.

## 5.1 NORMAL — B24 BLEND

UCOME FAME blended 24% with VLSFO. Check separator, filter differential, water content. FIFO fuel management.

### Actions Required

Step 1: Verify separator differential < 25%

Step 2: Check filter DP < 20%

Step 3: Water content < 0.2%

Step 4: Update Fuel Oil Record Book

Step 5: Log CII impact in SEEMP Part 3

## 5.2 CHANGEOVER — VLSFO → B30

Fuel changeover to B30 UCOME blend. Drain, flush, fill procedure. Check compatibility with engine OEM specification.

### Actions Required

Step 1: Verify OEM approves B30 blend

Step 2: Drain service tank — FIFO method

Step 3: Flush fuel lines at low load

Step 4: Load B30 blend — verify quality

Step 5: Monitor filter differential on new blend

## 5.3 FAME COMPATIBILITY ISSUE

FAME solvency dissolving old HFO deposits from tank coating and pipework. Filter blocking. Rubber seals degrading.

### Actions Required

Step 1: Acknowledge filter DP high alarm

Step 2: Clean or replace fuel filter

Step 3: Check rubber gaskets / seals

Step 4: Reduce FAME blend ratio temporarily

Step 5: Contact OEM — check seal compatibility

## 5.4 WATER CONTAMINATION

FAME absorbs water from atmosphere. Tank condensation. Water content rising above limit. Microbial growth risk.

### Actions Required

Step 1: Acknowledge water content alarm

Step 2: Drain water from tank bottom

Step 3: Treat fuel with biocide

Step 4: Increase separator frequency

Step 5: Test fuel after treatment — confirm < 0.2%

## 5.5 CII COMPLIANCE ASSESSMENT

Annual CII rating assessment. Biofuel blend improves CII. Calculate emissions with MEPC.1/Circ.905 methodology.

### Actions Required

Step 1: Compile annual fuel consumption records

Step 2: Apply Cf factor for certified biofuel

Step 3: Calculate CII — compare to required

Step 4: Update SEEMP Part 3 targets

Step 5: Report to flag administration

## 5.6 BIOFUEL BUNKERING — UCOME

Receiving UCOME FAME blend. Verify ISCC sustainability certificate. Check MSDS. Tank coating compatibility before loading.

### Actions Required

Step 1: Verify ISCC-EU certification

Step 2: Check MSDS — confirm biofuel spec

Step 3: Inspect tank coating compatibility

Step 4: Agree blend ratio with supplier

Step 5: Monitor bunkering — no FAME near drains

Step 6: Update Bunker Delivery Note with Cf factor

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## 01 Biofuel Regulatory Reference

## Biofuel Ship Simulator — Regulatory Reference

### MARPOL Annex VI · MEPC.1/Circ.905 · MEPC 83 (April 2025) · CII Framework

Primary regulations: MARPOL Annex VI (NOx, SOx, CII), MEPC.1/Circ.905 (biofuel CII methodology)

Latest update: MEPC 83 (April 2025) — B30 approved without additional NOx testing; MEPC.1/Circ.917 (biofuel blends on bunker ships up to B30)

Biofuels in service: HVO and FAME currently most deployed; B100 UCOME now commercially operational

Unique position: Drop-in capability — no new engine required for HVO; minimal modifications for FAME blends

---

### 1. Types of Marine Biofuels

| Type | Description | Engine Mod | GHG Reduction (WtW) |

|------|-------------|-----------|---------------------|

| **HVO** (Hydrotreated Vegetable Oil) | Vegetable oils/animal fats + hydrotreating | None (true drop-in) | 50–90% |

| **UCOME** (Used Cooking Oil Methyl Ester) | UCO transesterification = FAME | Minimal (seal check) | 65–84% |

| **POME ME** (Palm Oil Mill Effluent ME) | Palm industry residue = FAME | Minimal | 60–75% |

| **B100 FAME** | 100% fatty acid methyl ester | Seal compatibility check | Up to 84% |

| **Bio-LNG** | Biomethane liquefied | Same as LNG | Up to 90% |

| **Bio-methanol** | Renewable methanol | Same as CH₃OH | Up to 95% |

HVO and FAME (UCOME being most common) are the primary commercially deployed marine biofuels as of 2025.

---

### 2. CII Framework (MARPOL Annex VI, Regulation 28)

#### CII Ratings

Ships are rated A–E on Carbon Intensity Indicator annually:

**A:** ≥ 15% below required CII

**B:** 5–15% below required CII

**C:** 5% above to 5% below required CII

**D:** 5–15% above required CII

**E:** ≥ 15% above required CII (requires corrective action plan)

Required CII reduces each year — 5% stricter annually through 2026, then further reductions TBD.

#### Biofuel Impact on CII (per MEPC.1/Circ.905)

Certified biofuels that achieve WtW GHG reduction ≥ 65% compared to fossil reference:

**Cf factor = 0** (treated as zero-emission for CII calculation)

This can move a vessel from D or E rating to C or better

Must be certified by an approved scheme (ISCC-EU, RSB, etc.)

CII calculation:

```

CII = Total CO₂ emitted / (Distance × Deadweight)

CO₂ = Fuel consumption × Cf

For certified biofuel at ≥ 65% WtW reduction: Cf = 0

```

#### MEPC 83 (April 2025) Update

**B30 approved** without additional NOx emission testing under MARPOL Annex VI

Before MEPC 83: NOx retesting required for blends > 30% — now waived for B30

Biofuel blends up to B30 treated same as conventional petroleum fuels for NOx compliance

---

### 3. FAME-Specific Handling Issues

FAME (Fatty Acid Methyl Esters) has unique handling characteristics that require specific crew training — very different from HVO which behaves like conventional diesel.

#### 3.1 Material Compatibility

FAME is a solvent and reacts with certain materials:

| Material | Compatibility | Action |

|----------|---------------|--------|

| Nitrile rubber (NBR) gaskets | **INCOMPATIBLE** — degrades | Replace with EPDM or fluorocarbon |

| Natural rubber hoses | INCOMPATIBLE | Replace with synthetic rubber |

| Copper/brass fittings | INCOMPATIBLE — FAME corrodes | Stainless steel only |

| Zinc/lead components | INCOMPATIBLE | Stainless steel |

| Epoxy tank coatings | Check with manufacturer | Most modern coatings OK |

| Carbon steel pipes | Check lubricity | Generally compatible |

#### 3.2 Cold Flow Properties

FAME has a higher **Cloud Point** and **Pour Point** than conventional fuel

UCOME Cloud Point: typically 0 to −5°C

In cold climates: blend FAME with HVO or use trace heating on fuel lines

**B100 FAME in Arctic conditions:** Not recommended without heating systems

Cold filter plugging point (CFPP) must be verified for port of operation

#### 3.3 Water Absorption

FAME is hygroscopic — absorbs water from atmosphere:

| Water Level | Status | Action |

|-------------|--------|--------|

| < 0.2% vol | Normal | Monitor |

| 0.2–0.5% vol | Warning | Increase separator frequency, check drains |

| > 0.5% vol | Critical | Drain tank, treat with biocide |

Water in FAME promotes:

**Microbial growth** (bacteria, fungi) — creates slime and blocks filters

**Hydrolysis** — breaks down FAME, increases acid number

**Corrosion** of steel components

#### 3.4 Solvency Effect (Filter Fouling)

FAME acts as a powerful cleaning solvent:

When introduced to fuel systems that have previously run on HFO/VLSFO, FAME dissolves old tank deposits and pipe scale

These deposits clog filters rapidly in the first few days on FAME

**First load of FAME:** Expect more frequent filter cleaning for 2–4 days

Carry spare filter elements when loading FAME for the first time

#### 3.5 Oxidation Stability

FAME oxidises faster than conventional fuel during storage:

Maximum storage before quality degrades: **6 months**

After 6 months: acid number rises, gum deposits form

**First-In, First-Out (FIFO)** method mandatory

If stored > 6 months: retest fuel before use

Antioxidant additives can extend stability

---

### 4. Fuel Changeover Procedure

When changing from conventional fuel to FAME-based biofuel blend:

1. Verify OEM compatibility — check engine manufacturer's approval for the intended blend ratio (B10, B24, B30, B100)

2. Drain service tank — remove as much fossil fuel as practical

3. Clean service tank — if switching to high blend ratios (> B50), clean tank of old deposits

4. Prepare receiving tank — check coating compatibility

5. Load biofuel blend — verify quality certificate (ISCC or equivalent)

6. Flush fuel lines — operate at low load, monitor filter differential pressure

7. Inspect filters — change elements as needed during solvency cleaning period

8. Document in SMS — fuel changeover plan must be in SMS per ISM Code

9. Update SEEMP Part 3 — record biofuel consumption for CII reporting

Emergency generators and lifeboat engines:

FAME NOT recommended for long-term storage in emergency generators, lifeboat engines, or diesel-driven fire pumps. These must remain on conventional fuel due to:

Long standby periods (water absorption risk)

Critical safety function

Cold climate performance uncertainty

---

### 5. Bunkering Requirements

#### Biofuel Bunkering Checklist

[ ] Verify sustainability certification (ISCC-EU, RSPO, RSB, or equivalent)

[ ] Confirm WtW GHG reduction ≥ 65% (for Cf = 0 in CII calculation)

[ ] Check MSDS (Material Safety Data Sheet) per SOLAS Chapter VI Reg. 5-1

[ ] Verify tank coating compatibility before loading

[ ] Check flashpoint ≥ 60°C (SOLAS minimum) — FAME flashpoint typically 101°C+ ✓

[ ] Agree blend ratio — ensure supplier FAME content matches specification

[ ] No FAME within 3 metres of deck drains — water-miscible characteristics mean spills spread

[ ] Update Bunker Delivery Note with biofuel content and Cf factor

#### MARPOL limits on bunkering vessels:

**MARPOL Annex II**: no more than 25% FAME in bunker ship cargo tank (unless IMO Type II tanker)

**MEPC.1/Circ.917 (June 2025)**: B30 allowed on conventional bunker ships — significant regulatory improvement

Singapore port: first major hub to implement B30 rule (March 2025)

---

### 6. Comparison with Other Simulators

| Feature | Biofuel | LNG | CH₃OH | NH₃ |

|---------|---------|-----|-------|-----|

| Hazard level | **Low** (drop-in) | High (cryogenic) | High (fire/invisible) | Very High (toxic) |

| Engine modification | None (HVO) / minimal (FAME) | Major (dual-fuel) | Major | Major |

| Training focus | Fuel quality, CII, FAME compatibility | BOG, cold burns | Invisible fire, inerting | IDLH, ESD |

| IMO standard | MARPOL + MEPC.1/Circ.905 | IGF Code | MSC.1/Circ.1621 | MSC.1/Circ.1687 |

| GHG reduction | Up to 84% (certified UCOME) | ~22% fossil LNG | 0% (grey) / 95% (green) | 0% (grey) / 100% (green) |

| Infrastructure | Existing ports (mostly) | Specialised LNG ports | Limited methanol ports | Very limited |

| Ships in service | Many (blended) | 1,860+ fleet | 59 ships (May 2025) | Demo vessels only |

---

### 7. Sources

1. MARPOL Annex VI — NOx, SOx, CII regulations. IMO

2. MEPC.1/Circ.905 — Interim guidance on use of biofuels under MARPOL Annex VI regulations 26, 27, 28. IMO

3. MEPC.1/Circ.917 — Interim guidance: biofuel blends on conventional bunker ships (June 2025). IBIA, safety4sea

4. MEPC 83 outcomes (April 2025) — B30 without NOx retesting approved. OFI Magazine, IBIA

5. ABS Regulatory News — Biofuels as Marine Fuel (Jan 2024). eagle.org

6. DNV — "Use of biofuels in international shipping" (2023). dnv.com

7. Vitol — "Biofuels in maritime transportation" (April 2025). vitol.com

8. CIMAC Guideline — Marine fuels containing FAME (04/2024). cimac.com

9. IRClass — Guidelines on Use of Biofuels on Ships (December 2023). irclass.org

10. Lloyd's Register — Biofuel availability and quality tool, Fuel for Thought: Biofuel report (2024). lr.org

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System