BlueSim Training Guide

Hydrogen Fuel Ship Simulator

Fuel Simulators

Deep cryogenic hydrogen fuel system at −253°C — IMO CCC 11 / MSC.1/Circ.1647, covering vacuum insulation, BOG management, and ortho-para conversion.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Deep cryogenic hydrogen fuel system at −253°C — IMO CCC 11 / MSC.1/Circ.1647, covering vacuum insulation, BOG management, and ortho-para conversion.

## Key Features

Vacuum jacket integrity monitoring and BOG routing decisions.

H₂ leak detection (invisible flame), LFL 4% thresholds.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.1 Gauges & Indicators

| Parameter | Unit |
| --- | --- |
| HP Tank Pressure | bar |
| Tank Level | % |
| H₂ Flow | kg/h |
| Embrittlement | % |

Colour coding: Green = normal, Amber = warning, Red (blinking) = alarm.

## 4.2 Valve Controls

Click a valve button to toggle its state. Valve states are colour-coded:

Green: OPEN

Red: CLOSED

Amber: TRANSITIONING

Valve Name

`

${v.label}

kW

## 4.3 Alarm Definitions

The alarm management system monitors the following conditions:

| Alarm ID | Severity | Description |
| --- | --- | --- |
| LEL10_FC | WARNING | H₂ 10% LEL — Fuel Cell Room (ceiling) |
| LEL25_FC | F | H₂ 25% LEL — Fuel Cell Room — ESD IMMEDIATE |
| LEL10_ER | WARNING | H₂ 10% LEL — Engine / Power Room |
| LEL25_ER | F | H₂ 25% LEL — Engine / Power Room — ESD |
| LEL10_DK | WARNING | H₂ 10% LEL — Deck |
| PRESS_DROP | F | HP tank pressure drop — possible leak |
| EMB_WARN | WARNING | Embrittlement index rising — inspect H₂ lines |
| EMB_CRIT | F | Embrittlement critical — SHUTDOWN — material integrity risk |
| FC_FAULT | WARNING | Fuel cell stack degradation — output reduced |
| ESD_FIRED | F | EMERGENCY SHUTDOWN ACTIVATED |

## 4.4 Pipeline Line-Up

The pipeline panel allows valve-by-valve line-up verification for each operational mode.

| Valve ID | Description |
| --- | --- |
| H2_TANK_IV | H₂ Tank Primary Isolation Valve |
| H2_TANK_IV2 | H₂ Tank Secondary Isolation |
| H2_PRV_VENT | H₂ PRV Vent Stack Isolation |
| H2_FC_IV | Fuel Cell H₂ Supply Isolation |
| O2_FC_IV | Fuel Cell O₂/Air Supply Isolation |
| FC_PURGE | Fuel Cell Purge Valve (N₂) |
| H2_MFV | Master H₂ Fuel Valve |
| H2_DBB1 | H₂ Double Block — Block 1 |
| H2_DBB2 | H₂ Double Block — Block 2 |
| H2_BLEED | H₂ Double Block Bleed (N₂ purge) |
| COOL_LOOP_IV | Cooling Loop Isolation Valve |
| COOL_XV | Coolant Crossover Valve |
| H2_BUNK_IV | H₂ Bunkering Line Isolation |
| H2_BUNK_DBB | H₂ Bunker Double Block Valve |
| N2_PURGE_V | N₂ Purge/Inerting Valve |

# 5. Training Scenarios

Select a scenario from the modal dialog when the simulator loads. Each scenario has specific learning objectives and a checklist of actions to complete.

## 5.1 NORMAL OPERATIONS

H₂ fuel cell propulsion. Compressed storage 350 bar. Ceiling detectors. Odourless — detectors only warning. Embrittlement monitoring active.

### Actions Required

Step 1: Verify ceiling LEL detectors < 5%

Step 2: Check HP tank pressure stable

Step 3: Confirm fuel cell output nominal

Step 4: Log embrittlement index < 20%

## 5.2 H₂ LEAK — FUEL CELL ROOM

Hydrogen detected at ceiling sensors. H₂ is odourless — NO smell warning. Accumulates at ceiling. ESD before 25% LEL.

### Actions Required

Step 1: Acknowledge ceiling LEL 10% alarm

Step 2: Close HP supply valve

Step 3: Activate ESD if LEL reaches 25%

Step 4: Ventilate top-down — ceiling extraction

Step 5: No ignition — H₂ ignites at 0.017 mJ

## 5.3 HYDROGEN EMBRITTLEMENT EVENT

Embrittlement index rising. Microcracking possible in HP carbon steel components. Shutdown required before material failure.

### Actions Required

Step 1: Acknowledge embrittlement alarm

Step 2: Initiate controlled shutdown

Step 3: Inspect HP lines — ultrasonic testing

Step 4: Document in PMS maintenance log

## 5.4 FUEL CELL STACK FAULT

Membrane degradation detected. FC output declining. Switch to battery / diesel backup. Stack inspection required.

### Actions Required

Step 1: Acknowledge FC fault alarm

Step 2: Reduce H₂ supply to faulted stack

Step 3: Engage battery / diesel backup

Step 4: Notify engine room for inspection

## 5.5 H₂ BUNKERING — COMPRESSED GAS

High-pressure H₂ cascade filling. ESD link test. Purge with N₂ before and after. 350 bar transfer — extreme pressure safety.

### Actions Required

Step 1: ESD compatibility test completed

Step 2: Purge hose with N₂

Step 3: Connect HP H₂ hose — certified 700 bar rated

Step 4: Cascade fill — slow initial rate

Step 5: Monitor ceiling LEL at station

Step 6: Close valve — N₂ purge — disconnect

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## 01 Hydrogen Regulatory Reference

## Hydrogen Fuel Ship Simulator — Regulatory Reference

### IMO CCC 11 (Sept 2025) · MSC.1/Circ.1647 · EMSA H₂ Safety Study 2025

CCC 11 Guidelines: Finalised Sept 2025 — approval expected MSC 111 (May 2026)

Scope: Liquefied H₂ concepts (deck-mounted) + compressed gas (deck-mounted)

Also covers: MSC.1/Circ.1647 — Interim Guidelines for Fuel Cell Power Installations

Ships on order: 53 vessels (Jan 2026) — mainly ferries, short-sea, tugs

Key bodies: DNV, Bureau Veritas, Lloyd's Register — all issued H₂ class notations

---

### 1. Why Hydrogen is the Most Challenging Alternative Fuel

Hydrogen has unique properties that require completely different training from all other fuels:

| Property | Hydrogen (H₂) | LNG | Methanol | LPG |

|----------|--------------|-----|---------|-----|

| LEL / UEL | **4% / 75%** — widest of any fuel | 5%/15% | 6%/36.5% | 2.1%/9.5% |

| Ignition energy | **0.017 mJ** — 10× lower than diesel | 0.28 mJ | 0.14 mJ | 0.26 mJ |

| Flame visibility | **Colourless — invisible** | Visible | Invisible | Visible |

| Odour | **None** | None (no odorant) | Detectable | Odorant added |

| Vapour density | **0.07 × air** — rises fastest | 0.55 × air | 1.1 × | 1.5–2.0 × |

| Material attack | **Embrittlement of carbon steel** | None | Minor | None |

| Storage pressure | **350–700 bar** compressed OR **−253°C** LH₂ | −162°C | Ambient | 8 bar |

| GHG potential | **Zero** (green H₂) | Methane slip | Zero (green) | CO₂ only |

The four unique H₂ training hazards not found in other fuels:

1. Ultra-low ignition energy (0.017 mJ — static electricity is sufficient)

2. Widest flammability range (4–75% — most mixtures are ignitable)

3. Invisible, odourless flame — UV detector mandatory, not just visual

4. Hydrogen embrittlement — attacks carbon steel, causing microcracking

---

### 2. Storage Systems

#### Compressed Gas (Most Common in CCC 11 Scope)

**Type IV tanks:** Carbon fibre composite with polymer liner — does NOT embrittle

Operating pressure: 350–700 bar

All tanks fitted on **open deck** per CCC 11 guidelines

Metal components (valves, fittings, piping): must be austenitic stainless steel (316L), Inconel, or other H₂-compatible alloys

**Carbon steel is NOT acceptable** for any H₂ wetted component

#### Liquid Hydrogen (LH₂)

Storage at −253°C — only 20°C above absolute zero

Even more extreme cryogenic hazards than LNG

Vacuum-insulated double-walled tanks mandatory

BOG management critical — higher boil-off rate than LNG due to extreme temperature delta

Very limited operational experience at sea

#### Hydrogen Embrittlement

A unique hazard not found in other marine fuels:

H₂ atoms (smallest element) penetrate metal crystal lattice

Carbon steel becomes brittle — microcracking develops under stress

Can cause sudden catastrophic failure at normal operating pressures

**Prevention:** Use only compatible materials (316L SS, Inconel, copper, aluminium)

**Monitoring:** Ultrasonic testing during scheduled inspections; embrittlement index tracking

---

### 3. Gas Detection Thresholds

H₂ LEL = 4% vol — alarm thresholds are tighter than other fuels due to extremely low ignition energy:

| % LEL | H₂ Concentration | Action |

|-------|-----------------|--------|

| 10% LEL | 4,000 ppm (0.4% vol) | Warning — investigate |

| 25% LEL | 10,000 ppm (1.0% vol) | ALARM — ESD immediately |

| 50% LEL | 20,000 ppm (2.0% vol) | Extreme danger |

| 100% LEL | 40,000 ppm (4.0% vol) | At LEL — explosion |

Critical: ESD threshold is set at 25% LEL (not 50% as for methanol/LNG) due to:

1. Ultra-low ignition energy (0.017 mJ — cannot eliminate all ignition sources)

2. Extremely wide flammability range

Detector placement: CEILING LEVEL only — hydrogen is the lightest gas and rises immediately to the highest point in any space.

Detector technology: Catalytic bead, electrochemical, or optical. All must be rated for H₂ specifically — not all LEL detectors respond linearly to H₂.

---

### 4. Fuel Cell System (per MSC.1/Circ.1647)

The most common propulsion mode for H₂ vessels is the Proton Exchange Membrane Fuel Cell (PEMFC):

H₂ + O₂ → H₂O + Electricity (electrochemical reaction — no combustion)

Efficiency: ~55–65% (significantly better than combustion engines at ~30–40%)

Zero emissions at point of use: only water vapour exhaust

PEMFC operates at ~80°C — no extreme temperatures

Stack degradation: fuel cell membranes degrade over time — output monitoring mandatory

H₂ demand: Very low mass flow rate compared to conventional fuels:

A 500 kW PEMFC consumes approximately 0.18 kg H₂/h per 100 kW

High-pressure compressed storage required due to low density

Battery backup: All H₂ fuel cell ships carry battery buffer capacity for:

ESD response (fuel cell takes time to ramp down)

Peak power demands exceeding FC capacity

Propulsion during ESD / FC fault recovery

---

### 5. CCC 11 Key Requirements (Sept 2025)

Per DNV summary of CCC 11 (8–12 September 2025):

Guidelines include **functional requirements and detailed provisions for all 20 chapters**

Limited to: liquefied hydrogen concepts AND compressed hydrogen — **all storage fitted on open deck**

Regulatory framework: IMO CCC 11 finalised; awaiting MSC 111 approval (May 2026)

Entry into force: expected 1 July 2028

Key design principles from CCC 11:

H₂ fuel storage **must be on open deck** for natural ventilation and dispersion

Double barriers mandatory for all H₂ fuel lines in enclosed spaces

Gas detection at ceiling level in all enclosed spaces near H₂ systems

Emergency ventilation: extraction from ceiling level (top-down flow)

ESD response time: ≤ 30 seconds

All material in contact with H₂: must be embrittlement-compatible

---

### 6. Bunkering (High-Pressure Compressed H₂)

HP H₂ bunkering at 350–700 bar is fundamentally different from any other marine fuel:

1. ESD compatibility test — ship and shore ESD linked before any H₂ flow

2. N₂ purge — all lines purged with dry N₂ before and after H₂ transfer

3. Cascade filling — multiple pressure stages to manage heat of compression

4. Slow initial rate — thermal management of HP cylinders

5. Hose: rated for operating pressure + minimum 3× safety factor (>700 bar rated for 350 bar systems)

6. All hose and connection materials: H₂-compatible (no carbon steel or brass)

7. On completion: N₂ purge → pressure verification = 0 → disconnect

Heat of compression: Filling a 350 bar cylinder causes significant temperature rise. Fast filling can heat the cylinder, potentially reducing its safe pressure capacity. Cascade filling from multiple pre-pressurised sources reduces this effect.

---

### 7. Training Scenarios

#### Normal Operations

Monitor ceiling LEL (should be < 5%)

Monitor fuel cell output and efficiency

Check embrittlement index trend

Verify HP tank pressure stable

#### H₂ Gas Leak

**NO smell** — detector is the only warning

ESD at 25% LEL (not 50% as other fuels)

Ventilation: top-down extraction (unlike LPG which requires bottom extraction)

Eliminate all ignition sources including static (grounding straps)

#### Embrittlement Event

Controlled shutdown required before material inspection

Ultrasonic testing of HP lines and fittings

No restart until material integrity confirmed

Document in PMS and flag administration

#### Fuel Cell Fault

FC output drops → battery/diesel backup automatically

Reduce H₂ supply to faulted stack

Arrange inspection during next port call

Operating on backup power — document limitations

---

### 8. Sources

1. IMO CCC 11 — "Interim guidelines for the safety of ships using hydrogen as fuel, finalised" (Sept 2025). DNV.com, safety4sea.com

2. MSC.1/Circ.1647 — Interim Guidelines for Fuel Cell Power Installations. IMO

3. EMSA — "Study investigating the safety of hydrogen as fuel on ships" (2025). emsa.europa.eu

4. Maritime Education — "Understanding Cryogenic Fuel Handling Risks on Hydrogen-Powered Ships" (2025)

5. Bureau Veritas — "Hydrogen as fuel for ships". bureauveritas.com

6. Lloyd's Register — "Hydrogen's potential and its limits" Fuel for Thought report (2026). lr.org

7. Frontiers in Marine Science — "Hydrogen-powered vessels in green maritime decarbonization" (2025)

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System