BlueSim Training Guide

LNG Fuel Ship Simulator

Fuel Simulators

Operate an LNG-fuelled vessel fuel system — IGF Code compliant, covering BOG management, reliquefaction, and dual-fuel engine operation.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Operate an LNG-fuelled vessel fuel system — IGF Code compliant, covering BOG management, reliquefaction, and dual-fuel engine operation.

## Key Features

Cryogenic storage at −162°C, reliq plant and vaporiser controls.

ESD, bunkering, and gas leak emergency scenarios.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.1 Gauges & Indicators

| Parameter | Unit |
| --- | --- |
| Tank Pressure | bar g |
| Tank Temp | °C |
| Tank Level | % |
| Fuel Flow | t/h |

Colour coding: Green = normal, Amber = warning, Red (blinking) = alarm.

## 4.2 Valve Controls

Click a valve button to toggle its state. Valve states are colour-coded:

Green: OPEN

Red: CLOSED

Amber: TRANSITIONING

Valve Name

`

${v.label}

## 4.3 Alarm Definitions

The alarm management system monitors the following conditions:

| Alarm ID | Severity | Description |
| --- | --- | --- |
| LEL10_GVU | WARNING | CH₄ 10% LEL — GVU Room — investigate |
| LEL50_GVU | F | CH₄ 50% LEL — GVU Room — ESD required |
| LEL10_ER | WARNING | CH₄ 10% LEL — Engine Room |
| LEL50_ER | F | CH₄ 50% LEL — Engine Room — evacuate |
| LEL10_DK | WARNING | CH₄ 10% LEL — Deck / Bunker |
| PRESS_HI | WARNING | Tank pressure high — check BOG / reliq |
| PRESS_CRIT | F | Tank pressure critical — PRV may open |
| TEMP_HI | WARNING | Tank temperature rising — heat ingress |
| BOG_HIGH | B | BOG rate high — reliq at max capacity |
| RELIQ_FAIL | WARNING | Reliquefaction unit offline |
| GVU_FAIL | F | Gas Valve Unit fault — engine trip |
| SLIP_HIGH | B | Methane slip > 3 g/kWh — check engine mode |
| ESD_FIRED | F | EMERGENCY SHUTDOWN ACTIVATED |
| PRV_OPEN | F | PRV OPEN — gas venting to atmosphere |

## 4.4 Pipeline Line-Up

The pipeline panel allows valve-by-valve line-up verification for each operational mode.

| Valve ID | Description |
| --- | --- |
| TANK_PRV_TF | Tank Primary Isolation (Type C vessel) |
| TANK_SRV_TF | Tank Secondary Isolation (inner) |
| TANK_VENT | Tank Vent to Atmosphere Valve |
| MFV_1 | Master Fuel Valve (primary block) |
| MFV_2 | Master Fuel Valve (secondary block) |
| DBB_BLEED | Double Block & Bleed — Bleed Valve |
| GAS_SUPPLY_V | Gas Supply to GVU — Isolation |
| GVU_PILOT | GVU Pilot Valve (instrument air) |
| GVU_MAIN | GVU Main Gas Valve |
| GVU_SHUTOFF | GVU Shutdown Valve (ESD) |
| BOG_RELIQ_V | BOG to Reliquefaction Unit Valve |
| RETURN_V | LNG Fuel Return Line (vapour return) |
| BUNK_ESD | Bunker ESD Coupling Isolation |
| BUNK_DBB1 | Bunker DBB Primary Block |
| BUNK_DBB2 | Bunker DBB Secondary Block |
| BUNK_BLEED | Bunker DBB Bleed Valve |
| BUNK_HOSE_IV | Bunker Hose Connection Isolation |

# 5. Training Scenarios

Select a scenario from the modal dialog when the simulator loads. Each scenario has specific learning objectives and a checklist of actions to complete.

## 5.1 NORMAL OPERATIONS

LNG fuel system steady state. Tank −162°C. BOG consumed by engine or reliquefied. Methane slip monitoring active.

### Actions Required

Step 1: Verify tank temp < −160°C

Step 2: Confirm reliq unit online

Step 3: Check all LEL sensors < 5%

Step 4: Log BOG rate in Engine Log

Step 5: Monitor methane slip < 2 g/kWh

## 5.2 BOG HIGH — RELIQ OVERLOAD

Heat ingress increased. BOG generation exceeds engine demand + reliq capacity. Tank pressure rising toward PRV.

### Actions Required

Step 1: Acknowledge BOG high alarm

Step 2: Increase reliq unit duty to 100%

Step 3: Reduce engine LNG demand if needed

Step 4: Monitor pressure — alert if > 0.8 bar g

Step 5: Prepare for PRV operation

## 5.3 GAS LEAK — GVU ROOM

CH₄ detected in GVU room. LEL rising. GVU fault possible. Prepare ESD and engine mode switch to diesel.

### Actions Required

Step 1: Acknowledge LEL 10% alarm

Step 2: Check GVU — any valve fault?

Step 3: Activate ESD if LEL reaches 50%

Step 4: Switch engine to diesel mode

Step 5: Ventilate — confirm LEL declining

## 5.4 RELIQ UNIT FAILURE

Reliquefaction unit tripped. BOG now only consumed by engine. If BOG > engine demand, pressure will rise.

### Actions Required

Step 1: Acknowledge reliq trip alarm

Step 2: Check reliq fault — attempt restart

Step 3: Monitor tank pressure trend

Step 4: Increase engine LNG consumption if possible

Step 5: Contact technical team for repair

## 5.5 LNG BUNKERING OPERATION

Ship-to-shore LNG transfer. Cool-down lines, connect hose, transfer LNG. ESD test mandatory before start.

### Actions Required

Step 1: Cool down bunker line — confirm OK

Step 2: ESD compatibility test completed

Step 3: Connect cryogenic hose — leak test

Step 4: Open bunker valve — confirm flow

Step 5: Monitor LEL at bunker station

Step 6: Close valve — warm up and disconnect

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## 01 IGF Code LNG Regulatory Reference

## IMO IGF Code — LNG Fuel Ship Regulatory Reference

### For LNG Ship Simulator Training Use

Document: MSC.391(95) — International Code of Safety for Ships Using Gases or Other Low-flashpoint Fuels (IGF Code)

Adopted: June 2015 | In Force: 1 January 2017

Mandatory under: SOLAS Chapter II-1 Part G

Applies to: New ships using gases or other low-flashpoint fuels (flashpoint < 60°C)

Primary fuel: LNG (Liquefied Natural Gas — predominantly methane, CH₄)

Latest amendments: 2019 (ships from 1 Jan 2024), 2020 (cofferdams, fire suppression), 2022 (cryogenic materials)

---

### 1. Why LNG Leads — Market Context (Jan 2026)

**1,259 ships** on order with LNG fuel capability (LR/IHS/Clarksons orderbook)

Most mature alternative fuel framework — IGF Code development took over a decade

The IGF Code's original purpose was to provide clarity on vessel safety standards for LNG fuel. Formal guidelines only entered into force in 2017.

LNG reduces CO₂ by ~20–25% vs HFO, eliminates SOx, reduces NOx by ~80–85%

Key ongoing challenge: **methane slip** — LNG is predominately methane, a greenhouse gas over 25× more potent than CO₂

---

### 2. IGF Code Structure

The Code is organised into chapters covering:

| Chapter | Topic |

|---------|-------|

| 1 | General requirements — goal-based approach |

| 2 | Ship arrangements |

| 3 | Ship structure |

| 4 | Fuel containment |

| 5 | Fuel storage hold spaces |

| 6 | Piping |

| 7 | Bunkering |

| 8 | Gas supply to consumers |

| 9 | Power generation |

| 10 | Machinery installations |

| 11 | Fire safety |

| 12 | Explosion prevention |

| 13 | Machinery space safety |

| 14 | Electrical installations |

| 15 | Control, monitoring, and safety systems |

| 16 | Operations |

Part A-1 contains the specific prescriptive requirements for LNG as fuel — forming the primary basis of all LNG fuel ship training.

---

### 3. Key Definitions (IGF Code)

| Term | Definition |

|------|-----------|

| **Fuel containment system** | Arrangement for storage of LNG — the tank and its associated equipment |

| **Gas-safe space** | A space in which the gas concentration is maintained below 20% LEL |

| **Gas-dangerous space** | A space with potential for explosive gas concentrations |

| **GVU (Gas Valve Unit)** | Safety valve assembly controlling gas supply to consumers — pilot valve, safety shutoff valves |

| **FGSS (Fuel Gas Supply System)** | Complete system delivering LNG/gas from tank to engine |

| **BOG (Boil-off Gas)** | Vapour generated by heat ingress into the cryogenic tank |

| **Reliquefaction unit** | System that recondenses BOG back to liquid LNG |

| **Methane number** | Measure of fuel gas quality — affects combustion knock resistance |

| **Methane slip** | Unburnt CH₄ passing through engine — significant GHG impact |

| **PRV** | Pressure Relief Valve — protects tank from overpressure |

| **Type C tank** | Independent pressure vessel (cylindrical) — most common for fuel LNG |

| **ESD** | Emergency Shutdown System |

---

### 4. LNG Properties — Critical Differences

| Property | LNG (CH₄) | Methanol (CH₃OH) | Ammonia (NH₃) |

|----------|-----------|-----------------|---------------|

| Storage temp | **−162°C** | Ambient (~20°C) | −33°C (refrig) |

| Storage pressure | Near atm | Near atm | Near atm |

| Physical hazard | **Cryogenic cold burn** | Chemical skin burn | Toxic |

| Flammability range | 5–15% vol | 6–36.5% vol | 15–28% vol |

| LEL | **5% vol (50,000 ppm)** | 6% vol (60,000 ppm) | 15% vol |

| Flame visibility | Visible (blue/orange) | Invisible | Yellow/orange |

| GHG hazard | **Methane slip (×25 CO₂)** | CO₂ only | NOx |

| Primary detection | % LEL (CH₄) | % LEL (CH₄-equiv) | ppm (toxic) |

| Warning threshold | **10% LEL** | 10% LEL | 25 ppm |

| Critical alarm | **50% LEL** | 50% LEL | 100 ppm |

| Boil-off | **Continuous (0.1–0.15%/day)** | None | None |

| Special system | **Reliquefaction unit** | N₂ inerting | Cracker unit |

| Unique training risk | Cold burns + asphyxiation + methane slip | Invisible flame | IDLH toxicity |

#### LNG Physical Properties

| Property | Value |

|----------|-------|

| Chemical formula | CH₄ (primarily) + C₂H₆, C₃H₈, N₂ |

| Boiling point | −162°C (−161.5°C pure methane) |

| Density (liquid) | ~446 kg/m³ |

| LHV (energy content) | ~50 MJ/kg / ~22,200 MJ/m³ (liquid) |

| LEL | 5% vol (50,000 ppm) |

| UEL | 15% vol |

| Auto-ignition temp | 537°C |

| Methane GWP (100yr) | 25× CO₂ |

| Vapour density (vs air) | 0.55 — **lighter than air** (rises, disperses) |

| Colour | Colourless |

| Odour | Odourless (unlike pipeline gas — no odorant added to LNG) |

Critical: LNG vapour is lighter than air — unlike methanol or LNG spill scenarios, methane gas rises and disperses upward. Design ventilation accordingly.

---

### 5. Fuel Containment (IGF Code Chapter 4)

#### 5.1 Tank Types for LNG Fuel

Three main tank types are used for LNG fuel (as distinct from LNG cargo):

| Type | Description | Typical Pressure | Use Case |

|------|-------------|-----------------|----------|

| **Type A** | Prismatic/membrane, low pressure | ~0.25 bar g | Large fuel volumes, LNGC |

| **Type B** | Spherical or prismatic, semi-membrane | ~0.25 bar g | Large vessels |

| **Type C** | Cylindrical pressure vessel (bi-lobe) | 3–10 bar g | Most LNG-fuelled vessels |

Type C tanks are most common for LNG-fuelled ships (as distinct from LNG carriers) because they are self-supporting, require no secondary barrier, and can handle higher pressure to manage BOG.

#### 5.2 Key Tank Requirements

All fuel storage tanks: **pressure relief system** appropriate to design and fuel

Fuel storage hold spaces, interbarrier spaces, tank connection spaces: adequate pressure relief

Liquefied fuel pipes: **secondary enclosure** able to contain leakages (per 2019 amendment, mandatory for ships from 1 Jan 2024)

Where gas detection is not fit for purpose in secondary enclosure: pressure or temperature monitoring required

Secondary enclosure must withstand maximum pressure build-up

---

### 6. Boil-Off Gas (BOG) Management — The Key LNG-Specific System

#### 6.1 Why BOG is Unavoidable

LNG is stored at −162°C. Heat ingress from the environment is unavoidable regardless of insulation quality. This heat causes continuous evaporation — the boil-off.

**Typical BOG rate:** 0.10–0.15% of tank volume per day (Type C tanks higher, up to 0.25–0.45%/day depending on ambient temperature)

BOG is predominantly methane (CH₄)

BOG must be managed — it cannot be allowed to simply accumulate (overpressure risk) or vent (GHG/safety risk)

#### 6.2 BOG Management Options

1. Use as fuel — direct BOG to engine as primary or supplemental fuel

2. Reliquefaction — compress BOG and cool back to liquid (Reversed Brayton Cycle or similar)

3. Vent through mast — only in genuine emergency (last resort due to methane GHG impact)

Modern dual-fuel engine ships preferentially use BOG as fuel first, then reliquefy excess. If engine is on diesel mode, all BOG must be reliquefied.

#### 6.3 Reliquefaction System

BOG extracted from tank vapour space by compressors

Compressed BOG passes through heat exchangers (cooled by liquid LNG)

Condensed back to liquid and returned to tank bottom

Typical systems: Reversed Brayton Cycle (N₂ refrigerant) — Wärtsilä Compact Reliq, Cryostar

Reliq unit is energy-intensive: significant auxiliary power consumption

#### 6.4 Simulator BOG Alarm Thresholds

| BOG Rate | Status | Action |

|----------|--------|--------|

| < 0.20%/day | Normal | Monitor |

| 0.20–0.30%/day | High — Warning | Check reliq duty, reduce heat ingress |

| > 0.30%/day | Critical | Reliq at maximum, check for abnormal heat ingress |

| Any venting | PRV operation | ESD review, immediate investigation |

---

### 7. Methane Slip — The Invisible GHG Problem

#### 7.1 What is Methane Slip?

Methane slip = unburnt CH₄ passing through the engine and exiting through the exhaust. Because methane has a GWP of 25× CO₂, even small amounts significantly erode LNG's GHG benefit.

Methane slip affects different engine types differently:

| Engine Type | Typical CH₄ Slip | GHG Benefit vs HFO |

|-------------|-------------------|---------------------|

| 4-stroke low-pressure DF | 3–8 g/kWh | Minimal / negative |

| 4-stroke high-pressure GI | 0.1–0.5 g/kWh | ~25% better than HFO |

| 2-stroke LPDF (WinGD/MAN) | 2–5 g/kWh (at low load) | Variable |

| 2-stroke HPDF (X-DF2.0) | < 0.5 g/kWh | ~20–25% better |

| Spark-ignited gas engine | 0.5–2 g/kWh | ~15–20% better |

#### 7.2 Methane Slip Control

Engine operating mode: gas mode vs diesel pilot

Load: methane slip highest at low engine loads (< 30%)

GVU health: valve leakage through Gas Valve Unit

Pre-chamber design: modern engines use pilot-ignited pre-chambers to reduce slip

#### 7.3 Regulatory Framework

**MAMII (Methane Abatement in Maritime Innovation Initiative)** — LR-led, 20+ shipping stakeholders

**FuelEU Maritime (2025):** GHG intensity accounting includes CH₄ slip (WtW basis)

IMO CII (Carbon Intensity Indicator): increasing annual reductions required

Target benchmark for simulator: < 2 g/kWh methane slip

---

### 8. Gas Detection — LEL Thresholds (IGF Code Chapter 15)

Methane LEL = 5% vol (50,000 ppm) — much lower than methanol or LNG odorant warning systems.

| % LEL | CH₄ Concentration | Action Required |

|-------|------------------|-----------------|

| 10% LEL | 5,000 ppm (0.5% vol) | Warning — investigate source |

| 20% LEL | 10,000 ppm (1.0% vol) | High warning — prepare ESD |

| 50% LEL | 25,000 ppm (2.5% vol) | ALARM — ESD, evacuate, engine to diesel |

| 100% LEL | 50,000 ppm (5.0% vol) | At LEL — explosion imminent |

Detector technology for methane:

Catalytic bead (pellistor): Standard, reliable, but poisoned by silicones

Infrared (IR) absorption: Best performance, not poisoned

Ultrasonic: Detects leaks by sound, not concentration

All detectors: Response time T90 ≤ 30 seconds (IGF Code requirement)

Important: LNG vapour is lighter than air — detectors must be positioned near ceilings in enclosed spaces, not at floor level (unlike methanol or ammonia).

---

### 9. Emergency Shutdown System (IGF Code Chapter 15)

#### ESD Activation Triggers

Manual (from bridge, ECR, and at least one other accessible location)

CH₄ gas detection ≥ 50% LEL (automatic in most designs)

Loss of ventilation in gas-dangerous spaces

High pressure in fuel supply system

Power failure (fail-safe to closed)

#### ESD Cascade Actions

1. MFV (Master Fuel Valve) closes — isolates LNG tank from supply system

2. GVU safety valves close — isolates gas supply to engine

3. Engine switches to diesel/HFO mode automatically

4. FGSS compressors trip

5. Ventilation switches to emergency high mode (60+ ACH)

6. All gas-dangerous spaces: automatic isolation

#### Gas Valve Unit (GVU) — Critical Component

The GVU is the safety-critical valve assembly between the FGSS and the engine. It contains:

Two independent safety shutoff valves (fail-safe closed)

Vent valve (opens to safe location when shutoffs close)

Pressure regulators and gas filters

Position sensors and diagnostics

GVU fault = automatic engine trip to diesel mode.

---

### 10. Bunkering (IGF Code Chapter 7)

#### Key Differences from Conventional Bunkering

**Cryogenic hose** required — PTFE-lined, insulated, rated for −170°C

**Cool-down procedure** mandatory before each transfer — lines must be pre-cooled to prevent thermal shock and boil-off surge

**ESD compatibility test** between ship and shore systems — linked shutdown

**Vapour return** from ship tank to shore (manages BOG displacement during filling)

No odorant in LNG — gas detection the only leak indication

#### Bunkering Procedure Summary

1. Complete ship-shore safety checklist

2. ESD compatibility test (both parties activate, confirm shared shutdown)

3. Cool down bunker lines — circulate LNG at low rate until temperatures stable

4. Confirm LEL at manifold < 5%

5. Increase transfer rate — monitor tank level, pressure, temperature

6. Pre-set high-level cutoff at 97% tank level (leave room for thermal expansion)

7. Completion: close manifold, warm up lines (no cryogenic hose under pressure during disconnect)

8. Disconnect after pressure = 0 and line temperature > −50°C

---

### 11. Cryogenic Safety — Cold Burns

LNG at −162°C causes severe cryogenic cold burns — distinct from heat burns, and often initially painless (nerve damage). This is a hazard unique to LNG among the common alternative fuels.

#### Cold Burn Severity

| Exposure | Effect |

|----------|--------|

| Brief contact (< 5 sec) | Surface frostbite — may appear white/yellow |

| Sustained contact | Deep tissue freeze — vessels rupture |

| LNG splash to eyes | Immediate vision damage |

| Cryogenic spill to skin under clothing | Trapped cold — severe injury even after removal |

#### PPE for Cryogenic Work (per IGF Code Chapter 16)

**Cryogenic gloves** — loose-fitting (allow rapid removal if LNG splashes)

**Face shield** (not goggles — LNG can splash under goggles)

**Cryogenic apron/suit** for bunkering operations

Loose-fitting outer garments — tight clothing traps cryogenic liquid

Safety shoes with no lace loops — LNG can pool in lace loops

#### First Aid for Cold Burns

1. Remove victim from source — do NOT touch affected area with bare hands

2. Remove contaminated clothing carefully (if frozen to skin, do not pull)

3. Warm affected area with tepid water (37–42°C) — NOT hot water, NOT rubbing

4. Do NOT rub or massage — ice crystals in tissue cause mechanical damage

5. Hospital evaluation: even seemingly minor cold burns may be deeper than apparent

6. Inhalation of LNG vapour cloud (displaces oxygen): oxygen administration, fresh air

---

### 12. Training Requirements (STCW — Mandatory from Jan 2017)

STCW amendments entered into force on 1 January 2017 alongside the IGF Code, introducing new mandatory minimum requirements for training and qualifications of masters, officers, ratings, and other personnel on IGF Code ships.

#### Three-Level STCW Training for LNG Ships

| Level | Target Crew | Content |

|-------|-------------|---------|

| **Basic** (STCW V/3-1) | All crew on IGF ships | LNG properties, hazards, PPE, emergency procedures, fire response |

| **Advanced** (STCW V/3-2) | Officers responsible for LNG operations | Full fuel system management, BOG, bunkering, ESD, FGSS operation |

| **Operational** | Chief Engineer, Chief Officer responsible | Reliq systems, GVU maintenance, risk assessment, casualty management |

#### Drill Requirements

| Drill | Frequency |

|-------|-----------|

| Gas alarm response | Monthly |

| ESD activation (functional test) | Per class survey |

| Bunkering emergency drill | Before each bunkering or annually |

| Cryogenic PPE donning | Quarterly |

| Engine mode change (gas → diesel) | Monthly in training mode |

| BOG management scenario | Quarterly |

---

### 13. Sources

1. IMO MSC.391(95) — IGF Code (International Code of Safety for Ships Using Gases or Other Low-Flashpoint Fuels). In force 1 January 2017. imo.org

2. Wikipedia — IGF Code article with amendment history. en.wikipedia.org/wiki/IGF_Code

3. IMO.org — "Safety for gas-fuelled ships – new mandatory code enters into force" (2017). imo.org

4. ABS Advisory — Gas and Other Low-Flashpoint Fuels Advisory (eagle.org)

5. SAFETY4SEA — "IMO Sub-Committee finalizes amendments to the IGF Code" (2023). safety4sea.com

6. Gard Insights — "When cargo becomes fuel: Complexities of boil-off and reliquefaction in LNG shipping" (Nov 2025). gard.no

7. Ship Nerd News — "New Era For Boil-Off Gas (BOG) Management" (2025). shipnerdnews.com

8. Wärtsilä — "A new standard of BOG management" — Compact Reliq technology overview. wartsila.com

9. Lloyd's Register — "Alternative-fuelled ship orders remain significant in 2025" (Jan 2026). lr.org

10. USCG — "LNG as Fuel Design Considerations" — Type C tank, hazardous area classification. dco.uscg.mil

11. Marine Insight 360 — "Safety Features on LNG-Powered Ships" (2025). marineinsight360.com

12. Lloyd's Register — Rules and Regulations for Classification of Ships using Gases or Other Low-flashpoint Fuels (July 2025). lr.org

## 02 Training Scenarios Technical

## LNG Fuel Ship Simulator — Training Scenarios & Technical Documentation

---

### PART A: TRAINING SCENARIOS

#### Scenario 1 — Normal Operations

Normal Operating Parameters (Type C LNG Fuel Tank)

| Parameter | Normal | Warning | Alarm |

|-----------|--------|---------|-------|

| Tank pressure | 0.20–0.50 bar g | > 0.70 bar g | > 0.90 bar g |

| Tank temperature | −164 to −160°C | > −160°C | > −155°C |

| Tank level | 15–95% | < 18% | < 10% |

| Fuel flow | 1.5–6.0 t/h | < 0.8 t/h | < 0.4 t/h |

| BOG rate | < 0.20%/day | > 0.20%/day | > 0.30%/day |

| Reliq duty | 30–80% | > 90% | N/A (BOG alarm triggers) |

| Methane slip | < 2.0 g/kWh | 2–3 g/kWh | > 3 g/kWh |

| LEL (all zones) | < 5% | ≥ 10% | ≥ 50% |

Watch Handover Checks

1. Confirm tank pressure stable and within normal range

2. Verify reliq unit online and operating within duty range

3. Check all LEL sensors < 5% LEL

4. Note BOG rate trend — compare with previous watch

5. Record methane slip rate in Engine Log (MAMII compliance)

6. Verify GVU status — all valves normal

---

#### Scenario 2 — BOG High / Reliquefaction Overload

Background: Heat ingress has increased (summer ambient temperature increase, or change in ship motion causing sloshing). BOG generation rate is exceeding the combined demand of engine consumption + reliq capacity. Tank pressure is rising.

Progression

| Time | BOG Rate | Tank Pressure | Action |

|------|----------|---------------|--------|

| T+0 | 0.12%/day | 0.28 bar g | Normal |

| T+1:00 | 0.22%/day | 0.40 bar g | BOG warning — reliq at 90% |

| T+2:00 | 0.30%/day | 0.60 bar g | BOG alarm — reliq at max |

| T+3:00 | 0.38%/day | 0.75 bar g | Pressure high alarm |

| T+5:00 | 0.42%/day | 0.90 bar g | PRV may open — methane venting |

Required Actions

1. Acknowledge BOG high alarm — note trend

2. Increase reliq unit duty to maximum (100%)

3. If possible: increase engine LNG fuel demand (increase load/speed)

4. Monitor tank pressure — if approaching 0.90 bar g, prepare for PRV operation

5. If PRV opens: notify bridge — methane venting to atmosphere (GHG event, log required)

6. Investigate source of increased heat ingress (ambient temperature, insulation, sloshing)

7. Contact technical team if sustained high BOG rate

Pass/Fail Criteria

✓ BOG alarm acknowledged within 2 minutes: PASS

✓ Reliq duty increased: PASS

✓ Pressure trend monitored and reported: PASS

✗ No action taken — PRV opens without prior notification: FAIL

---

#### Scenario 3 — Gas Leak in GVU Room

Background: A methane leak has developed in the Gas Valve Unit room. LEL is rising. The GVU fault may cause automatic engine trip to diesel mode.

Progression

| Time | GVU Room LEL | GVU Status | Engine |

|------|-------------|------------|--------|

| T+0 | 0% | Normal | DF-Gas mode |

| T+0:30 | 10% | GVU warning | DF-Gas mode |

| T+1:00 | 30% | GVU fault active | Trip warning |

| T+1:30 | 50% | GVU TRIP | Engine → Diesel |

| T+2:00 | 65% | Isolated | Diesel only |

Required Actions (time-critical)

1. Acknowledge LEL 10% alarm — identify zone (GVU Room)

2. Do NOT enter GVU room until gas is below 5% LEL

3. Confirm GVU status: is it responding normally?

4. Prepare engine for mode switch to diesel (notify bridge)

5. Activate ESD if LEL reaches 50%

6. Increase ventilation in GVU room — high-flow mode

7. Monitor LEL trend — confirm declining after ESD + ventilation

8. Investigate source: GVU valve leakage, gasket failure, instrumentation fitting

Key Training Point: Engine trip from gas mode to diesel mode should be seamless (dual-fuel design). Bridge should not notice propulsion interruption.

Pass/Fail Criteria

✓ Alarm acknowledged within 2 min: PASS

✓ ESD activated at ≤ 50% LEL: PASS

✓ Engine mode change handled without propulsion loss: PASS

✗ Entry to GVU room with LEL > 5% without SCBA: FAIL

---

#### Scenario 4 — Reliquefaction Unit Failure

Background: The reliquefaction unit trips due to a compressor fault. BOG is now only managed by engine consumption. If engine cannot consume all BOG, pressure will rise.

Cascade Effects

BOG rate immediately becomes a pressure management problem

If engine is at low load: BOG exceeds fuel demand → pressure rises

If engine is at high load: BOG may be fully consumed → stable pressure

Decision point: increase engine load vs prepare for PRV operation

Required Actions

1. Acknowledge reliq trip alarm

2. Check fault code — attempt restart (single attempt only; if restart fails, call technical support)

3. Note current engine load vs BOG rate

4. Calculate: if BOG > fuel consumption, how long until PRV setpoint reached?

5. Consider increasing engine load/speed to consume more BOG

6. Notify bridge of situation and potential limitations

7. Document in SMS — plan for repair

BOG vs Consumption Calculation:

BOG rate 0.18%/day at 72% tank level = ~0.13% absolute rate

For 1000 m³ tank: 0.13 m³/day liquid LNG = ~0.054 t/day = ~0.0023 t/h

Engine at 74% load consuming 3.1 t/h LNG — easily absorbs BOG

But if engine is at very low load (manoeuvring, < 20%), BOG may exceed consumption

Pass/Fail Criteria

✓ Alarm acknowledged within 2 min: PASS

✓ Restart attempted: PASS

✓ BOG vs consumption balance evaluated: PASS

✓ Bridge notified: PASS

---

#### Scenario 5 — LNG Bunkering Operation

Key LNG Bunkering Differences from Conventional Fuels:

Cryogenic hose — must not be pressurised after shutoff (ice plug, rupture risk)

Cool-down mandatory — thermal shock can crack pipes and fittings

No odorant — gas detection is the only leak indicator

BOG/vapour return line — must be in service during transfer

Step-by-Step Procedure

Pre-bunkering

[ ] Ship-shore safety checklist completed and signed both parties

[ ] ESD compatibility test: activate ship ESD → confirm shore trip; activate shore ESD → confirm ship trip

[ ] Cryogenic hose inspected: no cracks, flexible section no kinks, rated for −170°C

[ ] LEL detectors confirmed operational at manifold area

[ ] Cold-burn PPE: cryo gloves, face shield, cryo apron — all personnel at manifold

[ ] Vapour return line connected

Cool-down Phase (Critical — unique to LNG)

1. Open manifold valve slightly — allow small LNG flow

2. Monitor manifold temperature — target: < −100°C before opening fully

3. Check for abnormal BOG/vapour — thermal shock causes massive BOG generation until pipes cooled

4. Typical cool-down time: 15–30 minutes depending on pipe length and initial temperature

5. Confirm temperatures stable before increasing flow rate

Transfer Phase

1. Open to full flow rate (per agreed plan)

2. Monitor BOG/vapour return flow — must remain active

3. Monitor tank LEL at manifold: must stay < 10% LEL

4. Monitor tank pressure and temperature throughout

5. High-level cutoff: 97% (leave 3% for thermal expansion of LNG)

Completion Phase

1. Close shore valve

2. Warm up bunker lines — critical step unique to LNG

- Leave warm BOG flowing through line until temperature > −50°C

- Do NOT disconnect a cryogenic hose under pressure (ice plug = pressure buildup)

3. Confirm pressure = 0 in line before disconnecting

4. Disconnect hose — blank manifold immediately

Pass/Fail Criteria

✓ ESD compatibility test completed: PASS

✓ Cool-down completed before full flow: PASS

✓ LEL maintained < 10% at manifold: PASS

✓ Lines warmed before disconnect: PASS

✗ Disconnect without warming lines: FAIL — critical cold snap hazard

---

### PART B: SIMULATOR TECHNICAL DOCUMENTATION

#### Architecture Overview

File: lng_ship_simulator.html (~52 KB)

Technology: Pure HTML5/CSS3/JavaScript

Regulatory basis: IMO IGF Code Part A-1, SOLAS II-1/G

#### Key State Variables

```javascript

const S = {

tankPressure: 0.28,   // bar g (Type C tank, operating range 0.2–0.9)

tankTemp:    -162.4,  // °C cryogenic

tankLevel:    72.6,   // %

fuelFlow:      3.1,   // t/h to engine

bogRate:      0.12,   // %/day boil-off rate

reliqDuty:    68.0,   // % reliq unit load

reliqOnline:  true,   // reliq unit state

slipRate:      0.4,   // g/kWh methane slip

engLoad:      74.2,   // % engine load

gvuOk:        true,   // GVU health

prvOpen:      false,  // PRV state (venting)

lel: {

gvuRoom:       0,   // % LEL in GVU Room

engRoom:       0,   // % LEL in Engine Room

deckFwd:       0,   // % LEL on deck

bunkerStation: 0,   // % LEL at bunker manifold

},

};

```

#### BOG Physics Model

```javascript

// BOG rate drift (normal ops):

S.bogRate = 0.12 + Math.sin(S.simTime  0.05)  0.01;

// Pressure rise when reliq fails:

if (!S.reliqOnline) {

S.tankPressure = Math.min(1.1, S.tankPressure + 0.003);

}

// BOG high scenario — increased heat ingress:

S.bogRate = Math.min(0.45, 0.12 + scenarioTime * 0.01);

S.tankPressure = Math.min(1.1, 0.28 + scenarioTime * 0.012);

```

#### Methane Slip Model

```javascript

// Normal: 0.4 g/kWh (modern high-pressure DF engine)

// GVU fault (valve leakage): 3.8 g/kWh

S.slipRate = S.gvuOk ? (0.4 + Math.sin(S.simTime  0.07)  0.1) : 3.8;

```

#### Regulatory Compliance Mapping

| Simulator Feature | IGF Code Reference |

|------------------|-------------------|

| % LEL detection (10% / 50% alarms) | IGF Code Chapter 15: gas detection thresholds |

| Double-wall pipe display | IGF Code Chapter 6 + 2019 amendments |

| MFV (Master Fuel Valve) | IGF Code §8: automatic shutoff |

| GVU health indicator | IGF Code Chapter 8: GVU requirements |

| BOG rate monitoring | IGF Code Chapter 7: BOG management |

| Reliquefaction unit display | IGF Code Chapter 7: pressure control |

| PRV status indicator | IGF Code Chapter 4: pressure relief |

| Methane slip g/kWh | FuelEU Maritime + MAMII targets |

| ESD cascade (MFV+GVU close) | IGF Code Chapter 15: ESD requirements |

| Ventilation 30/60 ACH | IGF Code Chapter 13: machinery space ventilation |

| Type C tank label | IGF Code Chapter 4: fuel containment |

| Cryogenic hazard zones | IGF Code Chapter 16: operational safety |

| Cool-down procedure (bunkering) | IGF Code Chapter 7: bunkering |

| ESD compatibility test | IGF Code Chapter 7: bunkering safety |

| Methane banner alarm | FuelEU + MAMII monitoring requirement |

---

### PART C: COMPARISON — ALL THREE SIMULATORS

| Feature | NH₃ Simulator | CH₃OH Simulator | LNG Simulator |

|---------|--------------|-----------------|---------------|

| IMO Regulation | MSC.1/Circ.1687 (2024) | MSC.1/Circ.1621 (2020) | IGF Code (2017) |

| Primary hazard | Toxicity (300 ppm IDLH) | Fire/explosion | Cryogenic + fire |

| Detection unit | ppm | % LEL | % LEL |

| Warning level | 25 ppm | 10% LEL | 10% LEL |

| Critical alarm | 100 ppm | 50% LEL | 50% LEL |

| Storage temp | −33°C | Ambient | −162°C |

| Unique system | Reliquefaction | N₂ inerting | BOG mgmt + reliq |

| Key GHG issue | NOx | (minimal) | Methane slip ×25 CO₂ |

| Invisible hazard | Toxic cloud (low conc.) | Invisible flame | Odourless gas |

| Ships on order | 45 | 385 | 1,259 |

| Regulatory maturity | Interim (2024) | Interim (2020) | Mandatory (2017) |

| Unique training | SCBA, IDLH | NO water jet, CO₂ | Cold burns, cool-down |

---

### PART D: README

```

lng_simulator_package/

├── README.md

├── simulator/

│   └── lng_ship_simulator.html   ← Open in any browser

└── docs/

├── 01_IGF_Code_LNG_Regulatory_Reference.md

└── 02_Training_Scenarios_Technical.md

```

Quick Start: Open simulator/lng_ship_simulator.html in Chrome, Firefox, or Edge.

Five Training Scenarios:

1. Normal Operations — BOG and methane slip monitoring

2. BOG High / Reliq Overload — pressure management

3. Gas Leak in GVU Room — ESD and engine mode change

4. Reliquefaction Unit Failure — BOG cascade management

5. LNG Bunkering — cool-down procedure, cryogenic safety

Critical Training Points Unique to LNG:

Methane is lighter than air — detectors at CEILING level

LNG burns with visible flame (unlike methanol)

Cold burns from cryogenic contact — loose-fitting cryo PPE

Bunker lines MUST be warmed before disconnect

BOG is continuous and must be managed every watch

Methane slip erodes GHG benefit — monitor g/kWh every watch

Complete Simulator Suite (all three packages together):

NH₃ Simulator: toxicity focus, IDLH management, cryogenic storage

CH₃OH Simulator: invisible fire, inerting, LEL management

LNG Simulator: BOG management, cryogenic safety, methane slip

Together: comprehensive alternative fuel training for STCW compliance

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System