BlueSim Training Guide

Methanol Fuel Ship Simulator

Fuel Simulators

Methanol-fuelled ship fuel system — IMO MSC.1/Circ.1621 compliant, covering flash point monitoring, inerting, and vapour management.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Methanol-fuelled ship fuel system — IMO MSC.1/Circ.1621 compliant, covering flash point monitoring, inerting, and vapour management.

## Key Features

Flash point 11°C — live vapour detection and N₂ blanket control.

Bunkering, ESD, and methanol leak scenarios.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.1 Gauges & Indicators

| Parameter | Unit |
| --- | --- |
| Tank Level | % |
| Tank Temp | °C |
| Tank Pressure | bar |
| Fuel Flow | t/h |
| N₂ Pressure | bar |

Colour coding: Green = normal, Amber = warning, Red (blinking) = alarm.

## 4.2 Valve Controls

Click a valve button to toggle its state. Valve states are colour-coded:

Green: OPEN

Red: CLOSED

Amber: TRANSITIONING

Valve Name

`

${v.label}

## 4.3 Alarm Definitions

The alarm management system monitors the following conditions:

| Alarm ID | Severity | Description |
| --- | --- | --- |
| LEL10_FP | WARNING | LEL 10% — Fuel Prep Room — investigate |
| LEL50_FP | F | LEL 50% — Fuel Prep Room — EVACUATE & ESD |
| LEL10_ER | WARNING | LEL 10% — Engine Room |
| LEL50_ER | F | LEL 50% — Engine Room — EVACUATE & ESD |
| LEL10_DK | WARNING | LEL 10% — Deck Forward |
| FLAME_FP | F | FLAME DETECTED — Fuel Prep Room — INVISIBLE FIRE |
| FLAME_ER | F | FLAME DETECTED — Engine Room — INVISIBLE FIRE |
| O2_HIGH | WARNING | Tank O₂ > 2% — inerting failure — explosion risk |
| N2_LOSS | F | N₂ supply pressure low — inerting compromised |
| DRIP_HIGH | WARNING | Drip tray level high — inspect for leakage |
| FLOW_LOSS | WARNING | Fuel flow loss — engine trip risk |
| ESD_FIRED | ALARM | EMERGENCY SHUTDOWN ACTIVATED |
| CO2_FIRED | ALARM | CO₂ SYSTEM RELEASED — evacuate space |

## 4.4 Pipeline Line-Up

The pipeline panel allows valve-by-valve line-up verification for each operational mode.

| Valve ID | Description |
| --- | --- |
| STTANK_IV | Service Tank Isolation Valve |
| STTANK_OUT | Service Tank Outlet Valve |
| PUMP1_SV | Fuel Pump 1 Suction Valve |
| PUMP1_DV | Fuel Pump 1 Discharge Valve |
| PUMP2_SV | Fuel Pump 2 Suction Valve (standby) |
| PUMP2_DV | Fuel Pump 2 Discharge Valve |
| HEATER_IV | Methanol Fuel Heater Isolation |
| VISC_XV | Viscosity Bypass Crossover Valve |
| MFV | Master Fuel Valve |
| DBB1 | Double Block Valve 1 |
| DBB2 | Double Block Valve 2 |
| DBB_BLEED | Double Block Bleed Valve |
| ENG_SUP_V | Engine Supply Valve |
| RETURN_V | Methanol Return Line Valve |
| BUNK_IV | Bunker Hose Connection Isolation |
| STTANK_FILL | Service Tank Fill Valve |

# 5. Training Scenarios

Select a scenario from the modal dialog when the simulator loads. Each scenario has specific learning objectives and a checklist of actions to complete.

## 5.1 NORMAL OPERATIONS

Steady-state methanol fuel. Ambient temp/pressure. N₂ inerting active. Invisible flame hazard — IR detectors required.

### Actions Required

Step 1: Verify O₂ < 2% — inerting confirmed

Step 2: Confirm all valves open / normal

Step 3: Check LEL detectors < 5% on all zones

Step 4: Log fuel consumption (t/h)

## 5.2 METHANOL LEAK → INVISIBLE FIRE

Flange leak in fuel prep room. LEL rising. Methanol flames are INVISIBLE — UV/IR detector required. ESD mandatory.

### Actions Required

Step 1: Acknowledge LEL 10% alarm

Step 2: Close supply valve — isolate fuel prep

Step 3: Activate ESD before LEL 50%

Step 4: Release CO₂ if flame detected

Step 5: Do NOT use water jet — spreads methanol

## 5.3 INERT GAS FAILURE — O₂ INGRESS

N₂ supply trips. O₂ entering tank ullage. O₂ > 2% creates flammable atmosphere. Restore inerting immediately.

### Actions Required

Step 1: Acknowledge N₂ loss / O₂ alarm

Step 2: Switch to backup N₂ supply

Step 3: Close tank vent to slow ingress

Step 4: Monitor O₂ — confirm declining

Step 5: Do not operate fuel system if O₂ > 8%

## 5.4 BUNKERING OPERATION

CH₃OH transfer. Vapour return required (MSC.1/Circ.1621 §12). Monitor LEL at manifold. ESD compatibility verified.

### Actions Required

Step 1: Connect hose — verify ESD compatibility

Step 2: Activate vapour return line

Step 3: Open bunker valve — confirm flow

Step 4: Monitor LEL at manifold < 10%

Step 5: Close valve — purge line before disconnect

Step 6: Drain and gas-free hose after transfer

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## 01 IMO MSC1 Circ1621 Summary

## IMO MSC.1/Circ.1621 — Interim Guidelines for Safety of Ships Using Methyl/Ethyl Alcohol as Fuel

### Complete Regulatory Reference for Methanol Ship Simulator

Document: MSC.1/Circ.1621

Approved: MSC 102nd session, 4–11 November 2020

Full Title: Interim Guidelines for the Safety of Ships Using Methyl/Ethyl Alcohol as Fuel

Regulatory Basis: IGF Code (MSC.391(95)) + SOLAS II-1/55 Alternative Design

Classification Notations: DNV LFSS(ML) / Lloyd's Register LFPF(GF,ML) / ABS METH

Status as of 2025: Interim — mandatory provisions under development (CCC 12 target, 2026)

---

### 1. Why Methanol Matters (Market Context as of 2025)

**59 methanol dual-fuel ships in operation** (DNV AFI, May 2025)

**369 ships on order** — container ships 60%, bulk carriers 15%, tankers 10%

First major commercial deployment: Maersk container ships (A.P. Møller-Maersk series)

Technology is mature enough for retrofit and new build across vessel types

IMO HTW 11 (2025): methanol/ethyl alcohol training is **highest priority** among alternative fuels — before ammonia, LPG, hydrogen

---

### 2. Four-Element Safety Concept (Core of MSC.1/Circ.1621)

The guidelines are built around four interdependent safety principles:

| Element | Description |

|---------|-------------|

| **1. Segregation** | Fuel system physically separated from accommodation, machinery of category A, and other ignition sources |

| **2. Double barriers** | Double-walled piping and secondary containment around fuel tanks; drip trays under all connections |

| **3. Leakage detection** | Continuous LEL and flame detection in all spaces containing fuel system components |

| **4. Automatic isolation** | Fail-safe valves that close automatically on ESD signal or loss of control power |

---

### 3. Methanol Properties — Key Differences from Ammonia and LNG

| Property | Methanol (CH₃OH) | Ammonia (NH₃) | LNG (CH₄) |

|----------|-----------------|---------------|-----------|

| Physical state | Liquid at ambient | Cryogenic or pressurised | Cryogenic |

| Storage temp | Ambient (~20°C) | −33°C (refrig) | −162°C |

| Storage pressure | Atmospheric | Near atm (refrig) | Near atm |

| Flashpoint | **11°C** (low-flashpoint) | Non-flammable at fuel conc. | −188°C |

| Flammability range | **6–36.5% vol** | 15–28% vol | 5–15% vol |

| Auto-ignition | 455°C | 651°C | 537°C |

| Flame visibility | **INVISIBLE** (pale blue) | Yellow/orange | Blue/orange |

| Primary hazard | **FIRE + EXPLOSION** | TOXICITY | Fire + asphyxiation |

| TLV-TWA | **200 ppm** (ACGIH) | 25 ppm | Asphyxiant |

| IDLH | **6,000 ppm** | 300 ppm | N/A |

| Water miscibility | **Fully miscible** | Partial | Immiscible |

| Biodegradable | Yes — fast | Partial | No |

| CO₂ equivalent | Zero (green methanol) | Zero (green) | ~CH₄ slip |

| Energy density (vol) | 15,800 MJ/m³ | 12,700 MJ/m³ | 20,900 MJ/m³ |

#### Critical Training Point: Invisible Flames

Methanol burns with a nearly invisible pale blue flame. This is the most dangerous characteristic:

A methanol fire may not be visible in daylight

Personnel can walk into a burning methanol fire without seeing it

**UV/IR (ultraviolet/infrared) flame detectors are mandatory** — visual inspection is insufficient

Standard smoke/heat detectors may activate too late

---

### 4. Fuel Storage Requirements (Chapter 5, MSC.1/Circ.1621)

#### 4.1 Tank Location Restrictions

Fuel tanks **must NOT** be located in:

- Accommodation spaces

- Service spaces

- Machinery spaces of Category A

- Control stations

Independent tanks may be accepted on open decks or in a fuel storage hold space

Fuel pipes must NOT be located less than **800 mm from the ship's side**

#### 4.2 Cofferdam Requirements

Cofferdams required around methanol fuel tanks **except where**:

- Tank is adjacent to ship's shell plating below the lowest possible waterline (methanol is water-miscible and biodegradable — reduced environmental risk)

- Tank is bounded by other methanol fuel tanks

- Tank is bounded by the fuel preparation space

Where cofferdams are provided: must have gas/leak detection and be arranged for inerting OR filling with water

#### 4.3 Tank Structural Requirements

Independent tanks must be secured to ship's structure — designed for maximum static/dynamic loads

Fuel tanks on open decks: must be protected against mechanical damage

Tank material: compatible with methanol (carbon steel acceptable; avoid zinc, brass, copper)

#### 4.4 Tank Venting (§6.3) — CRITICAL

All fuel tanks must be fitted with a **controlled tank venting system**

Pressure/Vacuum (P/V) valves required

If PRVs in vent line: vent outlet must have **flame arrestor** certified for endurance burning per MSC/Circ.677

**All fuel tanks must be inerted at all times during normal operation**

Inert gas: typically N₂ or CO₂

Target: **O₂ content < 2% by volume** in tank ullage at all times

Blanking arrangements fitted in inert gas supply line — removable spool piece

---

### 5. Inert Gas System (§6.3) — Most Unique Requirement vs Other Fuels

Methanol has a wide flammability range (6–36.5%) and low flashpoint (11°C). Without inerting, the ullage space above liquid methanol is nearly always within the flammable range at normal ship temperatures.

#### Requirements

Continuous N₂ (or CO₂) supply to all fuel tanks

Tank O₂ monitoring — continuous alarm at > 2% O₂

N₂ supply: minimum 2× normal consumption rate (to allow for losses)

Backup N₂ supply or alternative means of achieving O₂ < 2%

N₂ generator or stored N₂ bottles — sized for full voyage

#### Simulator Alarm Logic

O₂ > 2%: WARNING — "inerting compromised"

O₂ > 8%: ALARM — "ullage flammable — do not operate fuel system"

N₂ pressure < 2 bar: WARNING — "N₂ supply low"

---

### 6. LEL Detection Thresholds

Unlike ammonia (measured in ppm with IDLH at 300 ppm), methanol gas detection is measured in % LEL (Lower Explosive Limit). Methanol LEL = 60,000 ppm = 6% vol:

| % LEL | Methanol Concentration | Action |

|-------|----------------------|--------|

| 10% LEL | 6,000 ppm (0.6% vol) | Warning — investigate |

| 50% LEL | 30,000 ppm (3% vol) | High alarm — evacuate, ESD |

| 100% LEL | 60,000 ppm (6% vol) | At LEL — fire/explosion risk |

#### Detector Types

**Catalytic bead (pellistor):** Standard for LEL; suitable for methanol

**IR absorption:** Good for methanol vapour quantification

**Electrochemical:** Used for lower ppm toxic range

**UV/IR flame detectors:** Mandatory for methanol fire detection (visible spectrum detectors miss methanol flames)

---

### 7. Fuel Preparation Space Requirements (§7, MSC.1/Circ.1621)

Must be located **outside machinery spaces of Category A**

Must have **independent access directly from open deck**

Where separate open deck access is impracticable: airlock required

Door sill: minimum **300 mm** height

Doors: **self-closing, no hold-back arrangements**

Airlock: simple geometrical form

Ventilation: continuous forced extraction — minimum rates per gas dispersion analysis

All potential leak sources (pumps, filters, valves, flanges): in fuel preparation space or equivalent enclosed space with detection

---

### 8. Piping System (§8, MSC.1/Circ.1621)

Fuel pipes: minimum **800 mm from ship's side**

**Double-wall fuel pipes** in enclosed spaces — annulus monitored for leaks

Note: "Double wall fuel pipes are **not** considered as potential sources of release" — they achieve the double-barrier requirement

All pumps: protected against running dry

Pumps capable of exceeding design pressure: must have relief valves in closed circuit (return to upstream side)

Valves: **fail-safe type** — default to closed position on loss of control power

Tank inlet/outlet valves: as close to tank as possible; remotely operated if not easily accessible

#### Master Fuel Valve (MFV)

Automatic shut-off valve in fuel supply line to each consumer

Located **outside** the machinery space of the consumer

Automatically closes on ESD activation

Closes on: (a) ESD signal, (b) fuel leakage detection, (c) loss of ventilation in double-wall annulus

---

### 9. Emergency Shutdown System (ESD)

#### ESD Activation Triggers

Manual activation (bridge, ECR, fuel prep room — minimum 3 locations)

LEL detector > 50% (automatic in most designs)

UV/IR flame detector activation

High-pressure alarm in fuel system (pipe burst)

Loss of ventilation in fuel preparation space

Power failure (fail-safe)

#### ESD Cascade Actions

1. Master Fuel Valve (MFV) closes — isolates tank

2. Supply valve to fuel consumers closes

3. Return valve closes

4. Fuel pump trips

5. Engine switches to backup fuel (HFO/MGO) if dual-fuel

6. Ventilation switches to emergency high capacity

7. CO₂ fixed system armed (manual release by crew)

---

### 10. Fire Suppression (§11, MSC.1/Circ.1621)

#### Critical: Methanol Fires Require Specific Extinguishing Agents

| Agent | Effective on Methanol? | Notes |

|-------|----------------------|-------|

| CO₂ (carbon dioxide) | ✓ YES | Preferred for enclosed spaces; smothers fire |

| Alcohol-resistant foam (AFFF-AR) | ✓ YES | Must be alcohol-resistant — standard AFFF breaks down |

| Water spray/fog | ✓ (dilution) | Can dilute methanol to below flammable concentration; not effective by itself |

| Water jet | ✗ NO | Spreads burning methanol — dangerous |

| Standard AFFF foam | ✗ NO | Methanol destroys standard foam blanket |

| Dry powder (ABC) | Partial | Not ideal; leaves residue |

#### Spaces Requiring Fixed Fire Suppression

Fuel preparation room: CO₂ fixed system

Pump rooms and valve manifold areas

Engine room (where methanol fuel consumers are located)

#### Bunkering Station

Foam system (alcohol-resistant) at bunkering manifold

Emergency quick-closing valves within reach

---

### 11. Bunkering Requirements (§12, MSC.1/Circ.1621)

#### Critical Difference from Conventional Fuels: Vapour Return Mandatory

Methanol has a significant vapour pressure — during transfer, vapour displacement must be managed:

**Vapour return line** from ship tank back to shore (or vapour processing unit)

Prevents methanol vapour cloud formation at manifold

Reduces LEL buildup at bunkering station

#### Bunkering Checklist (per MSC.1/Circ.1621 §12 and MTF Guidelines Sept 2025)

#### Pre-bunkering

[ ] Bunkering plan signed by both parties

[ ] ESD system compatibility verified (ship ↔ shore ESD linked)

[ ] Hose certified for methanol service (PTFE or compatible material)

[ ] All hose connections pressure-tested before opening valves

[ ] Vapour return line connected and verified

[ ] LEL detectors confirmed operational at manifold

[ ] UV/IR flame detectors active at manifold

[ ] Alcohol-resistant foam ready at bunkering station

[ ] Personnel: SCBA available; chemical gloves and splash goggles worn

[ ] Communication confirmed ship-to-shore (primary + backup)

#### Connection

1. Connect bunker hose — methanol-compatible material, PTFE-lined preferred

2. Connect vapour return line

3. Pressure test connection with N₂ before opening fuel valves

4. Install drip tray under manifold connection

#### Transfer

1. Open shore valve slowly — confirm flow at low rate

2. Confirm vapour return flow active

3. Monitor LEL at manifold: must remain < 10% throughout

4. Confirm tank O₂ remains < 2% during filling

5. High-level cutoff: pre-set at 95% tank level

#### Completion

1. Close shore valve

2. Drain bunker hose with N₂ purge

3. Confirm hose pressure = 0 before disconnecting

4. Disconnect vapour return — blank both ends

5. Disconnect bunker hose — blank manifold

6. Document in fuel log

---

### 12. Crew Training Requirements (HTW 11, 2025 — Top Priority)

Per IMO HTW 11 (2025), methanol/ethyl alcohol training has been identified as the highest priority among all alternative fuel training programmes (ahead of ammonia, LPG, and hydrogen).

#### Basic Training Requirements

Methanol properties: invisible flame, flammability, water miscibility, toxicity

Normal operation: fuel system start-up, steady state monitoring, shutdown

LEL detection: alarm thresholds, response procedures

Inerting system: N₂ monitoring, O₂ alarm response

Fire response: correct extinguishing agents; water jet prohibition

PPE: chemical gloves, splash goggles, SCBA for enclosed space entry

#### Advanced Training Requirements

ESD activation and system restart

Fire casualty response: invisible flame identification and suppression

Bunkering: full procedure, vapour return, ESD compatibility

Medical response: methanol ingestion and vapour exposure

Emergency response planning and drill conduct

#### Medical Response — Methanol Exposure

Key hazard: Methanol is metabolised in the body to formic acid and formaldehyde — causes optic nerve damage and metabolic acidosis. Even small ingested quantities are life-threatening.

| Route | Effect | First Aid |

|-------|--------|-----------|

| Ingestion | Fatal in small quantities (as little as 10 mL) | Immediate hospital — antidote: ethanol IV |

| Inhalation | TLV-TWA 200 ppm; IDLH 6,000 ppm | Fresh air, O₂, hospital |

| Eye contact | Severe irritation, vision damage | 15-minute water flush, hospital |

| Skin contact | Absorbed through skin | Remove clothes, wash with water |

Critical: Methanol has a latent period of 12–24 hours before symptoms appear. A crew member may feel fine after exposure but deteriorate rapidly later. All methanol exposure incidents must result in hospital evaluation.

---

### 13. Comparison: Methanol vs Ammonia Simulator (Key Training Differences)

| Aspect | Methanol Simulator | Ammonia Simulator |

|--------|-------------------|-------------------|

| Primary hazard | Fire / explosion | Toxicity |

| Detection metric | % LEL | ppm |

| Warning threshold | 10% LEL | 25 ppm |

| Critical threshold | 50% LEL | 100 ppm (evacuate) |

| Immediately dangerous | 100% LEL | 300 ppm (IDLH) |

| Storage condition | Ambient liquid | Cryogenic −33°C |

| Tank inerting | Mandatory (N₂, O₂ < 2%) | Not required |

| Invisible flame | YES — UV/IR detectors critical | N/A |

| Fire suppression | CO₂, AR-AFFF (NOT water jet) | Water curtain, deluge |

| Bunkering vapour | Vapour return mandatory | ARMS for spill |

| Medical risk | Ingestion even more dangerous | Inhalation toxicity |

---

### 14. Sources

1. IMO MSC.1/Circ.1621 — Interim Guidelines for the Safety of Ships Using Methyl/Ethyl Alcohol as Fuel (7 December 2020). Full text: register-iri.com, wwwcdn.imo.org

2. Maritime Technologies Forum (MTF) — "Guidelines for Conducting Safe Inspection of Methanol Dual-Fuel Ships" (September 2025). maritimetechnologiesforum.com

3. CM Energy — "What are the IMO guidelines for methanol-fueled ships?" cm-energy.com

4. CR Classification Society — "Guidelines for Ships Using Methanol" (July 2022). crclass.org

5. ABS (American Bureau of Shipping) — "Methanol Bunkering: Technical and Operational Advisory" (April 2024). eagle.org

6. Lloyd's Register — LFPF(GF,ML) notation rules and methanol appendix. imorules.com

7. IBIA (International Bunker Industry Association) — "Progress for methanol as fuel for ships". ibia.net

8. DNV — Alternative Fuels Insight (AFI) platform data (59 vessels, 369 on order). dnv.com

9. BIMCO — HTW 11 training prioritisation update (2025). bimco.org

10. Ship Nerd News — "Methanol Safety & Toxicity" (2025). shipnerdnews.com

11. Methanol Institute — "Marine Methanol: Future-Proof Shipping" (May 2023). methanol.org

## 02 Training Scenarios Technical

## Methanol Ship Simulator — Training Scenarios & Technical Documentation

---

### PART A: TRAINING SCENARIOS

#### Scenario 1 — Normal Operations

Objective: Monitor methanol fuel system in steady state. Verify inerting, detection, and valve status.

Normal Operating Parameters

| Parameter | Normal | Warning | Alarm |

|-----------|--------|---------|-------|

| Tank level | 20–92% | < 18% | < 10% |

| Tank temp | 10–40°C | > 45°C | > 55°C |

| Tank pressure | 0.05–0.15 bar | > 0.6 bar | > 0.8 bar |

| Fuel flow | 1.5–5.0 t/h | < 0.8 t/h | < 0.4 t/h |

| N₂ pressure | 3–6 bar | < 2 bar | < 1 bar |

| Tank O₂ content | < 2% | > 2% | > 8% |

| LEL (all zones) | < 5% | ≥ 10% | ≥ 50% |

Trainee Tasks

1. Confirm O₂ < 2% on tank inerting display

2. Verify all LEL sensors reading < 5%

3. Check drip tray level — normal = 0%

4. Log fuel consumption (t/h) per engine load

5. Verify N₂ supply pressure 3–6 bar

6. Confirm flame detectors (UV/IR) showing no detection

---

#### Scenario 2 — Methanol Leak → Invisible Fire

Background: Flange leak in fuel preparation room. Methanol vapour rising, ignition risk increasing. Methanol fires are INVISIBLE in daylight — UV/IR detector activation is the primary indication.

Scenario Progression

| Time | Event | LEL (Fuel Prep) | Drip Tray |

|------|-------|-----------------|-----------|

| T+0 | Flange weep begins | 0% | 0% |

| T+0:30 | First LEL alarm | 10% | 8% |

| T+1:00 | Rising vapour | 30% | 20% |

| T+1:30 | LEL 50% alarm — ESD required | 50% | 35% |

| T+2:00 | Without ESD: ignition risk | 70%+ | 50%+ |

| T+2:30 | UV/IR flame detector activates | Burning | 60% |

Required Actions (in order)

1. Acknowledge LEL 10% alarm within 2 minutes

2. Close supply valve to fuel preparation room

3. Increase ventilation — emergency mode (high ACH)

4. Activate ESD if LEL reaches 50% or rising rapidly

5. Do NOT use water jet — spreads burning methanol

6. If flame detector activates: release CO₂ fixed system

7. Account for all persons — is anyone in fuel prep room?

8. Ventilate space after CO₂ discharge before re-entry

9. SCBA required for any entry after fire

Critical Safety Rule:

> A methanol fire gives NO visible warning. If LEL is rising and an ignition source exists, fire may already be present without visible sign. UV/IR detector is the only reliable indication.

Pass/Fail Criteria

✓ LEL 10% alarm acknowledged within 2 min: PASS

✓ Supply valve closed before LEL 50%: PASS

✓ ESD activated at or before LEL 50%: PASS

✓ CO₂ released on flame detection: PASS

✗ Water jet used on methanol fire: FAIL — critical

✗ Entry to space without SCBA after fire event: FAIL — critical

---

#### Scenario 3 — Inert Gas Failure (O₂ Ingress)

Background: N₂ supply system trips. O₂ begins entering tank ullage. As O₂ rises above 2%, the ullage atmosphere enters the flammable range. This is a common scenario on early methanol vessels.

Scenario Progression

| Time | Event | Tank O₂ | Action Required |

|------|-------|---------|-----------------|

| T+0 | N₂ trip | 2.1% | — |

| T+0:30 | O₂ alarm activates | 2.5% | Acknowledge alarm |

| T+1:00 | Inerting compromised | 4% | Switch to backup N₂ |

| T+2:00 | Without action: flammable ullage | 8% | Stop fuel system operations |

| T+3:00 | Without action: full risk | 12% | ESD + inert gas restoration |

Required Actions

1. Acknowledge N₂ pressure low / O₂ high alarm

2. Switch to backup N₂ supply (or backup N₂ bottles)

3. Close controlled vent (slows O₂ ingress rate)

4. Monitor O₂ decline — confirm inerting restored

5. If O₂ exceeds 8%: do not operate fuel system — stop fuel supply, switch engine to HFO

6. Document in SMS — root cause analysis required

Pass/Fail Criteria

✓ Alarm acknowledged within 2 min: PASS

✓ Backup N₂ activated: PASS

✓ Fuel system stopped if O₂ > 8%: PASS

✗ Continuing fuel system operation with O₂ > 8%: FAIL — explosion risk

---

#### Scenario 4 — Methanol Bunkering

Background: Ship alongside methanol bunkering terminal. Full transfer procedure with vapour return.

Key Difference from Conventional Bunkering:

**Vapour return mandatory** — methanol has significant vapour pressure; vapour displacement must go back to shore, not vent to atmosphere

**ESD compatibility must be confirmed** before connection — ship and shore ESD systems must be linked so either party can stop transfer

Hose and connections must be **methanol-grade** material (PTFE-lined or approved)

Step-by-Step Procedure

1. Confirm bunkering plan: quantity, rate, tank to receive, contingency procedures

2. Connect vapour return line first (before fuel hose)

3. Connect bunker hose — confirm methanol-compatible material

4. ESD link test: activate test signal — confirm both ship and shore ESD trip simultaneously

5. Pressure test connection with N₂ (10 minutes hold test)

6. Establish communication (radio check)

7. Open manifold valves — start transfer at slow rate first

8. Confirm vapour return flow active

9. Monitor LEL at manifold: must remain < 10% throughout transfer

10. Confirm tank O₂ remains < 2% during filling (N₂ compensation may be needed)

11. Pre-set high level cutoff at 95%

12. On completion: close shore valve → drain hose with N₂ → confirm zero pressure → disconnect

Pass/Fail Criteria

✓ Vapour return connected before transfer begins: PASS

✓ ESD compatibility test completed: PASS

✓ LEL maintained < 10% at manifold: PASS

✗ Hose disconnection before N₂ purge complete: FAIL

✗ No ESD link test performed: FAIL

---

### PART B: SIMULATOR TECHNICAL DOCUMENTATION

#### Architecture Overview

File: methanol_ship_simulator.html

Size: ~45 KB

Technology: Pure HTML5 / CSS3 / JavaScript

Regulatory basis: IMO MSC.1/Circ.1621, IGF Code Part A

Same simulation loop architecture as NH₃ simulator (setInterval @ 500ms) but with methanol-specific physics:

#### Key Physics Differences: Methanol vs Ammonia Simulator

| Aspect | Methanol Simulator | Ammonia Simulator |

|--------|-------------------|-------------------|

| Primary detection | % LEL (0–100%) | ppm (0–450+) |

| Tank physics | Ambient temp/pressure drift | Cryogenic refrigeration |

| Critical system | Inert gas (N₂) O₂ tracking | Reliquefaction unit |

| Fire model | UV/IR detector + flame state | N/A |

| Suppression | CO₂ fixed system | Water curtain |

| Unique scenario | O₂ ingress / inert gas failure | N/A in methanol sim |

#### State Object (Key Variables)

```javascript

const S = {

tankLevel:    68.4,  // % (ambient liquid storage)

tankTemp:     19.8,  // °C (ambient temperature)

tankPressure: 0.08,  // bar gauge (controlled vent system)

fuelFlow:     2.8,   // t/h to engine

o2Pct:        2.1,   // % O2 in tank ullage — must stay < 2%

n2Pressure:   4.5,   // bar (N2 inert gas supply)

dripLevel:    0,     // % drip tray fill

lel: {

fuelPrep:   0,     // % LEL in fuel preparation room

engRoom:    0,     // % LEL in engine room

deckFwd:    0,     // % LEL at deck/manifold

cofferdam:  0,     // % LEL in cofferdam

},

flame: {

fuelPrep:   false, // UV/IR detector state

engRoom:    false,

},

};

```

#### Inert Gas Model

```javascript

// O2 rise (inert gas failure scenario):

S.o2Pct = Math.min(12, S.o2Pct + scenarioTime * 0.08);

// Rises from 2.1% to ~12% over ~125 seconds

// O2 recovery (ESD + vent closed):

if (S.esdFired && S.o2Pct > 2.1)

S.o2Pct = Math.max(2.1, S.o2Pct - 0.08);

```

#### LEL Model

```javascript

// Methanol LEL = 6% vol (60,000 ppm)

// Simulator tracks % LEL (0–100%)

// Leak scenario — fuel prep room:

S.lel.fuelPrep = Math.min(80, scenarioTime * 9);

// 9% LEL/second → reaches 50% in ~5.5 seconds (fast-rising)

// CO2 suppression effect:

if (S.co2Active) S.lel.fuelPrep = Math.max(0, S.lel.fuelPrep * 0.90);

// ESD / valve closure decay:

S.lel[zone] = Math.max(0, S.lel[zone] * 0.96);

```

#### Extending the Simulator

Add vapour return flow indicator (for bunkering scenario):

```javascript

// Add to state:

S.vapourReturn = false;

S.vapourReturnFlow = 0; // kg/h

// Inject into bunkering scenario:

if (S.bunkering && S.vapourReturn) {

S.vapourReturnFlow = S.fuelFlow * 0.15; // ~15% of bunker flow

S.lel.deckFwd = Math.min(8, scenarioTime * 0.4); // lower LEL with vapour return

} else if (S.bunkering) {

S.lel.deckFwd = Math.min(25, scenarioTime * 2.5); // higher LEL without vapour return

}

```

Add methanol ingestion casualty scenario:

```javascript

// New scenario: crew member ingests methanol (accidental)

methanol_ingestion: {

title: 'METHANOL INGESTION CASUALTY',

desc: 'Crew member confused methanol for drinking water. Latent period — symptoms in 12-24hr. Immediate ethanol antidote required.',

checklist: [

{ text: 'Isolate casualty — remove from fuel area', done: false },

{ text: 'Call Master and medical officer', done: false },

{ text: 'Administer ethanol (antidote) if available', done: false },

{ text: 'Arrange immediate medevac', done: false },

{ text: 'Document exposure details for hospital', done: false },

],

}

```

#### Regulatory Compliance Mapping

| Simulator Feature | Regulatory Basis |

|-----------------|-----------------|

| LEL 10% warning | MSC.1/Circ.1621 §9: detection threshold |

| LEL 50% alarm + ESD trigger | MSC.1/Circ.1621 §10: ESD activation |

| O₂ < 2% inerting requirement | MSC.1/Circ.1621 §6.3: tank inerting |

| N₂ pressure monitoring | MSC.1/Circ.1621 §6.3: inert gas supply |

| Double-walled pipe display | MSC.1/Circ.1621 §8: double barriers |

| Drip tray level indicator | MSC.1/Circ.1621 §5.4: drip trays under connections |

| UV/IR flame detector | MSC.1/Circ.1621 §9: invisible flame detection |

| Master Fuel Valve (MFV) | MSC.1/Circ.1621 §7: automatic isolation |

| CO₂ fixed system | MSC.1/Circ.1621 §11: fire suppression |

| Fuel prep room with 300mm sill | MSC.1/Circ.1621 §7: fuel preparation space |

| Ventilation 30 ACH display | MSC.1/Circ.1621 §9: ventilation requirements |

| Vapour return bunkering | MSC.1/Circ.1621 §12: bunkering procedure |

| Cofferdam LEL monitoring | MSC.1/Circ.1621 §5.2: cofferdam requirements |

| Pipes ≥ 800mm from ship side | MSC.1/Circ.1621 §8.1: pipe location |

| Flame arrestor on vent | MSC.1/Circ.1621 §6.3: PRV + flame arrestor |

---

### PART C: README

#### Package Contents

```

methanol_simulator_package/

├── README.md

├── simulator/

│   └── methanol_ship_simulator.html   ← Open this in any browser

└── docs/

├── 01_IMO_MSC1_Circ1621_Summary.md

└── 02_Training_Scenarios_Technical.md  ← This file

```

#### Quick Start

Open simulator/methanol_ship_simulator.html in Chrome, Firefox, or Edge.

No internet, no server, no install required.

#### Scenarios

| Scenario | Primary Skill |

|----------|---------------|

| Normal Operations | Inerting monitoring, alarm management |

| Methanol Leak → Invisible Fire | ESD, CO₂, NO water jet rule |

| Inert Gas Failure | O₂ monitoring, N₂ backup |

| Bunkering | Vapour return, ESD link, LEL control |

#### Companion NH₃ Simulator

This package is designed to complement the NH₃ Fuel Ship Simulator (separate package).

Together they cover the two most commercially significant alternative marine fuels for STCW training.

#### Key Training Message for Both Fuels

| Methanol | Ammonia |

|----------|---------|

| **You cannot see the fire** | **You cannot smell it fast enough** |

| UV/IR detectors are your eyes | Gas detectors are your nose |

| CO₂ — not water jet | Water curtain — not CO₂ |

| O₂ must stay < 2% | Temperature must stay < −30°C |

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System