BlueSim Training Guide

Wind-Assisted Propulsion Simulator

Fuel Simulators

Flettner rotor and wing sail operations — IACS UR E27 and IMO MEPC.1/Circ.896 compliant, with live CII rating, Magnus effect physics, and fuel savings dashboard.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Flettner rotor and wing sail operations — IACS UR E27 and IMO MEPC.1/Circ.896 compliant, with live CII rating, Magnus effect physics, and fuel savings dashboard.

## Key Features

5 rotors with deploy/stow, RPM, and spin direction controls per rotor.

Live CII A–E badge, TWA wind rose, and fuel/CO₂ savings readout.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| FLETTNER ROTOR CONTROL | Displays real-time system data, controls, and status indicators. |
| WASP SCHEMATIC — SIDE ELEVATION &amp; WIND DIAGRAM | Displays real-time system data, controls, and status indicators. |
| ALARM MANAGEMENT SYSTEM | Displays real-time system data, controls, and status indicators. |
| WIND &amp; PROPULSION GAUGES | Displays real-time system data, controls, and status indicators. |
| SCENARIO / EFFICIENCY TREND | Displays real-time system data, controls, and status indicators. |
| FUEL SAVINGS &amp; CII DASHBOARD | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.1 Gauges & Indicators

| Parameter | Unit |
| --- | --- |
| True Wind Speed | kn |
| True Wind Angle | ° |
| App Wind Speed | kn |
| Ship Speed | kn |
| WASP Thrust | kN |
| WASP Power Equiv | kW |
| Heel Angle | ° |
| Fuel Saving | % |
| Saved t/day | t/day |
| CO₂ Saved | t voyage |
| ME Load | % |

Colour coding: Green = normal, Amber = warning, Red (blinking) = alarm.

## 4.2 Valve Controls

Click a valve button to toggle its state. Valve states are colour-coded:

Green: OPEN

Red: CLOSED

Amber: TRANSITIONING

Valve Name

WING SAIL

ANGLE: 45°

−

+

AUTO BALLAST

## 4.4 Pipeline Line-Up

The pipeline panel allows valve-by-valve line-up verification for each operational mode.

| Valve ID | Description |
| --- | --- |
| HYD_PUMP1 | Hydraulic Pump 1 Supply Valve |
| HYD_PUMP2 | Hydraulic Pump 2 Supply Valve |
| HYD_XV | Hydraulic System Crossover Valve |
| ROTOR1_IV | Rotor Sail 1 Hydraulic Isolation |
| ROTOR2_IV | Rotor Sail 2 Hydraulic Isolation |
| ROTOR3_IV | Rotor Sail 3 Hydraulic Isolation |
| ROTOR4_IV | Rotor Sail 4 Hydraulic Isolation |
| HYD_RETURN | Hydraulic Return Line Valve |
| HYD_FILTER | Hydraulic Filter Bypass Valve |
| PWR_R1 | Rotor 1 Power Contactor (deploy) |
| PWR_R2 | Rotor 2 Power Contactor (deploy) |
| CTRL_BUS | Control System Bus Isolation |

# 5. Training Scenarios

Select a scenario from the modal dialog when the simulator loads. Each scenario has specific learning objectives and a checklist of actions to complete.

## 5.1 NORMAL OPERATIONS

Steady-state monitoring. All five rotors deployed at beam reach. CII tracking active.

### Actions Required

Step 1: Confirm all 5 rotors deployed

Step 2: Verify TWS within operating range

Step 3: Check heel angle < 3°

Step 4: Log CII attained value

## 5.2 BEAM REACH OPTIMISATION

TWA 90° — optimal Magnus conditions. Maximise WASP thrust by tuning rotor RPM and direction.

### Actions Required

Step 1: Set TWA to 90° (beam reach)

Step 2: Deploy all 5 rotors

Step 3: Set RPM to 180–220 (spin ratio 2.5–3)

Step 4: Achieve fuel saving > 15%

Step 5: Confirm CII rating A or B

## 5.3 UPWIND PENALTY ASSESSMENT

TWA 30° — adverse angle. WASP provides minimal benefit. Assess penalty and optimise.

### Actions Required

Step 1: Note reduced WASP thrust at TWA 30°

Step 2: Reduce RPM to minimise drag

Step 3: Increase main engine assist

Step 4: Document fuel consumption increase

## 5.4 HEAVY WEATHER — ROTOR STOW

Wind speed rising above limits. Execute mandatory stow procedure per IACS UR E27 §5.2.

### Actions Required

Step 1: Acknowledge wind speed alarm

Step 2: Reduce rotor RPM to zero

Step 3: Stow all rotors (retract)

Step 4: Confirm all rotors stowed

Step 5: Increase main engine to 100%

## 5.5 ROTOR 3 MOTOR FAILURE

Motor overcurrent on rotor 3. Isolate unit, stow, redistribute thrust to remaining rotors.

### Actions Required

Step 1: Acknowledge rotor 3 overcurrent alarm

Step 2: Stop rotor 3 (RPM to zero)

Step 3: Stow rotor 3

Step 4: Increase RPM on rotors 1,2,4,5 to compensate

## 5.6 MAXIMUM EFFICIENCY RUN

All conditions optimal. Target CII A, maximise fuel savings, show CO₂ reduction.

### Actions Required

Step 1: Deploy all 5 rotors

Step 2: Tune RPM to spin ratio 3 (optimal)

Step 3: Set TWA to optimal sector 60–150°

Step 4: Achieve > 20% fuel saving

Step 5: Confirm CII rating A

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System