BlueSim Training Guide

GMDSS Console Trainer

Bridge & Navigation

GMDSS console training covering VHF/MF/HF DSC, EPIRB, SART, NAVTEX, and Inmarsat-C procedures per SOLAS IV and STCW A-IV/2.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

GMDSS console training covering VHF/MF/HF DSC, EPIRB, SART, NAVTEX, and Inmarsat-C procedures per SOLAS IV and STCW A-IV/2.

## Key Features

VHF DSC Ch16 and Ch70 monitoring with distress alert simulation.

EPIRB and SART status panels with activation sequences.

NAVTEX receiver with authentic message display.

MAYDAY/PAN PAN call procedure guidance.

GMDSS Log Book with auto-timestamped entries.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| VHF DSC CONTROLLER | Displays real-time system data, controls, and status indicators. |
| GMDSS EQUIPMENT STATUS | Displays real-time system data, controls, and status indicators. |
| GMDSS LOG BOOK | Displays real-time system data, controls, and status indicators. |
| DISTRESS PROCEDURE | Displays real-time system data, controls, and status indicators. |
| WATCH SCHEDULE | Displays real-time system data, controls, and status indicators. |
| COMMUNICATION CHECKLIST'; rightPanel.appendChild(hdr); | Displays real-time system data, controls, and status indicators. |
| SEA AREA & CONTACTS'; rightPanel.appendChild(hdr2); c | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## GMDSS Console — Technical Reference

### Overview

The Global Maritime Distress and Safety System (GMDSS) is an internationally agreed-upon set of safety procedures, equipment types, and communication protocols used to increase safety and make it easier to rescue distressed ships, boats, and aircraft. This simulator trains operators in distress alert procedures, communication protocols, and equipment operation.

### Regulatory Framework

**SOLAS Chapter IV** — Radiocommunications (entire chapter)

**STCW A-IV/2** — Mandatory minimum requirements for GMDSS radio operators

**ITU Radio Regulations** — International telecommunications union rules

**IMO MSC/Circ.1082** — Revised performance standards for GMDSS

**IMO Assembly Resolution A.801(19)** — Criteria for provision of mobile satellite communication systems

**IMO MSC.306(87)** — Amendments to SOLAS Chapter IV

### Sea Areas

| Area | Coverage | Required Equipment |

|------|----------|--------------------|

| A1 | Within VHF DSC range (20-50nm) | VHF DSC, SART, EPIRB |

| A2 | Within MF DSC range (400nm) | VHF + MF DSC, SART, EPIRB |

| A3 | Within geostationary satellite coverage (70°N/S) | All above + Inmarsat or MF/HF |

| A4 | Polar regions | All above + HF DSC |

### Equipment Overview

| Equipment | Function | Watch Frequency |

|-----------|----------|-----------------|

| VHF DSC | Short-range distress/urgency/safety calls | Ch70 DSC + Ch16 voice |

| MF DSC | Medium-range distress calls | 2187.5 kHz DSC |

| HF DSC | Long-range distress calls | 4207.5/6312/8414.5/12577/16804.5 kHz |

| EPIRB | Emergency Position-Indicating Radio Beacon | 406 MHz (Cospas-Sarsat) |

| SART | Search and Rescue Transponder | 9 GHz radar band |

| NAVTEX | Navigational text messages | 518 kHz (English international) |

| Inmarsat-C | Data/telex communications | L-band satellite |

### DSC Distress Procedure

1. Lift guard/remove safety cover

2. Press and hold DISTRESS button for 5 seconds

3. Unit transmits MMSI, nature of distress, position (from GPS), time (UTC)

4. Monitor Ch16 for acknowledgement

5. If no acknowledgement in 5 minutes, retransmit

6. On acknowledgement: switch to Ch16 for voice communication

### MAYDAY Call Format (Voice)

```

MAYDAY MAYDAY MAYDAY

THIS IS [ship name] [ship name] [ship name]

MAYDAY [ship name]

MMSI: [9-digit MMSI]

POSITION: [lat/lon or bearing/distance from landmark]

NATURE OF DISTRESS: [fire/flooding/collision/person overboard/etc.]

NUMBER OF PERSONS ON BOARD: [number]

ANY OTHER RELEVANT INFORMATION: [list/cargo/assistance required]

OVER

```

### PAN PAN Call Format (Urgency)

```

PAN PAN PAN PAN PAN PAN

ALL STATIONS (or specific coast station)

THIS IS [ship name]

MMSI: [number]

POSITION: [position]

NATURE OF URGENCY: [man overboard/medical/etc.]

OVER

```

### NAVTEX Message Categories

| Code | Category |

|------|---------|

| A | Navigational warnings |

| B | Meteorological warnings |

| C | Ice reports |

| D | Search and rescue information |

| E | Meteorological forecasts |

| F | Pilot service messages |

### EPIRB

**406 MHz EPIRB**: Sends encoded digital signal with vessel MMSI/registration to Cospas-Sarsat satellites

**121.5 MHz homing signal**: For SAR aircraft/vessel homing

**Activation**: Manual (break glass and pull pin) or automatic (hydrostatic release at 1-4m depth)

**Registration**: Must be registered with national authority; update if vessel changes flag/name

### Battery Requirements

GMDSS batteries: Must be capable of powering all distress equipment simultaneously for minimum 1 hour

NAVTEX/EPIRB: Independent battery supply

Annual inspection and testing required

### Watch Schedule (SOLAS)

VHF Ch16: Continuous (when practicable)

VHF Ch70 DSC: Continuous automated watch

MF 2187.5 kHz DSC: Continuous automated watch (Area A2+)

NAVTEX: Continuous reception (recommended)

### Scenarios in Simulator

| Scenario | Key Learning Points |

|----------|---------------------|

| Distress Call Activation | DSC alert sequence, MAYDAY broadcast, Ch16 watch |

| Man Overboard MAYDAY | MOB position relay, coordination with MRCC |

| NAVTEX Weather Warning | Message parsing, voyage decision |

| Urgency PAN PAN Call | Format, relay obligations |

### References

IMO GMDSS Manual 2022 Edition

ITU Radio Regulations Volume 1 (2020)

STCW Convention and STCW Code, 2017 Edition

IMO Assembly Resolution A.803(19): Performance standards for shipborne radio equipment

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System