BlueSim Training Guide

Inert Gas System Simulator

Cargo & Tanks

Interactive flue-gas IGS simulator for oil tankers covering inerting, topping-up, purging and O₂ alarm response per SOLAS II-2/Reg.4.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Interactive flue-gas IGS simulator for oil tankers covering inerting, topping-up, purging and O₂ alarm response per SOLAS II-2/Reg.4.

## Key Features

Live O₂ content monitoring with 8% alarm threshold and trend chart.

Dual IG blower start/stop with fault simulation and standby switchover.

4-tank valve lineup with animated flow schematic and pressure display.

Three training scenarios: inerting sequence, O₂ high alarm, blower failure.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| SYSTEM STATUS & CONTROLS | Displays real-time system data, controls, and status indicators. |
| IGS PROCESS SCHEMATIC | Displays real-time system data, controls, and status indicators. |
| ALARMS & TANK O₂ | Displays real-time system data, controls, and status indicators. |
| PIPELINE LINEUP | Displays real-time system data, controls, and status indicators. |
| O₂ TREND — DECK MAIN | Displays real-time system data, controls, and status indicators. |
| OPERATIONS LOG | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## Inert Gas System (IGS) — Technical Reference

### Regulatory Framework

**SOLAS II-2/Regulation 4**: Requires inert gas systems on tankers of 20,000 DWT and above carrying crude oil or petroleum products with flash points not exceeding 60°C.

**IGC Code Chapter 9**: Requirements for inert gas systems on gas carriers.

**IBC Code Chapter 9**: Requirements for chemical tankers.

**ISGOTT 6th Edition**: Industry guidance for safe tanker operations.

**IMO MSC/Circ.353**: Revised guidelines for inert gas systems.

### Operating Principles

#### Flue Gas Type IGS

The flue gas IGS uses combustion gases from the ship's boiler or a dedicated IG generator. Flue gases (primarily N₂ and CO₂) are cooled and scrubbed to remove sulfur dioxide and particulates before delivery to cargo tanks.

Gas Composition (typical flue gas):

Nitrogen (N₂): 83–85%

Carbon Dioxide (CO₂): 13–15%

Oxygen (O₂): 0.5–5% (target < 5% for inerting)

Sulfur Dioxide (SO₂): < 0.1% (after scrubbing)

#### Inerting Process

Inerting replaces the hydrocarbon/air mixture in cargo tanks with inert gas, reducing O₂ content below the level that supports combustion.

Flammability Triangle:

Fuel-rich mixtures (above UFL) are non-flammable but dangerous during dilution

Inerted atmospheres with O₂ < 8% cannot support ignition of hydrocarbon vapours

Target O₂ ≤ 5% (vol) for safe inerting (ISGOTT recommendation)

### Key Parameters

| Parameter | Normal Range | Alarm Level | Action Level |

|-----------|-------------|-------------|--------------|

| O₂ Content (deck) | < 5% vol | > 8% vol | STOP operations |

| IG Pressure (tank) | 100–1000 mmWC | < 100 mmWC (low) | Top up immediately |

| IG Pressure (deck) | 500–2000 mmWC | > 2500 mmWC | Check P/V breaker |

| Scrubber outlet temp | < 45°C | > 65°C | Increase cooling |

| IG delivery temp | < 45°C | > 65°C | Alarm, check scrubber |

| Scrubber water flow | 15–25 m³/h | < 10 m³/h | Alarm |

### Deck Seal

The deck water seal prevents back-flow of cargo vapours to the machinery spaces. It must always be operational when the IG supply valve is closed and cargo operations are ongoing.

Water seal height: minimum 200 mm

P/V breaker setting: +/- 2,100 mmWC (pressure), -350 mmWC (vacuum)

### Pressure/Vacuum Breaker (P/V Breaker)

Protects tanks from over-pressure and excessive vacuum

Pressure setting: +2,100 mmWC (+210 mbar)

Vacuum setting: −350 mmWC (−35 mbar)

### Operational Modes

#### 1. Inerting (Gas-Freeing Preparation)

Used before maintenance or gas-freeing. Reduces O₂ to < 5% vol.

Sequence: Open uptake valve → Start scrubber → Start IG blower → Open deck seal → Open deck main valve → Open individual tank valves

Monitor O₂ content continuously

#### 2. Topping-Up

Used to maintain positive pressure in tanks during voyage.

Supply IG when tank pressure drops below 100 mmWC

Typical supply: 15–30 minutes per set of tanks

#### 3. Purging

Replaces inert gas with fresh air for tank entry (after inerting, before gas-freeing).

Must reduce HC vapour to < 1% LFL before introducing air

Monitor both O₂ AND HC concentration

### Equipment Components

1. IG Blower (Fan): Centrifugal type, 2 units (duty/standby). Typical capacity: 6,000–8,000 m³/h at 1,500–2,500 mmWC.

2. Scrubber Tower: Counter-current water scrubbing. Removes SO₂ and cools gas.

3. Deck Water Seal: Hydraulic non-return valve.

4. Deck Isolating Valve: Main valve to cargo main.

5. Pressure/Vacuum Breaker: Emergency protection device.

6. O₂ Analyser: Paramagnetic or zirconia cell type. Installed in deck main.

7. Non-Return Valve: In IG supply line, additional protection.

### Alarm Conditions

| Alarm | Cause | Corrective Action |

|-------|-------|-------------------|

| O₂ High (>8%) | Blower fault, uptake valve issue | Stop blower, investigate |

| IG Pressure Low | Tank valve open, no supply | Start blower, check lineup |

| Scrubber Water Low | Pump failure, low sea suction | Switch to standby pump |

| IG Temp High (>65°C) | Scrubber inefficiency, high sea temp | Increase scrubber water flow |

| Deck Seal Fault | Low water, blocked drain | Restore water supply |

| Blower Trip | Motor overload, vibration | Reset/switch to standby |

### IMO Key Definitions

**Inerted condition**: O₂ content reduced to and maintained at or below 8% by volume

**Gas-free**: Free of toxic, flammable vapours; safe for hot work when O₂ > 20.9%

**Purging**: Using IG to reduce HC vapour concentration to < 2% vol before introducing air

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System