BlueSim Training Guide

Incinerator Operation Simulator

Engine Room Equipment

Marine incinerator operations for solid waste and sludge oil disposal per MARPOL Annex V, Annex I and IMO MEPC.76(40).

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Marine incinerator operations for solid waste and sludge oil disposal per MARPOL Annex V, Annex I and IMO MEPC.76(40).

## Key Features

Teamtec OG 200C type incinerator with primary and secondary chamber temperature monitoring.

Sludge oil burner (OWC) and solid waste combustion (SWC) with atomising air system.

Port restriction enforcement — incineration prohibited within port limits per MARPOL.

Garbage Record Book logging with position, category and quantity entries.

Scenarios: Normal Waste Incineration, Low Temperature Fault, Port Restriction Warning.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| CONTROLS | Displays real-time system data, controls, and status indicators. |
| P&amp;ID SCHEMATIC — INCINERATOR OG 200C | Displays real-time system data, controls, and status indicators. |
| GAUGES &amp; ALARMS | Displays real-time system data, controls, and status indicators. |
| ACTIVE ALARMS | Displays real-time system data, controls, and status indicators. |
| PIPELINE LINEUP | Displays real-time system data, controls, and status indicators. |
| OPERATION LOG &amp; GARBAGE RECORD BOOK | Displays real-time system data, controls, and status indicators. |
| CHECKLIST &amp; GRB | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.2 Valve Controls

Click a valve button to toggle its state. Valve states are colour-coded:

Green: OPEN

Red: CLOSED

Amber: TRANSITIONING

Valve Name

SLUDGE INL [C]

ATOM AIR [C]

COOLING W [C]

FLU DAMPER [C]

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## Incinerator Operation — Technical Reference

### Regulatory Framework

#### MARPOL Annex I — Oil Pollution Prevention

**Regulation 13**: Sludge oil must be managed onboard. Incineration is a permitted disposal method.

Sludge oil (OWC — Oily Waste Category) may be incinerated in approved marine incinerators.

Incineration of cargo residues of noxious liquid substances is NOT permitted.

#### MARPOL Annex V — Garbage Management

**Regulation 3**: Prohibits discharge of garbage except where permitted.

Incineration is an approved method for disposal of solid waste at sea.

**Cannot operate within port limits** or special areas unless local authority permits.

Food waste, paper, rags, glass, metal, crockery — all permissible for incineration at sea.

#### IMO MEPC.76(40) — Standard Specification for Shipboard Incinerators

Adopted 1997; applies to incinerators installed on or after 1 January 2000.

Requires type approval from flag state administration.

Prohibits incineration of: PVC, contaminated packaging materials with heavy metals, polychlorinated biphenyls (PCBs), sewage sludge/OWC when underway in port, exhaust gas cleaning residues.

---

### Equipment: Teamtec OG 200C / Alfa Laval Type

#### Primary Chamber (Solid Waste Combustion — SWC)

| Parameter | Value |

|-----------|-------|

| Operating temperature | 850–1000°C |

| Minimum ignition temperature | 650°C |

| Thermal capacity | 200,000 kcal/h |

| Solid waste feed rate | Up to 100 kg/h |

| Residence time | ≥2 seconds at >850°C |

#### Secondary Chamber (Afterburner / OWC)

| Parameter | Value |

|-----------|-------|

| Operating temperature | >900°C |

| Purpose | Complete combustion of volatiles, CO reduction |

| Flue gas residence time | ≥1 second at >900°C |

#### Sludge Oil Burner (OWC)

| Parameter | Value |

|-----------|-------|

| Fuel | Sludge oil / HFO sludge |

| Flow rate | 0–35 kg/h |

| Atomising medium | Compressed air (5–7 bar) |

| Sludge oil pressure | 2.5–4.0 bar |

| Preheating temp | 50–80°C |

#### Combustion Air System

| Parameter | Value |

|-----------|-------|

| Primary air | Underfire air — promotes solid combustion |

| Secondary air | Overfire air — turbulence in secondary chamber |

| Fan type | Centrifugal, direct-drive |

#### Cooling / Stack

| Parameter | Value |

|-----------|-------|

| Flue gas cooling | Seawater-cooled stack or exhaust dilution |

| Max flue gas outlet temp | 350°C |

| Cooling water flow | 3–5 m³/h |

| Stack height above deck | Per class rules |

---

### Operational Limits

#### Port Restriction

MARPOL and MEPC.76(40) strictly prohibit operation in port limits without explicit port authority permission.

Visual and audible warning must be acknowledged by operator before any start attempt.

Simulator models a 3nm port exclusion zone.

#### Temperature Hold Requirement

Primary chamber must reach and maintain ≥850°C for a minimum of **2 seconds** before waste feed begins (MEPC.76(40) Section 2.3).

Failure to hold temperature triggers LOW TEMP alarm and automatic waste feed cutoff.

---

### Alarm Setpoints

| Alarm ID | Description | Setpoint | Type |

|----------|-------------|----------|------|

| INC-001 | Primary chamber low temp | <850°C | Alarm |

| INC-002 | Secondary chamber low temp | <900°C | Warning |

| INC-003 | Flue gas high temp | >350°C | Alarm |

| INC-004 | Sludge oil high pressure | >4.5 bar | Alarm |

| INC-005 | Cooling water low flow | <2.5 m³/h | Alarm |

| INC-006 | Port restriction — incineration prohibited | In port | Warning |

| INC-007 | Primary chamber high temp | >1050°C | Warning |

---

### Waste Logging — Garbage Record Book (GRB)

Per MARPOL Annex V Regulation 10, a Garbage Record Book must be maintained on all ships of 100 GT and above and all ships certified to carry 15 or more persons.

#### Mandatory GRB Entries for Incineration:

Date and time of operation

Ship's position (latitude/longitude)

Description of garbage incinerated

Estimated amount (m³)

Name/signature of officer in charge

Categories (per Annex V):

**A** — Plastics (incineration only, not overboard)

**B** — Food waste

**C** — Domestic waste (incl. paper, rags)

**D** — Cooking oil

**E** — Incinerator ash (must be recorded separately)

**F** — Operational waste

**G** — Cargo residues (non-HME)

**H** — EGC residues

---

### Start-Up Procedure (Normal)

1. Confirm vessel is outside port limits (>3nm from land as minimum).

2. Check sludge oil tank level; ensure preheating to 60°C.

3. Open cooling water supply and confirm flow >3 m³/h.

4. Start combustion air fan; confirm airflow.

5. Open flue damper to OPEN position.

6. Ignite primary chamber burner; monitor temperature rise.

7. Wait for primary chamber to reach ≥850°C (hold 2 seconds).

8. Start sludge oil burner if OWC incineration required.

9. Confirm secondary chamber >900°C before loading solid waste.

10. Open solid waste feed door and load waste.

11. Monitor flue gas temperature: must not exceed 350°C.

---

### References

IMO MEPC.76(40) — Standard Specification for Shipboard Incinerators

MARPOL Annex I, Regulation 13 — Sludge Tanks

MARPOL Annex V — Prevention of Pollution by Garbage

ISM Code Section 7 — Shipboard Operations

SOLAS II-2/Reg.16 — Arrangements for fuel oil

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System