BlueSim Training Guide

Mooring Winch Operation

Deck Equipment

Mooring winch and line management trainer with 8-line OCIMF MEG4 arrangement, constant tension mode, brake control, and line parting simulation.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Mooring winch and line management trainer with 8-line OCIMF MEG4 arrangement, constant tension mode, brake control, and line parting simulation.

## Key Features

8 mooring lines displayed on ship plan SVG with per-line tension indicators.

Per-winch controls: HEAVE / HOLD / PAY OUT with drum speed and motor current.

Constant tension mode: winch auto-renders/heaves to maintain set tension.

Line condition display: SAFE / WARNING / OVERLOAD / PARTED.

Ship drift simulation: tide and wind effects on line tensions.

Line parting simulation when tension exceeds 80% MBL.

Auto-render alarm at 60% MBL.

Four scenarios: Berthing, Departure, Storm Mooring, Line Part Emergency.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| FORWARD STATION | Displays real-time system data, controls, and status indicators. |
| MOORING ARRANGEMENT — SHIP PLAN | Displays real-time system data, controls, and status indicators. |
| AFT STATION | Displays real-time system data, controls, and status indicators. |
| ALARMS | Displays real-time system data, controls, and status indicators. |
| OPERATIONS LOG | Displays real-time system data, controls, and status indicators. |
| ENVIRONMENT &amp; SCENARIO | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## Mooring Winch Operation Simulator — Technical Reference

### Overview

Mooring winches control the lines that keep a vessel secured to a berth or buoy. This simulator covers hydraulic and electric mooring winches on a typical LPG/product tanker with 8 mooring lines in the standard OCIMF MEG4 configuration.

### Mooring Line Configuration (OCIMF MEG4)

#### Forward Lines (Bow Station)

1. Head Line 1 (HL1) — Forward breast line: runs perpendicular to ship

2. Forward Spring 1 (FS1) — Runs forward along quay from forward lead

3. Forward Spring 2 (FS2) — Runs aft along quay from forward lead

#### Aft Lines (Stern Station)

4. Stern Line 1 (SL1) — After breast line: runs perpendicular to ship

5. After Spring 1 (AS1) — Runs forward along quay from aft lead

6. After Spring 2 (AS2) — Runs aft along quay from aft lead

#### Mid-ship Lines

7. Breast Line Fwd-Mid (BM1) — Mid-ship forward breast

8. Breast Line Aft-Mid (BM2) — Mid-ship aft breast

### Winch Specifications

#### Electric Mooring Winch (Typical)

**Rated pull (heaving)**: 10–25 tonnes

**Line speed (heaving)**: 10–15 m/min

**Rendering/paying out speed**: 15–20 m/min

**Drum capacity**: 150–200 m (typical mooring rope)

**Brake holding load**: 60–80% MBL

**Motor**: 22–55 kW, 440V, 60 Hz

#### Hydraulic Mooring Winch

**Rated pull**: 15–30 tonnes

**Hydraulic pressure**: 200–250 bar

**Constant tension range**: 5–20 tonnes

### MBL (Minimum Breaking Load)

Typical 52mm HMPE line: MBL ≈ 130 tonnes

Typical 56mm polyester: MBL ≈ 100 tonnes

Steel wire (38mm): MBL ≈ 80 tonnes

### Line Tension States

| Condition | Tension % MBL | Status |

|-----------|--------------|--------|

| Slack | < 5% | SLACK |

| Normal | 5–25% | SAFE |

| Elevated | 25–40% | ELEVATED |

| Warning | 40–60% | WARNING |

| High | 60–80% | DANGER |

| Critical | > 80% | OVERLOAD — at risk of parting |

| Parted | — | PARTED |

### Constant Tension Mode

Winch motor runs continuously at low torque

Sensor monitors line tension

If tension exceeds set point: winch renders (pays out) to relieve load

If tension drops below lower limit: winch heaves to recover slack

Set point typically: 15–25% MBL

Protects against ship movement, tidal changes, vessel loading/discharge

### Brake

Manual or power-operated disc/band brake

Brake holding capacity: set as % MBL

If tension exceeds brake holding capacity: line renders uncontrolled (brake slip)

Operator must monitor: if brake slipping, either tighten brake or re-moor

### Environmental Effects

**Tide**: Rising tide → breast lines slacken, springs tighten

**Wind**: Beam wind → increases tension on offshore lines, reduces near-side

**Swell**: Induces cyclic loads, peak loads can exceed MBL

**Current**: Fore-and-aft current → spring line tensions change

### Operations

#### Berthing — Taking Lines Ashore

1. Approach berth, heave lines sent ashore first

2. Manila messenger lines sent

3. Mooring ropes walked ashore by linesmen

4. Each winch: HEAVE mode until line is taut

5. Set appropriate tension, engage constant tension

6. All-fast — notify bridge

#### Departure — Let Go All Lines

1. Confirm tugs/pilot ready

2. Single up to minimum lines

3. Let go: forward lines first, then aft

4. Last line let go on Master's order

#### Storm Mooring

Double-up mooring lines

Increase brake holding load

Constant tension mode on all winches

Monitor tensions every 15 minutes

Reduce ship's drafts/GM as needed

### Line Parting Emergency

Immediate: bystanders clear of deck

Activate emergency stop on affected winch

Re-moor with backup line if possible

Investigate: inspect drum, check rope condition

Log incident, report to Master

### Regulatory References

**OCIMF MEG4**: Mooring Equipment Guidelines, 4th Edition

**SOLAS II-1**: Ship equipment — mooring arrangements

**ISM Code Chapter 7**: Emergency preparedness

**IHM (Inventory of Hazardous Materials)**: Rope/wire certifications

### Alarm Summary

| Alarm Tag | Description | Level |

|-----------|-------------|-------|

| MW-001-H | HL1 — High Tension Warning | W |

| MW-001-X | HL1 — OVERLOAD / Parting Risk | A |

| MW-002-H | FS1 — High Tension | W |

| MW-003-H | FS2 — High Tension | W |

| MW-004-H | SL1 — High Tension | W |

| MW-005-H | AS1 — High Tension | W |

| MW-006-H | AS2 — High Tension | W |

| MW-007-H | BM1 — High Tension | W |

| MW-008-H | BM2 — High Tension | W |

| MW-010 | Auto-Render Activated (Winch) | W |

| MW-011 | Line Parted — Emergency | A |

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System