BlueSim Training Guide

ODME — Oil Discharge Monitoring Equipment

Engine Room Equipment

Tanker ODME simulator per MARPOL Annex I Reg.31 and MEPC.108(49) — monitors instantaneous discharge rate (30 l/nm limit), position (50nm restriction), and logs to ORB Part II.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Tanker ODME simulator per MARPOL Annex I Reg.31 and MEPC.108(49) — monitors instantaneous discharge rate (30 l/nm limit), position (50nm restriction), and logs to ORB Part II.

## Key Features

Real-time instantaneous discharge rate (l/nm) with automatic overboard valve trip at >30 l/nm.

Live position tracking with 50nm from land restriction and special area enforcement.

Total discharged tracking against 1/30,000 cargo quantity MARPOL limit.

Recording strip chart showing discharge rate trend.

Oil Record Book Part II (ORB) entry logging with position, rate, and total.

Three scenarios: Normal Ballast Discharge, Rate Exceedance Trip, Position Restriction.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| ODME CONTROLS | Displays real-time system data, controls, and status indicators. |
| ODME SYSTEM SCHEMATIC & DISPLAY | Displays real-time system data, controls, and status indicators. |
| READINGS & ALARMS | Displays real-time system data, controls, and status indicators. |
| SYSTEM VALVES | Displays real-time system data, controls, and status indicators. |
| RECORDING STRIP CHART | Displays real-time system data, controls, and status indicators. |
| OIL RECORD BOOK Pt.II | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.2 Valve Controls

Click a valve button to toggle its state. Valve states are colour-coded:

Green: OPEN

Red: CLOSED

Amber: TRANSITIONING

Valve Name

OPEN

CLOSE

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## ODME (Oil Discharge Monitoring Equipment) — Technical Reference

### Regulatory Framework

#### MARPOL Annex I — Regulation 31: Discharge of Oil from Oil Tankers

For oil tankers of 150 GT and above, discharge of oil or oily mixture into the sea is prohibited EXCEPT when ALL of the following conditions are met:

1. The tanker is not within a Special Area.

2. The tanker is more than 50 nautical miles from the nearest land.

3. The tanker is proceeding en route.

4. The instantaneous rate of discharge of oil content does not exceed 30 litres per nautical mile.

5. The total quantity of oil discharged does not exceed:

- For existing tankers: 1/15,000 of the total quantity of the particular cargo.

- For new tankers: 1/30,000 of the total quantity of the particular cargo.

6. The tanker has in operation an Oil Discharge Monitoring and Control System (ODME) and a slop tank arrangement.

#### IMO MEPC.108(49) — Revised Guidelines and Specifications for ODME

Specifies technical requirements for ODME equipment.

ODME must continuously calculate and record:

- Ship's speed (knots).

- Instantaneous discharge rate (litres/nm).

- Total quantity discharged (litres).

- Position (latitude/longitude).

- Date and time.

ODME must automatically shut off the overboard discharge valve if rate exceeds 30 l/nm.

Recording must use tamper-evident memory (typically printout or electronic log).

#### MARPOL Annex I — Regulation 36: Oil Record Book Part II (Cargo/Ballast Operations)

For oil tankers, all cargo and ballast operations must be recorded in ORB Part II, including:

Loading and internal transfer of oil cargo.

Ballasting of cargo tanks and dedicated clean ballast tanks.

Cleaning of cargo tanks.

Discharge of ballast water (via ODME or to slop tank).

Condition of oil/water interface detector in slop tanks.

Transfer to slop tanks — quantities and times.

Discharge to sea — ODME readings, rate, total quantity.

#### 50nm and Special Area Restrictions

| Area | Restriction |

|------|------------|

| Within 50nm of land | No discharge (general) |

| MARPOL Special Areas | Zero discharge |

| Baltic Sea | Zero discharge |

| Mediterranean Sea | Zero discharge |

| Black Sea | Zero discharge |

| Red Sea | Zero discharge |

| Gulf Area (Persian/Gulf of Oman) | Zero discharge |

| Antarctic Area | Zero discharge |

#### MARPOL Special Areas for Tankers

In these areas, no discharge of oil or oily mixture is permitted from any tanker, except under the following condition (Reg. 15):

Clean or segregated ballast, provided the tanker is not in the Antarctic Area.

---

### ODME System Components

#### Hardware

1. Oil Content Meter (OCM): Continuously samples discharge flow; measures ppm or concentration.

2. Flow meter: Measures discharge rate in m³/h or similar.

3. Computer/Controller: Calculates instantaneous rate (l/nm) and cumulative total.

4. Speed Input: From GPS or ship's log — essential for rate calculation.

5. Position Input: GPS receiver — records position for ORB.

6. Overboard Valve Control: Relay output to automatic shut-off valve.

7. Printer: For tamper-resistant hard copy record.

8. Non-volatile Memory: Stores last 12+ months of operating data.

#### Instantaneous Discharge Rate Calculation

```

Rate (l/nm) = Flow Rate (l/h) ÷ Ship's Speed (knots)

```

Or equivalently:

```

Rate (l/nm) = [Pump Rate (l/s)] × 3600 ÷ Speed (nm/h)

```

Example:

Pump rate: 200 m³/h = 200,000 l/h

Ship's speed: 12 knots

Rate = 200,000 ÷ 12 = 16,667 l/nm → EXCEEDS 30 l/nm limit? No (16,667 > 30 → YES, EXCEEDS!)

Wait — 30 l/nm is very restrictive — implies very low discharge rates or high speeds.

Practical limits:

If speed = 12 kn, max flow = 30 × 12 = 360 l/nm × nm/h = 360 l/h = 0.36 m³/h.

If speed = 15 kn, max flow = 30 × 15 = 450 l/h = 0.45 m³/h.

This explains why only very clean ballast water can be discharged, and why slop tank washing water must generally be retained.

#### Maximum Allowable Total Discharge

For a new tanker (post 1979):

```

Max total (litres) = Cargo capacity (litres) ÷ 30,000

```

Example: VLCC with 200,000 m³ cargo = 200,000,000 litres → Max = 6,667 litres ≈ 6.7 m³

This is an extremely small quantity, meaning in practice, only very dilute clean water can be discharged.

---

### ODME Operating Procedure

#### Pre-Discharge Checks

1. Verify ODME is powered and functioning — self-test passed.

2. Confirm ship's position >50nm from nearest land.

3. Confirm ship is proceeding en route (not at anchor, not in port).

4. Verify ship is not in a Special Area.

5. Confirm ODME speed input active — GPS or log connected.

6. Check slop tank level and oil/water interface.

7. Confirm ORB Part II is available and being maintained.

8. Inspect overboard valve — automatic shut-off functioning.

#### Discharge Operation

1. Start discharge pump slowly.

2. Monitor ODME display: instantaneous rate (l/nm), oil content (ppm).

3. If rate exceeds 30 l/nm: ODME automatically closes overboard valve.

4. Ensure rate remains below limit by adjusting pump speed or increasing ship's speed.

5. Monitor total quantity discharged — compare to 1/30,000 limit.

6. Log all readings at minimum 1-hour intervals in ORB.

7. On completion: close overboard valve, stop pump, record final totals.

#### Response to Rate Exceedance Trip

1. ODME auto-closes overboard valve (pump continues to discharge to slop tank or stops).

2. Acknowledge alarm.

3. Do NOT manually bypass the overboard valve.

4. Investigate cause:

- Speed reduction?

- Oil content increase in ballast water?

- Wrong pump rate?

5. Correct the cause before restarting.

6. Record event in ORB with time, position, and reason.

7. Inform Chief Officer and Master.

---

### Slop Tank System

#### Slop Tank Arrangement

Oil tankers must have a **slop tank** or multiple slop tanks.

Purpose: collect tank washing water, ballast water containing residues, and other oily water.

Minimum capacity: at least 2% of cargo carrying capacity (3% for certain tanker types).

**MARPOL Retention Principle**: Slops retained until adequate segregation of oil and water has occurred. Clean water portion discharged via ODME. Oil retained for next port or incineration.

#### Oil/Water Interface Detector

Required in slop tanks.

Indicates height of oil/water interface.

Allows operator to know how much clean water is below the oil layer.

Only water below the interface can be discharged via ODME.

---

### ODME Recording Strip Chart

#### What is Recorded

Real-time instantaneous discharge rate (l/nm).

Total oil discharged.

Ship's speed.

Position (latitude/longitude).

Date and time.

Any trip events and operator interventions.

The strip chart or electronic log is tamper-evident.

#### PSC Inspection of ODME Records

Port State Control inspectors routinely inspect:

1. ODME printouts/records vs ORB Part II entries.

2. Correlation between dates, positions, and ORB entries.

3. Evidence of any valve bypass operations.

4. Calibration dates for OCM sensors.

5. Battery back-up for ODME (must maintain records on power failure).

---

### Key Alarm Conditions

| Alarm | Trigger | Action |

|-------|---------|--------|

| Rate Exceedance | >30 l/nm | Auto-trip overboard valve |

| Total Exceeded | >1/30,000 cargo qty | Alarm, stop discharge |

| Within 50nm | Position <50nm | Alarm, no discharge |

| Special Area | Position in list | Alarm, zero discharge |

| ODME Failure | System malfunction | Stop discharge, record failure in ORB |

| Low Speed | Speed <1 knot | Rate calculation unreliable — alarm |

| OCM Fault | Sensor failure | Stop discharge |

---

### References

MARPOL Annex I, Regulation 31 — Discharge of Oil from Oil Tankers

MARPOL Annex I, Regulation 36 — Oil Record Book Part II

IMO MEPC.108(49) — Revised Guidelines and Specifications for ODME

IMO MEPC.1/Circ.761 — Guidance on the Application of Regulation 34 and the Protection of Special Areas

IMO MEPC.55(33) — Guidelines for the Implementation of Regulation 15(3)(c) of MARPOL

US Coast Guard: MARPOL Implementation and Enforcement Guidance

ClassNK: Guidance on ODME Installation and Maintenance

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System