BlueSim Training Guide

Oily Water Separator (15 ppm)

Engine Room Equipment

15 ppm bilge separator with OCM and automatic 3-way divert valve per MARPOL Annex I Reg.14 and MEPC.107(49) — includes Oil Record Book entry simulation.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

15 ppm bilge separator with OCM and automatic 3-way divert valve per MARPOL Annex I Reg.14 and MEPC.107(49) — includes Oil Record Book entry simulation.

## Key Features

Oil Content Monitor (OCM) with real-time ppm display and automatic 15 ppm trip.

Three-way valve auto-divert to bilge holding tank when oil content exceeds 15 ppm.

OCM zero and span calibration procedure simulation.

Oil Record Book (ORB Part I) entry form with volume, position, and signature logging.

Alarms: high oil content, bilge tank high level, pump fault.

Three scenarios: Normal Discharge, High Oil Content Response, Calibration Check.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| CONTROLS | Displays real-time system data, controls, and status indicators. |
| OWS P&ID SCHEMATIC | Displays real-time system data, controls, and status indicators. |
| ALARMS & GAUGES | Displays real-time system data, controls, and status indicators. |
| VALVE LINEUP | Displays real-time system data, controls, and status indicators. |
| OCM TREND (ppm) | Displays real-time system data, controls, and status indicators. |
| OIL RECORD BOOK | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## Oily Water Separator (15 ppm) — Technical Reference

### Regulatory Framework

#### MARPOL Annex I — Regulation 14: Control of Discharge of Oil

Reg. 14(1): Any discharge into the sea of oil or oily mixtures from the machinery space bilges of a ship shall be prohibited EXCEPT when all of the following conditions are satisfied:

1. The oily mixture is processed through an oily-water separating equipment.

2. The oil content of the effluent without dilution does not exceed 15 parts per million (15 ppm).

3. The oily mixture does not originate from cargo pump room bilges.

4. In the case of ships of 400 GT and above, the oily mixture is not mixed with oil cargo residues.

#### MARPOL Annex I — Regulation 17: Oil Record Book Part I

All discharges (or accidental/other exceptional discharges) must be recorded in the Oil Record Book (ORB) Part I. Required entries:

Date, time, and position of ship when discharge commenced and stopped.

Amount of oily bilge water (in m³) discharged.

Identity of tank or space from which discharge was made.

Method of discharge used.

Failure to maintain accurate ORB is a criminal offence under most flag state legislation.

#### MEPC.107(49) — Revised Guidelines and Specifications for Pollution Prevention Equipment

Requires type-approval of the Oily Water Separator (OWS) system.

OWS must include an **Oil Content Meter (OCM)** / **Oil Content Monitor (OCM)** set to 15 ppm.

A **three-way valve** (also called automatic stopping device) must automatically divert overboard flow to bilge tank if oil content exceeds 15 ppm.

The 15 ppm alarm must be activated before the 15 ppm limit is reached (alarm at ~13 ppm typical).

Manual override of the three-way valve bypass is a MARPOL violation.

#### MEPC.107(49) — OCM Requirements

OCM must be capable of continuously monitoring the oil content of the effluent.

Must record oil content automatically.

The automatic stopping device must respond within 5 seconds of limit exceedance.

Zero and span calibration required at least every 12 months.

Calibration oil: n-hexadecane (C16H34) or equivalent standard.

#### Special Areas (MARPOL Annex I — Regulation 15)

In the following Special Areas, discharge of ANY oily water is prohibited (zero discharge):

Baltic Sea, Black Sea, Red Sea, Gulfs Area (Persian Gulf/Gulf of Oman), Gulf of Aden, Antarctic Area, North West European Waters, Oman Area, Southern South African Waters.

In Special Areas: all oily bilge water must be retained on board.

#### Port State Control (PSC) Focus Areas

Mismatch between ORB entries and actual tank soundings (sounding book).

Evidence of bypass piping around OWS.

OCM calibration records.

Blocked or inoperative automatic stopping device (three-way valve).

Discrepancy between bunkered fuel and ORB retention/transfer records.

---

### OWS Equipment — Technical Parameters

#### Typical OWS System (e.g., Victor Marine / RWO type)

| Parameter | Value |

|-----------|-------|

| Capacity | 1.0 m³/h |

| Maximum Inlet Oil Content | 5,000 ppm (pre-separator) |

| Outlet Oil Content | <15 ppm (guaranteed) |

| Operating Pressure | 0.2–0.6 bar |

| Separator Type | Gravity + coalescing filter |

| OCM Response Time | <5 seconds |

| OCM Range | 0–25 ppm |

| OCM Alarm Setting | 15 ppm (auto-trip at 15 ppm) |

| Operating Temperature | 5–40°C |

#### Separator Stages

1. Primary separation chamber: Gravity separation — oil droplets rise to surface, water drains.

2. Coalescing chamber: Fine oil droplets coalesce on oleophilic material (polypropylene media) and rise.

3. Oil content monitor: Samples effluent before discharge; triggers three-way valve on >15 ppm.

4. Three-way valve: OPEN = discharge overboard; DIVERTED = return to bilge holding tank.

---

### OCM Calibration Procedure (Zero and Span)

#### Zero Calibration

1. Flush OCM cell with clean, oil-free water.

2. Press ZERO on OCM panel.

3. Display should read 0.0 ppm.

4. If deviation >0.5 ppm, repeat flush and re-zero.

5. Record calibration in ORB and maintenance log.

#### Span Calibration

1. Prepare known-concentration reference solution (15 ppm or 10 ppm oil-in-water standard).

2. Introduce reference solution to OCM sample cell.

3. Adjust SPAN potentiometer until OCM reads known concentration.

4. Verify automatic trip activates at 15 ppm.

5. Record calibration certificate number and date in logbook.

#### Calibration Interval

Mandatory: at least every **12 months** (class survey).

Recommended: every 6 months operational check.

After any repair or replacement of OCM sensor.

---

### Bilge System Layout

#### Typical Engine Room Bilge Sources

| Source | Typical Oiliness |

|--------|-----------------|

| ME crankcase drip tray | High (5,000–50,000 ppm) |

| Purifier sludge overflow | Very high |

| Bilge well accumulation | Variable (100–10,000 ppm) |

| Deck drains | Lower (50–500 ppm) |

| Stern tube packing drip | Medium |

| FW generator condensate | Very low |

#### Bilge Tank Capacity Requirements

Gross tonnage 400–10,000 GT: no fixed minimum requirement, but sufficient for voyage.

Company best practice: 7–10 days autonomy without discharge.

MARPOL requires recording of all transfers in ORB.

---

### Oil Record Book (ORB) — Required Entries

#### Code C: Collection, Transfer, and Disposal of Oil Residues (Sludge)

C.1: Amount of oil residue (sludge) collected in retention/settling tank.

C.2: Method of disposal (shoreside reception, incineration, transfer).

C.3: Quantity of oil residue remaining.

#### Code D: Non-Automatic Discharge of Bilge Water

D.1: Quantity discharged (m³).

D.2: Time started/stopped.

D.3: Ship's position.

D.4: Method of disposal / OWS used.

#### Code F: Condition of OWS/OCM Equipment

F.1: Period of failure of OWS/OCM.

F.2: Reason for failure.

F.3: Period and reason any OWS/OCM not in use.

---

### Operational Checks Before OWS Operation

1. Confirm bilge holding tank level — adequate space for diversion.

2. Check OWS filter element condition.

3. Verify OCM zero/span calibration date is within 12 months.

4. Confirm three-way valve (automatic stopping device) functional.

5. Verify bilge pump operational, no cavitation.

6. Check overboard discharge valve (not in port, not in special area, distance from land).

7. Confirm position: >12 nm from nearest land for machinery space bilge.

---

### Distance Restrictions

| Area | Restriction |

|------|------------|

| MARPOL Special Areas | Zero discharge |

| Other areas | ≥12 nm from nearest land, ≤15 ppm |

| In port | No discharge (regardless of content) |

| At anchor | No discharge |

---

### References

MARPOL Annex I, Regulation 14 — Control of Discharge of Oil

MARPOL Annex I, Regulation 17 — Oil Record Book

IMO MEPC.107(49) — Revised Guidelines for OWS and OCM

IMO MEPC.1/Circ.511 — Guidance on Applicable Requirements for OWS

IMO MEPC.1/Circ.642 — Effective implementation of existing MARPOL requirements

IACS Rec. 97 — Guide for Oil Record Book Keeping

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System