BlueSim Training Guide

Purifier / Separator

Engine Room Equipment

Alfa Laval FOPX/LOPX centrifugal purifier simulator for fuel oil and lube oil separation per MEPC.1/Circ.642, covering bowl speed, desludging, and gravity disc selection.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Alfa Laval FOPX/LOPX centrifugal purifier simulator for fuel oil and lube oil separation per MEPC.1/Circ.642, covering bowl speed, desludging, and gravity disc selection.

## Key Features

Dual mode: FO Purifier (FOPX) and LO Purifier (LOPX) with mode-specific parameters.

Live gauges: bowl speed (RPM), feed temperature, throughput, back pressure, vibration.

Interactive gravity disc selection with oil/water interface position visualisation.

Automated desludge shot with sludge tank level tracking and shot log.

Alarms: high vibration auto-trip, low bowl speed, low feed temp, sludge tank high.

Three scenarios: Normal Start-Up, High Vibration Alarm, Gravity Disc Selection.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| CONTROLS | Displays real-time system data, controls, and status indicators. |
| PURIFIER P&ID SCHEMATIC | Displays real-time system data, controls, and status indicators. |
| ALARMS & STATUS | Displays real-time system data, controls, and status indicators. |
| VALVE LINEUP | Displays real-time system data, controls, and status indicators. |
| RPM TREND & SHOT LOG | Displays real-time system data, controls, and status indicators. |
| PARAMETERS | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.2 Valve Controls

Click a valve button to toggle its state. Valve states are colour-coded:

Green: OPEN

Red: CLOSED

Amber: TRANSITIONING

Valve Name

SMALL

MEDIUM

LARGE

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## Purifier / Separator — Technical Reference

### Regulatory Framework

#### MEPC.1/Circ.642 — Guidance on Best Practice for Fuel Oil Bunkering

Purification of fuel oil and lube oil is essential for protection of main and auxiliary engines.

Centrifugal separators must be operated at correct temperatures, throughput, and gravity disc settings.

#### MARPOL Annex I — Sludge Management

Sludge generated by purifiers must be collected in a sludge tank and cannot be discharged overboard.

Sludge tank capacity: minimum based on engine output and fuel consumption.

All sludge discharge recorded in Oil Record Book Part I.

#### Class Requirements (DNV GL, LR, ABS)

Fuel oil purifier capacity: sufficient to purify the daily fuel consumption plus 10% margin.

Two purifiers recommended for vessels dependent on HFO.

Lube oil purifier: continuous centrifugal purification of main engine lube oil.

---

### Alfa Laval FOPX / LOPX Centrifugal Purifier — Technical Parameters

#### Type: Self-Cleaning Centrifugal Purifier (Disc-Type)

The FOPX series is designed for fuel oil purification; LOPX for lube oil.

Both use a rotating disc stack to generate centrifugal force of approximately 6,500 g.

#### FO Purifier (FOPX 613) — Key Parameters

| Parameter | Value |

|-----------|-------|

| Throughput (rated) | 6,500 l/h at 38 cSt/50°C fuel |

| Bowl Speed | 8,700–9,100 RPM |

| Motor Power | 15 kW |

| Feed Temperature (HFO) | 95–98°C |

| Feed Temperature (MDO) | 40–50°C |

| Water Transducer (gravity disc) diameter | Depends on fuel density |

| Back Pressure | 0.5–1.0 bar |

| Sludge Shot Interval | Every 15–60 min (adjustable) |

| Sludge Discharge Volume | ~0.5–1.0 L per shot |

| Max Density of Fuel | 991 kg/m³ at 15°C |

#### LO Purifier (LOPX 705) — Key Parameters

| Parameter | Value |

|-----------|-------|

| Throughput (rated) | 5,000–6,000 l/h |

| Bowl Speed | 6,000–7,500 RPM |

| Motor Power | 11 kW |

| Feed Temperature | 90–95°C |

| Back Pressure | 0.5–0.8 bar |

| Operation Mode | Purifier (water separation) |

| Sludge Shot Interval | Every 30–90 min |

#### Gravity Disc / Water Transducer Selection

The gravity disc (also called paring disc or interface disc) controls the oil/water interface position within the bowl.

Gravity Disc Selection Formula:

Disc size depends on fuel oil density at operating temperature.

Fuel density > 930 kg/m³ at 15°C → larger disc (moves interface inward)

Fuel density < 900 kg/m³ at 15°C → smaller disc (moves interface outward)

Using the wrong disc size causes:

- Too large: oil loss through water outlet

- Too small: water carry-over into clean oil

Density-Temperature Correction:

Density at operating temp = density at 15°C – 0.65 × (T_op – 15) kg/m³

#### Bowl Speed Ranges

| Condition | Speed |

|-----------|-------|

| Normal Operation | 8,700–9,100 RPM |

| Low Speed Warning | <8,500 RPM |

| Very Low Speed Alarm | <8,000 RPM (may indicate bearing wear, imbalance) |

| Overspeed Trip | >9,500 RPM |

---

### Operating Procedures

#### FO Purifier Start-Up Sequence

1. Pre-heat: Open steam to FO heater. Raise feed temperature to 95–98°C.

2. Prime the bowl: Introduce a small water charge to form the sealing water ring.

3. Start motor: Engage clutch (if friction-type) or start direct-drive motor.

4. Check speed: Verify bowl speed 8,700+ RPM on tachometer.

5. Open feed valve: Slowly open FO inlet valve.

6. Set back pressure: Adjust back-pressure valve to 0.5–1.0 bar.

7. Check outlet: Verify clean oil temperature and colour.

8. Set shot interval: Programme automatic desludging timer.

9. Monitor: Check vibration levels, back pressure, and oil outlet condition regularly.

#### Sludge Discharge (Shot) Sequence (Alfa Laval type)

1. Timer or manual trigger initiates shot.

2. Water injection to bowl — creates hydraulic pressure on bowl bottom.

3. Bowl opens (bottom discs separate by ~0.5 mm) for 0.5–1.0 second.

4. Sludge + water + accumulated solids ejected to sludge space.

5. Bowl closes — hydraulic pressure re-established.

6. Priming water re-introduced to re-establish water seal.

7. Resume normal purification — typically 60 seconds recovery time.

#### High Vibration Response

Causes of high vibration:

1. Imbalance — accumulated sludge not fully discharged.

2. Bowl speed variation — worn bearings.

3. Mechanical damage — bent spindle, damaged discs.

4. Soft starter issues — if too slow acceleration.

Response procedure:

1. Reduce throughput immediately.

2. Initiate manual sludge shot to clear bowl.

3. If vibration persists, stop purifier.

4. Allow bowl to spin down completely before opening.

5. Inspect bowl, discs, and bearings during next maintenance.

---

### Alarm Settings

#### FO Purifier Alarms

| Alarm | Setpoint | Action |

|-------|----------|--------|

| High Vibration | Level 1: >7 mm/s | Reduce throughput |

| High Vibration | Level 2: >12 mm/s | Auto-stop |

| Low Bowl Speed | <8,500 RPM | Warning |

| Very Low Bowl Speed | <8,000 RPM | Trip |

| Low Feed Temperature | <90°C | Warning (water risk) |

| High Back Pressure | >1.5 bar | Warning |

| Sludge Tank High Level | 85% | Warning |

| Sludge Tank Full | 95% | Alarm, stop purifier |

#### LO Purifier Alarms

| Alarm | Setpoint | Action |

|-------|----------|--------|

| LO Low Pressure (engine feed) | <2.5 bar | Trip purifier |

| High Vibration | >7 mm/s | Alarm |

| Low Feed Temperature | <85°C | Warning |

| High Water in LO | — | Oil cloudiness alarm |

---

### Purifier vs Clarifier Mode

**Purifier mode** (with water seal): Removes both water AND solids. Requires correct gravity disc.

**Clarifier mode** (dam plate instead of gravity disc): Removes solids only. No water separation. Used when fuel is water-free (MDO/MGO).

For HFO: always use purifier mode.

For distillate fuels (MGO/MDO): clarifier mode acceptable, but purifier mode preferred.

---

### Throughput vs Efficiency

**Rated throughput** is based on standard test conditions.

Actual throughput should be reduced to 70–85% of rated for effective purification.

High-density fuels require lower throughput.

As fuel viscosity increases, throughput should decrease.

Recommended throughput = (daily consumption × 1.15) / operating hours per day

---

### Safety Notes

NEVER open bowl inspection ports while bowl is rotating.

Allow minimum 30 minutes for complete bowl spindown before access.

Balancing plugs must be re-installed after maintenance — critical for bowl balance at 9,000 RPM.

Centrifugal force at rated speed ≈ 6,500 g — bowl failure is catastrophic.

Vibration monitoring is the primary safety indicator.

---

### References

Alfa Laval FOPX 613 TM Operation Manual (EN-OM-FOPX613)

Alfa Laval LOPX 705 TM Operation Manual

MEPC.1/Circ.642 — Guidance on Best Practice for Fuel Oil Bunkering

MARPOL Annex I — Regulation 12: Tanks for Oil Residues (Sludge)

IMO MEPC.1/Circ.791 — Guidance on Management of Oily Water Residues

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System