BlueSim Training Guide

Scrubber System (EGCS) Simulator

Engine Room Equipment

Hybrid open/closed loop exhaust gas cleaning system (EGCS) for SOx compliance per MARPOL Annex VI Reg.14 and IMO MEPC.259(68).

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Hybrid open/closed loop exhaust gas cleaning system (EGCS) for SOx compliance per MARPOL Annex VI Reg.14 and IMO MEPC.259(68).

## Key Features

Hybrid EGCS with open loop, closed loop and dry bypass mode selector.

Continuous SO2/CO2 ratio monitoring with MEPC.259(68) compliance thresholds.

ECA entry/exit detection with automatic mode-change prompts and interlock.

Washwater pH, PAH and temperature monitoring with overboard discharge interlock.

EGCS Data Recorder (EDR) log strip with position, ratio and mode entries.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| CONTROLS | Displays real-time system data, controls, and status indicators. |
| P&amp;ID — HYBRID EGCS SCRUBBER | Displays real-time system data, controls, and status indicators. |
| GAUGES &amp; ALARMS | Displays real-time system data, controls, and status indicators. |
| ACTIVE ALARMS | Displays real-time system data, controls, and status indicators. |
| PIPELINE LINEUP | Displays real-time system data, controls, and status indicators. |
| EGCS DATA RECORDER (EDR) | Displays real-time system data, controls, and status indicators. |
| COMPLIANCE STATUS | Displays real-time system data, controls, and status indicators. |
| PAH:${S.pah.toFixed(0)} Flow:${S.wwFlow.toFixed(0)} | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.2 Valve Controls

Click a valve button to toggle its state. Valve states are colour-coded:

Green: OPEN

Red: CLOSED

Amber: TRANSITIONING

Valve Name

SW INLET [C]

WW OVBD [C]

CL RECIRC [C]

BYPASS [C]

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## Exhaust Gas Cleaning System (EGCS / Scrubber) — Technical Reference

### Regulatory Framework

#### MARPOL Annex VI — Prevention of Air Pollution from Ships

#### Regulation 14 — Sulphur Oxides (SOx)

**Global Sulphur Cap (since 1 Jan 2020)**: 0.50% m/m fuel oil sulphur limit — IMO MEPC.340(77)

**ECA Sulphur Limit**: 0.10% m/m in Emission Control Areas (ECAs)

ECAs: Baltic Sea, North Sea, North American ECA, US Caribbean Sea ECA

EGCS is an approved "equivalent means" of compliance per Regulation 4

#### Regulation 4 — Equivalents

Ships may use an approved EGCS instead of low-sulphur fuel

System must meet SO₂/CO₂ ratio criterion or specific SO₂ outlet concentration

Wash water discharge subject to Regulation 14 and MEPC.259(68) guidelines

#### IMO MEPC.259(68) — 2015 Guidelines for EGCS

Performance: SO₂/CO₂ ≤ 4.3 ppm/% (Open Loop), equivalent to 0.5% S fuel

ECA compliance: SO₂/CO₂ ≤ 2.1 (equivalent to 0.1% S fuel)

Wash water pH ≥ 6.5 at 4m from discharge point (Open Loop)

PAH ≤ 50 μg/l above background (Open Loop)

#### IMO MEPC.340(77) — 2021 Amendments

Further guidance on EGCS residue (sludge) management

EGCS residue must NOT be discharged into sea

Record keeping requirements for EGCS log book

---

### Operating Modes

#### Open Loop (OL) Mode

Uses seawater alkalinity to neutralise sulphur dioxide

Suitable for ocean/coastal areas with sufficient seawater alkalinity (>1.5 mM)

Wash water discharged overboard after treatment

Not permitted in ports, inland waterways, ECAs with specific prohibition

Requirements: pH ≥ 6.5, PAH ≤ 50 μg/l, turbidity ≤ 25 FTU above background

#### Closed Loop (CL) Mode

Uses freshwater with NaOH (caustic soda) dosing

No wash water overboard discharge — recirculated through holding tank

Required in ECAs, ports, low-alkalinity waters

pH maintained at 8.0–12.0 (internal circuit)

Wash water periodically discharged to port reception facility

#### Dry Mode (Bypass)

Bypass damper opens; exhaust bypasses scrubber

Must use compliant (low sulphur) fuel when in dry/bypass mode

Used for maintenance, scrubber fault recovery

---

### Key Parameters

#### Compliance Monitoring

| Parameter | OL Criterion | CL (Internal) |

|-----------|-------------|---------------|

| SO₂/CO₂ ratio | ≤ 4.3 ppm/% (global), ≤ 2.1 (ECA) | Same as OL |

| SO₂ inlet | Varies with fuel | Varies with fuel |

| SO₂ outlet | Monitored continuously | Monitored continuously |

| Washwater pH (OL) | ≥ 6.5 | ≥ 8.0 (circuit) |

| PAH (OL) | ≤ 50 μg/l above BG | N/A |

| Turbidity | ≤ 25 FTU above BG | N/A |

#### Flow Rates

| Component | Rate |

|-----------|------|

| Seawater pump (OL) | 45–300 m³/h |

| Recirculation pump (CL) | 20–80 m³/h |

| NaOH dosing (CL) | 0–50 l/h |

| Scrubber inlet gas flow | ~35,000 Nm³/h (10 MW engine) |

#### Temperature Limits

| Location | Normal | Maximum |

|----------|--------|---------|

| Scrubber inlet | 350–500°C | 600°C |

| Scrubber outlet | 40–80°C | 95°C |

| Wash water | 20–45°C | 65°C |

---

### EGCS Data Recorder

Per MEPC.259(68), the EGCS Data Recorder (EDR) must log:

Date/time (UTC)

Ship's position

SO₂/CO₂ ratio (inlet and outlet)

Wash water pH, PAH, turbidity

Seawater flow rate

EGCS mode (OL/CL/Bypass)

Any alarms or shutdowns

Record retention: minimum 18 months.

---

### Alarm Setpoints

| ID | Description | Setpoint | Type |

|----|-------------|----------|------|

| EGC-001 | SO₂/CO₂ ratio exceeded | >4.3 OL | Alarm |

| EGC-002 | Washwater pH low (OL) | <6.5 | Alarm |

| EGC-003 | Washwater pH low (CL) | <8.0 | Warning |

| EGC-004 | Washwater high temp | >65°C | Alarm |

| EGC-005 | PAH high | >50 μg/l | Alarm |

| EGC-006 | SW pump fault | Stopped | Alarm |

| EGC-007 | ECA entry — mode change required | ECA | Warning |

| EGC-008 | NOx out of range | — | Warning |

---

### Start-Up Procedure (Open Loop)

1. Confirm vessel not in port or inland waterway.

2. Start seawater pump; confirm flow rate >45 m³/h.

3. Open seawater inlet valve.

4. Confirm scrubber online — monitor SO₂/CO₂ ratio.

5. Confirm wash water pH ≥ 6.5 before opening overboard valve.

6. Set mode selector to OPEN LOOP.

7. Log EGCS start in EDR.

### ECA Entry Procedure (Switch to Closed Loop)

1. Receive ECA approach notification (minimum 1 hour before entry).

2. Start NaOH dosing pump.

3. Open recirculation loop valve.

4. Close overboard discharge valve.

5. Confirm pH ≥ 8.0 in recirculation circuit.

6. Switch mode selector to CLOSED LOOP.

7. Log mode change in EDR.

---

### References

MARPOL 73/78 Annex VI — Prevention of Air Pollution from Ships

IMO MEPC.259(68) — 2015 Guidelines for EGCS

IMO MEPC.340(77) — 2021 EGCS Guidance

IMO MEPC.184(59) — 2009 Guidelines for EGCS

IMO Circular MSC-MEPC.5/Circ.5 — EGCS Approval

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System