BlueSim Training Guide

Sewage Treatment Plant Simulator

Engine Room Equipment

Biological sewage treatment plant operation and discharge management per MARPOL Annex IV, MEPC.159(55) and MEPC.227(64).

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Biological sewage treatment plant operation and discharge management per MARPOL Annex IV, MEPC.159(55) and MEPC.227(64).

## Key Features

Three-stage biological STP: aeration, settling and chlorination with live parameter monitoring.

BOD, SS, residual chlorine, DO and pH gauges with MEPC.159(55) effluent quality limits.

4nm discharge restriction enforced — effluent overboard valve blocked within 4nm of land.

Blower fault scenario with sludge recirculation and DO recovery procedure.

Scenarios: Normal Treatment Cycle, Discharge Violation Alert, Blower Failure.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| CONTROLS | Displays real-time system data, controls, and status indicators. |
| P&amp;ID SCHEMATIC — BIOLOGICAL STP | Displays real-time system data, controls, and status indicators. |
| GAUGES &amp; ALARMS | Displays real-time system data, controls, and status indicators. |
| ACTIVE ALARMS | Displays real-time system data, controls, and status indicators. |
| PIPELINE LINEUP | Displays real-time system data, controls, and status indicators. |
| OPERATION LOG | Displays real-time system data, controls, and status indicators. |
| EFFLUENT PARAMETERS | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

## 4.2 Valve Controls

Click a valve button to toggle its state. Valve states are colour-coded:

Green: OPEN

Red: CLOSED

Amber: TRANSITIONING

Valve Name

SW INLET [C]

AER AIR [C]

EFFL OVBD [C]

SLUDGE RTN [C]

HOLD BYPASS [C]

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## Sewage Treatment Plant (STP) — Technical Reference

### Regulatory Framework

#### MARPOL Annex IV — Prevention of Pollution by Sewage from Ships

**Regulation 11**: Prohibits discharge of sewage unless treated to approved standard.

**Regulation 12**: Exceptions: vessel >12nm from nearest land (comminuted/disinfected), or >3nm (approved treatment system).

**Regulation 11.3**: Treated sewage effluent must meet standard: BOD ≤50 mg/l, SS ≤35 mg/l, residual chlorine 0.2–0.5 mg/l.

#### IMO Resolution MEPC.159(55) — Standards for Performance of STP

Defines effluent quality standards for type-approved systems.

BOD₅ ≤25 mg/l (enhanced standard), SS ≤35 mg/l, residual chlorine 0.2–0.5 mg/l.

Fecal coliforms ≤100 per 100 ml (treated effluent).

#### IMO MEPC.227(64) — 2012 Guidelines

Updated standards for sewage treatment plant type approval.

Enhanced nutrient reduction requirements for sensitive sea areas.

Equipment must have valid IOPP Certificate and STP Type Approval Certificate.

---

### Equipment: Hamworthy / Wärtsilä Biological STP

#### Process Overview

Three-stage biological treatment:

1. Aeration Tank — Aerobic biological degradation by activated sludge

2. Settling Tank — Gravity separation of sludge from treated effluent

3. Chlorination Chamber — Disinfection before overboard discharge

#### Aeration Tank

| Parameter | Normal Range |

|-----------|-------------|

| Tank level | 60–80% |

| Dissolved oxygen (DO) | 2.0–4.0 mg/l |

| pH | 6.5–8.5 |

| Temperature | 15–35°C |

| Aeration blower pressure | 0.3–0.8 bar |

#### Settling Tank

| Parameter | Value |

|-----------|-------|

| Hydraulic retention time | 2–4 hours |

| Sludge settling rate | >0.5 m/h |

| Surface loading rate | <1.5 m³/m²·h |

| Sludge blanket level | <60% tank depth |

#### Chlorination Chamber

| Parameter | Value |

|-----------|-------|

| Chlorine contact time | ≥15 minutes |

| Residual chlorine | 0.2–0.5 mg/l |

| Chlorine source | Sodium hypochlorite solution |

| Dosing rate | Proportional to flow rate |

---

### Effluent Quality Standards

| Parameter | MEPC.159(55) | MEPC.227(64) Enhanced |

|-----------|-------------|----------------------|

| BOD₅ | ≤50 mg/l | ≤25 mg/l |

| Suspended Solids (SS) | ≤35 mg/l | ≤35 mg/l |

| Residual Chlorine | 0.2–0.5 mg/l | 0.2–0.5 mg/l |

| Fecal Coliforms | ≤250/100ml | ≤100/100ml |

| pH | 6–9 | 6–9 |

---

### Discharge Restrictions

#### Geographic Limits

**4nm rule**: Must not discharge within 4nm of nearest land unless STP is running and effluent meets standards.

**Special Areas**: Baltic Sea, Black Sea, Antarctic — NO discharge permitted regardless of treatment.

Port Reception Facility (PRF) must be used when in port.

#### Vessel Movement

Discharge of raw sewage is only permitted when vessel is underway at speed ≥4 knots and >12nm from nearest land.

Treated effluent (from approved STP) may be discharged when >3nm from land, vessel underway.

---

### Equipment Alarm Setpoints

| Alarm ID | Description | Setpoint | Type |

|----------|-------------|----------|------|

| STP-001 | High BOD | >50 mg/l | Alarm |

| STP-002 | Low residual chlorine | <0.2 mg/l | Alarm |

| STP-003 | High residual chlorine | >0.5 mg/l | Warning |

| STP-004 | Aeration tank high level | >85% | Alarm |

| STP-005 | Aeration blower fault | Stopped unintended | Alarm |

| STP-006 | Low DO in aeration tank | <1.5 mg/l | Warning |

| STP-007 | High SS in effluent | >35 mg/l | Alarm |

| STP-008 | Discharge within 4nm | ECA position | Warning |

| STP-009 | pH out of range | <6.0 or >9.0 | Warning |

---

### Component Descriptions

#### Aeration Blower

Type: Rotary lobe / centrifugal

Duty: Provides oxygen for biological treatment

Standby: Auto-changeover if duty blower fails

#### Sludge Recirculation Pump

Returns settled sludge from settling tank back to aeration tank

Maintains active sludge concentration (MLSS: 2000–4000 mg/l)

Recirculation rate: 50–100% of influent flow

#### Chlorinator / Dosing Pump

Peristaltic or diaphragm dosing pump

Dosing rate controlled by effluent flow signal

Sodium hypochlorite: 12–15% active chlorine

#### Effluent Discharge Pump

Transfers treated effluent overboard

Interlocked with position monitoring (4nm limit)

Flow meter records discharge volume

---

### Start-Up Procedure (Normal)

1. Open sewage inlet valve; check aeration tank level.

2. Start aeration blower; confirm air flow to diffusers.

3. Start sludge recirculation pump.

4. Start chlorinator dosing pump.

5. Monitor DO rise to 2.0 mg/l (may take 30–60 min for biological activity).

6. Check BOD and SS readings (allow stabilisation time — 2–4 hours).

7. When effluent meets standards and vessel >4nm from land, enable discharge pump.

8. Log effluent discharge in Ship's Log and ORB.

---

### References

MARPOL 73/78 Annex IV — Prevention of Pollution by Sewage

IMO Resolution MEPC.159(55) — Revised Guidelines on Standards for STP

IMO Resolution MEPC.227(64) — 2012 Guidelines on Implementation of Effluent Standards

SOLAS II-1 — Construction requirements

ISM Code — Operational procedures

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System