BlueSim Training Guide

Steering Gear (Normal & Emergency)

Deck Equipment

Electro-hydraulic steering gear trainer with bridge and emergency local control, dual pump units, FU/NFU modes, and emergency steering changeover drill.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Electro-hydraulic steering gear trainer with bridge and emergency local control, dual pump units, FU/NFU modes, and emergency steering changeover drill.

## Key Features

Dual pump units (P1/P2) with auto-start on failure.

FU (Follow-Up) and NFU (Non-Follow-Up) control modes.

Animated rudder position indicator: digital readout and SVG arc display.

Emergency steering panel — local tiller room NFU control.

Emergency steering changeover checklist with guided steps.

Hydraulic pressure, oil temp, and motor current gauges.

Hard-over limit alarms (±33°) and hunting gear position.

Four scenarios including Pump 1 failure auto-start and emergency drill.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| BRIDGE CONTROL | Displays real-time system data, controls, and status indicators. |
| STEERING GEAR SCHEMATIC &amp; RUDDER INDICATOR | Displays real-time system data, controls, and status indicators. |
| EMERGENCY STEERING | Displays real-time system data, controls, and status indicators. |
| ALARMS | Displays real-time system data, controls, and status indicators. |
| OPERATIONS LOG | Displays real-time system data, controls, and status indicators. |
| SCENARIO INFO | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## Steering Gear Simulator — Technical Reference

### Overview

The steering gear converts helm orders from the bridge into physical rudder movement. This simulator covers an electro-hydraulic steering gear of the type manufactured by Rolls-Royce Marine and Wärtsilä (formerly MacGregor), using two hydraulic pump units and a ram-type actuator.

### System Description

#### Electro-Hydraulic Ram Type

Hydraulic pressure from pump unit drives hydraulic rams

Two rams (or four for twin-ram type) connected to tiller arm on rudder stock

Rudder angle controlled by position feedback (hunting gear)

Full helm to full helm (35° to 35°) in ≤ 28 seconds at full power (SOLAS requirement)

#### Pump Units

**Pump 1**: Variable delivery axial-piston hydraulic pump driven by electric motor

**Pump 2**: Identical standby unit; auto-start on Pump 1 failure

Motor rating: typically 75–185 kW per pump, 440V AC, 60 Hz

Hydraulic oil type: ISO VG 46 or VG 68

#### Hunting Gear

Mechanical feedback linkage from tiller to pump swashplate control

Reduces pump displacement as rudder approaches ordered angle

Provides smooth deceleration and prevents overshoot

### Operating Modes

#### Follow-Up (FU) Mode

Bridge autopilot or helm sends angle command → hunting gear reduces pump output as rudder reaches commanded position → rudder stops at ordered angle. Normal sailing mode.

#### Non-Follow-Up (NFU) Mode

Joystick/tiller on bridge: hold port/stbd → rudder moves continuously while held → release → rudder stops. Used for manual helm in restricted waters.

#### Autopilot Mode

Autopilot computer sends FU signals to steering gear; makes continuous small corrections to maintain course.

#### Emergency Steering

Activated when bridge control fails. Steering gear room (tiller room) takes direct local control. NFU joystick or push-buttons at local panel. Officer on bridge communicates by telephone/radio with engineer in steering gear room.

### Rudder Limits

**Normal operational limit**: ±35°

**Hard-over limit alarm**: ±33° (warning)

**Mechanical stop**: ±36° (physical stop fitted to tiller)

Time to 35° from 0°: ≤ 28 seconds (SOLAS II-1/29)

Time to 35° from 35° one side to other: ≤ 30 seconds

### Hydraulic System Parameters

| Parameter | Normal | Warning | Alarm/Trip |

|-----------|--------|---------|-----------|

| Working Pressure | 120–180 bar | >190 bar | >210 bar |

| Oil Temperature | 30–55°C | >60°C | >70°C |

| Pump Motor Current | 50–130 A | >140 A | >160 A |

| Rudder Response | 2–4°/sec | — | — |

### Emergency Steering Changeover Procedure

1. Bridge: Notify Master and engine room of intention to change to emergency steering

2. Engine Room / Tiller Room: Engineer proceeds to steering gear room

3. Bridge: Attempt to confirm loss of bridge steering control (NFU test)

4. Tiller Room: Confirm steering gear running (pump on, pressure OK)

5. Bridge: Disengage bridge steering control — set to NEUTRAL

6. Tiller Room: Engage local NFU control

7. Bridge: Communicate helm orders by telephone/radio

8. Tiller Room: Operate local joystick per bridge orders; report rudder angle

9. Tiller Room: Monitor hydraulic pressure and oil temperature throughout

10. After Drill: Return to bridge control — restore FU mode, test response

### Regulatory Requirements

#### SOLAS II-1/Regulation 29

Two independent steering gear systems on cargo ships > 10,000 GT

Each capable of putting rudder over 15° to 15° in 60 seconds

Main steering gear: 35° to 35° in ≤ 28 seconds at max service speed

Emergency steering must be operational within 2 minutes

#### MSC/Circ.1318

Emergency steering drill requirements

Every 3 months: emergency steering exercise

Procedures posted in steering gear room

Communication procedures tested

#### ISM Code

Emergency procedures documented in SMS

Crew trained in emergency steering procedure

Records of drills maintained

### Alarms

| Alarm Tag | Description | Level |

|-----------|-------------|-------|

| SG-001-H | Hydraulic High Pressure | A |

| SG-002-H | Oil High Temperature | A |

| SG-003-L | Oil Low Level | A |

| SG-004 | Pump 1 Motor Fault | A |

| SG-005 | Pump 2 Motor Fault | A |

| SG-006-H | Rudder Hard Over Port | W |

| SG-007-H | Rudder Hard Over Stbd | W |

| SG-008 | Bridge Control Failure | A |

| SG-009 | Auto-Start Pump 2 Activated | W |

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System