BlueSim Training Guide

Tank Gauging System Simulator

Cargo & Tanks

Integrated tank gauging system simulator with 12-tank monitoring, VCF density correction, stability computer and custody transfer measurement.

# 1. Display Requirements

| Optimal Resolution | 1920 x 1080 (Full HD) |
| --- | --- |
| Minimum Resolution | 1366 x 768 |
| Aspect Ratio | 16:9 (recommended) or 16:10 |
| Browser | Chrome 90+, Firefox 90+, Edge 90+ (fullscreen recommended) |
| Input | Mouse + keyboard (touchscreen not supported) |

Note: The simulator is designed for desktop use. On screens below 980 x 600 px, a "Screen Too Small" notice will appear. For the best experience, press F11 for fullscreen mode after launching the simulator.

# 2. Simulator Overview

Integrated tank gauging system simulator with 12-tank monitoring, VCF density correction, stability computer and custody transfer measurement.

## Key Features

12-tank radar/float/manual dip gauging with live level, volume, mass and temperature per tank.

Loading computer integration: GM, trim, heel, free surface effect and displacement.

Custody transfer mode with VEF calculation and ASTM VCF correction at observed temperature.

Four training scenarios: pre-load measurement, high-level alarm, stability monitoring, density correction.

# 3. Interface Layout

The simulator uses a multi-panel layout with the following regions:

| Panel | Purpose |
| --- | --- |
| EXIT | Displays real-time system data, controls, and status indicators. |
| STABILITY & LOADING COMPUTER | Displays real-time system data, controls, and status indicators. |
| TANK GAUGING — ALL TANKS | Displays real-time system data, controls, and status indicators. |
| TANK LAYOUT & OVERFLOW | Displays real-time system data, controls, and status indicators. |
| CARGO SUMMARY | Displays real-time system data, controls, and status indicators. |
| TANK LEVEL TREND (AVG) | Displays real-time system data, controls, and status indicators. |
| GAUGING LOG | Displays real-time system data, controls, and status indicators. |

Header Bar: Shows simulator title, regulatory reference, live clock, fullscreen toggle, and exit button.

Left Panel: System gauges, valve controls, and operational parameters.

Center Panel: P&ID schematic or main visual display (canvas/SVG).

Right Panel: Alarm management, emergency controls, and scenario buttons.

Bottom Panels: Trend charts, gas detection, scenario checklists, and supplementary data.

# 4. System Components

# 5. Training Scenarios

Scenario information will be displayed in the simulator modal on launch. Follow the on-screen checklist to complete each training objective.

# 6. Emergency Procedures

When an alarm condition occurs:

1. Identify the alarm in the Alarm Management panel (right side).

2. Acknowledge the alarm by clicking on it.

3. Assess the severity: WARNING (amber) requires monitoring; ALARM (red) requires immediate action.

4. If Emergency Shutdown (ESD) is required, press the ESD button.

5. Follow the scenario checklist for step-by-step response.

6. After resolving the condition, use the RESET button to restore normal operations.

# 7. Reference Documentation

## reference

## Tank Gauging System — Technical Reference

### Regulatory Framework

**MARPOL Annex I, Reg. 13G**: Requirements for tank condition monitoring.

**SOLAS II-1 Chapter B-1**: Ship stability requirements; integrated with gauging for loading computer.

**ICS/OCIMF Tanker Safety Guide 6th Ed.**: Measurement and gauging procedures.

**ASTM D1250 / IP Table 54**: Standard volume correction tables for petroleum products.

**OIML R 85 / API MPMS Chapter 17**: Custody transfer measurement standards.

**IMO Resolution MSC.267(85) — IS Code 2008**: Intact stability criteria with loading computer integration.

### Gauging Methods

#### 1. Radar Gauging (Primary — Non-Intrusive)

Microwave pulse time-of-flight or FMCW (Frequency-Modulated Continuous Wave)

Accuracy: ± 1 mm typical

Manufacturers: Saab RosemountTank Radar, Kongsberg, Emerson Roxar

Requires calibration table (strapping table) per tank

Unaffected by vapour density, temperature gradients in vapour space

Level range: 0 mm to tank height (ullage = tank height − liquid level)

#### 2. Float Gauge

Mechanical float on wire or tape

Accuracy: ± 5 mm

Used as backup/check measurement

Requires manual reading at deck level gauge head

#### 3. Manual Dip (Ullage/Innage)

Weighted tape lowered through ullage plug

Innage: measurement from bottom up (actual liquid depth)

Ullage: measurement from top down (empty space)

Correction: trim correction, list correction, wedge calculation

Required for custody transfer per ASTM D1250

### Key Measurements Per Tank

| Measurement | Symbol | Unit | Method |

|-------------|--------|------|--------|

| Liquid Level | L | mm | Radar/Float/Dip |

| Ullage | U | mm | Calculated |

| Observed Volume | V_obs | m³ | From calibration table |

| Temperature Corrected Volume | V_15 | m³ | ASTM Table 54 |

| Density at Temperature | ρ_t | kg/m³ | Hydrometer or inline |

| Standard Density (15°C) | ρ_15 | kg/m³ | Calculated |

| Mass / Weight | MT | metric tonnes | V_15 × ρ_15 |

| Vapour Pressure | P_vap | mbar | Calculated from temp |

### Volume Correction Factor (VCF)

VCF = correction factor to convert volume at observed temperature to volume at 15°C.

VCF = f(density at 15°C, observed temperature)

From ASTM Table 54A (crude oil) / 54B (refined products)

Standard temperature: 15°C (metric) or 60°F (US)

### Vessel Experience Factor (VEF)

Used in custody transfer to reconcile ship measurement with shore measurement.

VEF = accumulated shore quantity ÷ accumulated ship quantity (BOL)

Typical VEF range: 0.997–1.003

Apply VEF to ship measured quantity for bill of lading

### Trim and Stability (Loading Computer Integration)

**GM (Metacentric Height)**: minimum 0.15 m (stability requirement)

**Trim**: forward or aft; affects tank free surfaces and gauging accuracy

**Free Surface Effect**: reduces effective GM; critical during partial filling

**Stability Curve (GZ curve)**: area under GZ curve ≥ 0.055 m·rad to 30°

Loading computer calculates: displacement, trim, list, GM, BM, KB

### High Level Alarms

| Level | Alarm | Action |

|-------|-------|--------|

| 95% | HIGH Level Warning | Reduce loading rate |

| 98% | HIGH-HIGH Level | Stop loading immediately |

| 100% | OVERFLOW | Emergency stop all operations |

### Slop Tanks

Collect cargo residues, washings from COW (Crude Oil Washing)

Slop tank level monitored separately

Retention: MARPOL requires retention on board and decanting

Slop measurement: dip method (interface between oil and water)

### Capacity Tables

Each tank has an individual calibration table (strapping table)

Tables give: volume (m³) per mm of level

Trim corrections: longitudinal and transverse trim

Dead freight: cargo space not utilised (important for charter party)

# 8. Trainee Assessment Checklist

The instructor should verify the trainee can:

Identify all panels and their functions.

Read and interpret gauge values and colour codes.

Operate valve controls correctly.

Acknowledge and respond to alarms within appropriate time.

Execute the Emergency Shutdown procedure.

Complete at least two training scenarios with all checklist items done.

Explain the regulatory basis for the system (where applicable).

| Assessment Item | Result (Pass/Fail) | Remarks |
| --- | --- | --- |
| Scenario Completion |  |  |
| Alarm Response |  |  |
| Emergency Procedure |  |  |

Generated by BlueSim Training Guide System